3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240441

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4838 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NBIOT_508Check_CP_MONonSMSDataTransfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to the Koffset sending out the DL messages takes around 550msec, if RLC Ack and Service accept are sent separately then this leads to RRC Connection release not being sent at the given activation time. In order to ensure that the RLC ACK ( for service request) and Service Acccept message is sent out before the RRC Connection release procedure is triggered a delay is proposed similar to function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(p_CellId))/500.0); for NTN executions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.5, 22.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections

## Change 1

| Function name | f_NBIOT_508Check_CP_MONonSMSDataTransfer |
| --- | --- |
| Reason for change | Due to the Koffset sending out the DL messages takes around 550msec, if RLC Ack and Service accept are sent separately then this leads to RRC Connection release not being sent at the given activation time. In order to ensure that the RLC ACK ( for service request) and Service Acccept message is sent out before the RRC Connection release procedure is triggered a delay is proposed similar to function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 |
| Summary of change | Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(p_CellId))/500.0); for NTN executions |
| TTCN module | NBIOT_CommonProcedures |
| MCC160 Comment | Accepted, for maintenance reasons implemented slightly differently, see an altenative implementation introducing a new function f_NBIOT_NTN_Get_T_NTNoffset_InSeconds. Similar changes are made to f_NBIOT_508Check_CP_ResponseToPagingForMTAccess and f_NBIOT_508Check_CP_MOSMSDataTransfer in accordance to R5-245649 |

Before Change:

function f_NBIOT_508Check_CP_MONonSMSDataTransfer(NBIOT_CellId_Type p_CellId, template (present) UserDataContainer p_DataExpected, template (present) EstablishmentCause_NB_r13 p_EstCause := mo_Data) runs on NBIOT_PTC { var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_MSG_Indication_Type v_NasInd_Expected; var NAS_MSG_Indication_Type v_NasInd_Rxd; … … … // Step 4 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); }

After Change:

function f_NBIOT_508Check_CP_MONonSMSDataTransfer(NBIOT_CellId_Type p_CellId, template (present) UserDataContainer p_DataExpected, template (present) EstablishmentCause_NB_r13 p_EstCause := mo_Data) runs on NBIOT_PTC { var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_MSG_Indication_Type v_NasInd_Expected; var NAS_MSG_Indication_Type v_NasInd_Rxd; … … … // Step 4 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); // added a delay to ensure Service accept and ACK for service request is sent before the RRC Connection release procedure. if(pc_NB_ntn_only_Connectivity_EPC){ //@sic R5s240230 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(p_CellId))/500.0); } }

MCC 160 alternative implementation:

function f_NBIOT_508Check_CP_MONonSMSDataTransfer(NBIOT_CellId_Type p_CellId, template (present) UserDataContainer p_DataExpected, template (present) EstablishmentCause_NB_r13 p_EstCause := mo_Data) runs on NBIOT_PTC { var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var template (present) NAS_MSG_Indication_Type v_NasInd_Expected; var NAS_MSG_Indication_Type v_NasInd_Rxd; … … … // Step 4 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); // Step 5 //For NTN access, SS waits for timer 2* TNTN,offset expiry (Note 1) /*@sic R5-245649 sic@*/ f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(p_CellId)); //for non-NTN access the delay will result in 0 } } function f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return float { return int2float(f_NBIOT_NTN_Get_T_NTNoffset(p_CellId))/1000.0 }