3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240443

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3474 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR-DC RRC latency check test case 8.2.6.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to SS processing delays, it is not always guaranteed that Harq Ack will always be handled by TTCN before SRB Message. Also Test case has extensive check for the time difference between the Harq Ack and SRB message. Hence it is proposed to handle Harq Ack and SRB message in any order. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Function f_NR5GC_RRC_ProcedureDelay is modified to repeat the handling of Harq Ack or SRB message, in case any of the other message is not handled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_RRC_ProcedureDelay () |
| --- | --- |
| Reason for change | Due to SS processing delays, it is not always guaranteed that Harq Ack will always be handled by TTCN before SRB Message. Also Test case has extensive check for the time difference between the Harq Ack and SRB message. Hence it is proposed to handle Harq Ack and SRB message in any order. |
| Summary of change | Function f_NR5GC_RRC_ProcedureDelay is modified to repeat the handling of Harq Ack or SRB message, in case any of the other message is not handled. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted as improvement and as according to clause 7.1.8 in 38.523-3 (agreed in R5-244431) |

Before Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false, boolean p_InitialAccess := false, template (value) NR_ImcsValue_Type p_ImcsValue := tsc_NR_DefaultGrant_Imcs ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 R5s240284: added p_InitialAccess R5s240229: added p_ImcsValue for TC8_1_5_8_1 sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //@sic R5s230158 sic@ timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if (p_InitialAccess) { //For RRCSetup DCI 1_0 is used - Set K1 as per Table 8.1.5.8.1.3.3-11 v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1; } //Calculate T3 v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; //@sic R5s210842 change 2 sic@ v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //T3 configure SS with periodic UL Grant at T3 //@sic R5s240229 sic@ f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), p_ImcsValue, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //@sic R5s230158 sic@ if (p_NRDC_PduSession) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedNR_SRB_COMMON_IND; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } return v_ReceivedNR_SRB_COMMON_IND; }

After Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false, boolean p_InitialAccess := false, template (value) NR_ImcsValue_Type p_ImcsValue := tsc_NR_DefaultGrant_Imcs ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 R5s240284: added p_InitialAccess R5s240229: added p_ImcsValue for TC8_1_5_8_1 sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //@sic R5s230158 sic@ timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if (p_InitialAccess) { //For RRCSetup DCI 1_0 is used - Set K1 as per Table 8.1.5.8.1.3.3-11 v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1; } //Calculate T3 v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; //@sic R5s210842 change 2 sic@ v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //T3 configure SS with periodic UL Grant at T3 //@sic R5s240229 sic@ f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), p_ImcsValue, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; //repeat; if ( v_T4Flag ) { t_Watchdog.stop; } else { repeat; } } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; //t_Watchdog.stop; if ( v_T2Flag ) { t_Watchdog.stop; } else { repeat; } } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //@sic R5s230158 sic@ if (p_NRDC_PduSession) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedNR_SRB_COMMON_IND; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } return v_ReceivedNR_SRB_COMMON_IND; }