3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240451

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3477 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Addition of Rel-17 Redcap test case 7.1.1.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 7.1.1.1.15 to the 5G/NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.1.15 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.15.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	10

3	Branches executed	12

4	Execution Log Files	12

4.1	Mediatek MT6815 UE	12

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.1.15 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendar Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1.1.1.15

ATS Version:	iwd-TTCN3-B2022-09_D24wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function fl_TC_7_1_1_1_3and15_TestBody |
| --- | --- |
| Reason for change | To correct implementation of 7.1.1.1.15.3.2 Test Procedure Sequence. Current implementation does not take into account values of Preamble Index as specified in Table 7.1.1.1.15.3.3-1 (i.e., si-RequestConfigRedCap-r17 ra-PreambleStartIndex). Therefore, even with UE sending correct Preamble Index, test fails. |
| Summary of change | Modified function fl_TC_7_1_1_1_3and15_TestBody to add condition so it can correctly check for parameters specific to TC 7.1.1.1.15 as specified in Table 7.1.1.1.15.3.3-1 (i.e., si-RequestConfigRedCap-r17 ra-PreambleStartIndex). |
| TTCN module | MAC_SiRequest_NR5GC.ttcn |
| MCC160 Comment | Accepted, implemented in a simplest way with a new variable v_NR_PrachPreambleIndex - see proposed implementation below |

Before Change

function fl_TC_7_1_1_1_3and15_TestBody( boolean p_TC711115 := false) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { //@sic R5-198883 sic@ cs_NR_CellPower(nr_Cell1, -90, -91), cs_NR_CellPower(nr_Cell11, -84, -82) } var template SI_SchedulingInfo v_SI_SchedulingInfo; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5-199423 sic@ var template (value) SI_RequestConfig v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var integer i; var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; timer t_Watchdog := 11.0; // General wait timer - at most 2 modification periods //@siclog "Step 1" siclog@ //The SS changes SS/PBCH EPRE level of NR Cell 11 according to the row "T1" in Table 7.1.1.1.3.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@sic R5-198883 sic@ //@siclog "Step 2" siclog@ //Wait 60s to ensure UE detects NR Cell 11 @sic R5-194822 sic@ f_Delay(60.0); //Initialise SI_RequestConfig as per Table 7.1.1.1.3.3.3-1 //Reconfigure SS with SI_RequestConfig if (p_TC711115) { //@sic R5-232940 sic@ //For RedCap TC: ra-PreambleStartIndex = 53 f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-, cs_NR_SI_RequestConfig(cs_SI_RequestConfig(v_RACH_ConfigGeneric, tsc_RA_PreambleStartIndex ))))); } else { f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-, cs_NR_SI_RequestConfig(v_SI_RequestConfig)))); } //Pre-configure SS not to respond to 3 RACH preambles and then to transmit RAPID only in MSG2 f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponde3_RapidOnly(v_NR_PhysicalParameters) ); //Reconfigure SS to report PRACH preambles to TTCN f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@sic Steps 3-4 sic@ //SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1of Short Message field is set to 1 to indicate the SI modification. //The SS changes the parameter 'Qoffset' in SIB3 of NR Cell 1 according to the row "T2" in Table 7.1.1.1.3.3.2-1/2. The valueTag for SIB3 in the SIB1 message is increased and si-BroadcastStatus for SIB3 is set to 'notBroadcasted'. //Set SIB1 as per Table 7.1.1.1.3.3.3-1 f_NR_CellInfo_SetSIB1_SiRequestConfig(nr_Cell1, v_SI_RequestConfig ); v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(nr_Cell1); v_SI_SchedulingInfo.schedulingInfoList[1] := cs_NR_SchedulingInfoDef(tsc_SIB3_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType3, 1)}, notBroadcasting); f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell1, valueof(v_SI_SchedulingInfo)); if (p_TC711115) { //@sic R5-232940 sic@ //Initialise and reconfigure RedCap specific Initial UL BWP f_NR_ConfigRedCapUL_BWP(nr_Cell1, cs_PUCCH_ConfigCommon_ResourceCommonRedCap, omit); //Set si-RequestConfigRedCap-r17 in SIB1 as per 38.523-1 Table 7.1.1.1.15.3.3-1 v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric, tsc_RA_PreambleStartIndexRedCap); f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap(nr_Cell1, v_SI_RequestConfig ); } //Modify Sysinfo in SS - SS stops broadcasting SIB3 f_NR_ModifySysinfo(nr_Cell1, - ,- , false); //@sic R5-201006 R5-210575 sic@ //@siclog "Step 5" siclog@ //Check: Does the UE transmit a preamble on PRACH using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1 in Table 7.1.1.1.3.3.3-1? t_Watchdog.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, 52)) ->value v_NR_SYSTEM_IND { v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } } //@siclog "Steps 6-7-8" siclog@ //Check: Does the UE re-transmit a preamble on PRACH after ra-ResponseWindow using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1 in Table 7.1.1.1.3.3.3-1? //ra-ResponseWindow = sl20 = 20 slots for (i:=0; i<3; i:=i+1) { t_Watchdog.start(2.0); alt{ []SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, 52)) ->value v_NR_SYSTEM_IND { v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_CheckRaResponseWindow(nr_Cell1, v_TimingInfo1, v_TimingInfo2, 20, "Step 6"); v_TimingInfo1 := v_TimingInfo2; t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } } } … };

After Change

function fl_TC_7_1_1_1_3and15_TestBody( boolean p_TC711115 := false) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { //@sic R5-198883 sic@ cs_NR_CellPower(nr_Cell1, -90, -91), cs_NR_CellPower(nr_Cell11, -84, -82) } var template SI_SchedulingInfo v_SI_SchedulingInfo; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5-199423 sic@ var template (value) SI_RequestConfig v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var integer i; var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; timer t_Watchdog := 11.0; // General wait timer - at most 2 modification periods //@siclog "Step 1" siclog@ //The SS changes SS/PBCH EPRE level of NR Cell 11 according to the row "T1" in Table 7.1.1.1.3.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@sic R5-198883 sic@ //@siclog "Step 2" siclog@ //Wait 60s to ensure UE detects NR Cell 11 @sic R5-194822 sic@ f_Delay(60.0); //Initialise SI_RequestConfig as per Table 7.1.1.1.3.3.3-1 //Reconfigure SS with SI_RequestConfig if (p_TC711115) { //@sic R5-232940 sic@ //For RedCap TC: ra-PreambleStartIndex = 53 f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-, cs_NR_SI_RequestConfig(cs_SI_RequestConfig(v_RACH_ConfigGeneric, tsc_RA_PreambleStartIndex ))))); } else { f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-, cs_NR_SI_RequestConfig(v_SI_RequestConfig)))); } //Pre-configure SS not to respond to 3 RACH preambles and then to transmit RAPID only in MSG2 f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponde3_RapidOnly(v_NR_PhysicalParameters) ); //Reconfigure SS to report PRACH preambles to TTCN f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@sic Steps 3-4 sic@ //SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1of Short Message field is set to 1 to indicate the SI modification. //The SS changes the parameter 'Qoffset' in SIB3 of NR Cell 1 according to the row "T2" in Table 7.1.1.1.3.3.2-1/2. The valueTag for SIB3 in the SIB1 message is increased and si-BroadcastStatus for SIB3 is set to 'notBroadcasted'. //Set SIB1 as per Table 7.1.1.1.3.3.3-1 f_NR_CellInfo_SetSIB1_SiRequestConfig(nr_Cell1, v_SI_RequestConfig ); v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(nr_Cell1); v_SI_SchedulingInfo.schedulingInfoList[1] := cs_NR_SchedulingInfoDef(tsc_SIB3_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType3, 1)}, notBroadcasting); f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell1, valueof(v_SI_SchedulingInfo)); if (p_TC711115) { //@sic R5-232940 sic@ //Initialise and reconfigure RedCap specific Initial UL BWP f_NR_ConfigRedCapUL_BWP(nr_Cell1, cs_PUCCH_ConfigCommon_ResourceCommonRedCap, omit); //Set si-RequestConfigRedCap-r17 in SIB1 as per 38.523-1 Table 7.1.1.1.15.3.3-1 v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric, tsc_RA_PreambleStartIndexRedCap); f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap(nr_Cell1, v_SI_RequestConfig ); } //Modify Sysinfo in SS - SS stops broadcasting SIB3 f_NR_ModifySysinfo(nr_Cell1, - ,- , false); //@sic R5-201006 R5-210575 sic@ //@siclog "Step 5" siclog@ //Check: Does the UE transmit a preamble on PRACH using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1 in Table 7.1.1.1.3.3.3-1? if (p_TC711115){ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, 53)) ->value v_NR_SYSTEM_IND { v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } } }else{ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, 52)) ->value v_NR_SYSTEM_IND { v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } } } //@siclog "Steps 6-7-8" siclog@ //Check: Does the UE re-transmit a preamble on PRACH after ra-ResponseWindow using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1 in Table 7.1.1.1.3.3.3-1? //ra-ResponseWindow = sl20 = 20 slots if (p_TC711115){ for (i:=0; i<3; i:=i+1) { t_Watchdog.start(2.0); alt{ []SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, 53)) ->value v_NR_SYSTEM_IND { v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_CheckRaResponseWindow(nr_Cell1, v_TimingInfo1, v_TimingInfo2, 20, "Step 6"); v_TimingInfo1 := v_TimingInfo2; t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } } } }else{ for (i:=0; i<3; i:=i+1) { t_Watchdog.start(2.0); alt{ []SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, 52)) ->value v_NR_SYSTEM_IND { v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_CheckRaResponseWindow(nr_Cell1, v_TimingInfo1, v_TimingInfo2, 20, "Step 6"); v_TimingInfo1 := v_TimingInfo2; t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } } } } … };

MCC160 proposed implementation

function fl_TC_7_1_1_1_3and15_TestBody( boolean p_TC711115 := false) runs on NR5GC_PTC

{

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {                     //@sic R5-198883 sic@

cs_NR_CellPower(nr_Cell1, -90, -91),

cs_NR_CellPower(nr_Cell11, -84, -82)

}

var template SI_SchedulingInfo v_SI_SchedulingInfo;

var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5-199423 sic@

var template (value) SI_RequestConfig v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

var NR_SYSTEM_IND v_NR_SYSTEM_IND;

var SubFrameTiming_Type v_TimingInfo1;

var SubFrameTiming_Type v_TimingInfo2;

var integer i;

var NG_NAS_MSG_Indication_Type v_NasInd;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig;

var integer v_NR_PrachPreambleIndex := 52;  //value for TC 7.1.1.1.3 @sic R5s240451 sic@

timer t_Watchdog := 11.0; // General wait timer - at most 2 modification periods

//@siclog "Step 1" siclog@

//The SS changes SS/PBCH EPRE level of NR Cell 11 according to the row "T1" in Table 7.1.1.1.3.3.2-1.

f_NR_SetCellPowerList(v_CellPowerList_AtT1);     //@sic R5-198883 sic@

//@siclog "Step 2" siclog@

//Wait 60s to ensure UE detects NR Cell 11 @sic R5-194822 sic@

f_Delay(60.0);

//Initialise SI_RequestConfig as per Table 7.1.1.1.3.3.3-1

//Reconfigure SS with SI_RequestConfig

if (p_TC711115) { //@sic R5-232940 sic@

//For RedCap TC: ra-PreambleStartIndex = 53

f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-, cs_NR_SI_RequestConfig(cs_SI_RequestConfig(v_RACH_ConfigGeneric, tsc_RA_PreambleStartIndex )))));

} else {

f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-, cs_NR_SI_RequestConfig(v_SI_RequestConfig))));

}

//Pre-configure SS not to respond to 3 RACH preambles and then to transmit RAPID only in MSG2

f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponde3_RapidOnly(v_NR_PhysicalParameters) );

//Reconfigure SS to report PRACH preambles to TTCN

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable));

//@sic Steps 3-4 sic@

//SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1of Short Message field is set to 1 to indicate the SI modification.

//The SS changes the parameter 'Qoffset' in SIB3 of NR Cell 1 according to the row "T2" in Table 7.1.1.1.3.3.2-1/2. The valueTag for SIB3 in the SIB1 message is increased and si-BroadcastStatus for SIB3 is set to 'notBroadcasted'.

//Set SIB1 as per Table 7.1.1.1.3.3.3-1

f_NR_CellInfo_SetSIB1_SiRequestConfig(nr_Cell1, v_SI_RequestConfig );

v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(nr_Cell1);

v_SI_SchedulingInfo.schedulingInfoList[1]  := cs_NR_SchedulingInfoDef(tsc_SIB3_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType3, 1)}, notBroadcasting);

f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell1, valueof(v_SI_SchedulingInfo));

if (p_TC711115) { //@sic R5-232940 sic@

v_NR_PrachPreambleIndex := 53;     //@sic R5s240451 sic@

//Initialise and reconfigure RedCap specific Initial UL BWP

f_NR_ConfigRedCapUL_BWP(nr_Cell1, cs_PUCCH_ConfigCommon_ResourceCommonRedCap, omit);

//Set si-RequestConfigRedCap-r17 in SIB1 as per 38.523-1 Table 7.1.1.1.15.3.3-1

v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric, tsc_RA_PreambleStartIndexRedCap);

f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap(nr_Cell1, v_SI_RequestConfig );

}

//Modify Sysinfo in SS - SS stops broadcasting SIB3

f_NR_ModifySysinfo(nr_Cell1, - ,- , false);         //@sic R5-201006 R5-210575 sic@

//@siclog "Step 5" siclog@

//Check: Does the UE transmit a preamble on PRACH using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1 in Table 7.1.1.1.3.3.3-1?

t_Watchdog.start;

alt {

[] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, v_NR_PrachPreambleIndex)) ->value v_NR_SYSTEM_IND {

v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

t_Watchdog.stop;

}

[] t_Watchdog.timeout {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5");

}

}

//@siclog "Steps 6-7-8" siclog@

//Check: Does the UE re-transmit a preamble on PRACH after ra-ResponseWindow using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1 in Table 7.1.1.1.3.3.3-1?

//ra-ResponseWindow = sl20 = 20 slots

for (i:=0; i<3; i:=i+1) {

t_Watchdog.start(2.0);

alt{

[]SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, v_NR_PrachPreambleIndex)) ->value v_NR_SYSTEM_IND

{

v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

f_NR_CheckRaResponseWindow(nr_Cell1, v_TimingInfo1, v_TimingInfo2, 20, "Step 6");

v_TimingInfo1 := v_TimingInfo2;

t_Watchdog.stop;

}

[] t_Watchdog.timeout {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5");

}

}

}

…

};

## Change 2

| Function name | function f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | TC follows same steps and body as TC 7.1.1.1.3 which is a Early Contention resolution case.Therefore, this TC needs to be added to same list to complete attach process on NRCELL11 at Step 11. |
| Summary of change | Add TC 7.1.1.1.15 in list of Early Contention Resolution TCs |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted but as improvement it is better to remove the attribute Early Contention resolution for both test cases 7.1.1.1.3 and 7.1.1.1.15, to comply with 38.523-3 clause 7.3.5.1. As a result the UL grant shall be started after step 11. See proposed implementation below. |

Before Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_11_NR5GC") { return true; } case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_2_NR5GC") { return true; } case ("TC_11_7_3_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_6_4_2_3_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } … };

After Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_11_NR5GC") { return true; } case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_2_NR5GC") { return true; } case ("TC_11_7_3_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_6_4_2_3_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_15_NR5GC") { return true; } … };

MCC160 proposed implementation

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean

{

select (p_Testcase) {

case ("TC_11_3_11_NR5GC") { return true; }

case ("TC_11_3_2_NR5GC") { return true; }

case ("TC_11_3_6_NR5GC") { return true; }

case ("TC_11_3_6a_NR5GC") { return true; }

case ("TC_11_7_2_NR5GC") { return true; }

case ("TC_11_7_3_NR5GC") { return true; }

case ("TC_6_4_1_1_NR5GC") { return true; }

case ("TC_6_4_2_2_NR5GC") { return true; }

case ("TC_6_4_2_3_NR5GC") { return true; }

//     case ("TC_7_1_1_1_3_NR5GC") { return true; }

…

};

function fl_TC_7_1_1_1_3and15_TestBody( boolean p_TC711115 := false) runs on NR5GC_PTC

{

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {                     //@sic R5-198883 sic@

cs_NR_CellPower(nr_Cell1, -90, -91),

cs_NR_CellPower(nr_Cell11, -84, -82)

}

var template SI_SchedulingInfo v_SI_SchedulingInfo;

var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5-199423 sic@

var template (value) SI_RequestConfig v_SI_RequestConfig := cs_SI_RequestConfig(v_RACH_ConfigGeneric);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

var NR_SYSTEM_IND v_NR_SYSTEM_IND;

var SubFrameTiming_Type v_TimingInfo1;

var SubFrameTiming_Type v_TimingInfo2;

var integer i;

var NG_NAS_MSG_Indication_Type v_NasInd;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig;

var integer v_NR_PrachPreambleIndex := 52;  //value for TC 7.1.1.1.3 @sic R5s240451 sic@

timer t_Watchdog := 11.0; // General wait timer - at most 2 modification periods

…

alt {

//@siclog "Step 10" siclog@

//Check: Does UE send Msg3 containing an RRCSetupRequest message in the grant associated to the Random Access Response received in step 9?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(?, ?))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10");

}

//@siclog "Step 11" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.3-1 indicate that the UE is camped on NR Cell 11 belonging to a new TA?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(?, ?))) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

}

}

f_NR_ULGrantConfiguration_Start(nr_Cell11);  //@sic R5s240451 sic@

//generic test procedure in TS 38.508-1 [4] Table 4.9.3-1 in NR Cell11

f_NR_RRCSetup_Def(nr_Cell11);

// receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH):

v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell11,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?),

tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NasInd.Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo);

};

# Branches executed

The test case was executed on

- NR primary band n41 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6815 UE

The Mediatek MT6815 UE passed this test case on Platform Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_1_15_LOG.xml

- PICS/PIXIT parameter file:

TC_7_1_1_1_15_PIXIT.txt

# References

R5s240452: NR5GC FR1: Supporting information for addition of Rel-17 Redcap test case 7.1.1.1.15