3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240455

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3480 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC Test case 10.1.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-07-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step10, step 15, step21 and step 26, was expecting to use different DNN than step9. So need to other DNN what is being used at step9. Also at step7 UE performed switch off procedure and its not used DNN defined pc_APN_ID_Internet, So this DNN can be utilized again after step 7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated DNN with value of pc_APN_ID_Internet at step10, step 15, step21 and step 26. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.8.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_10_1_8_2_Body |
| --- | --- |
| Reason for change | - At step10, step 15, step21 and step 26, was expecting to use different DNN than step9. So need to other DNN what is being used at step9. Also at step7 UE performed switch off procedure and its not used DNN defined pc_APN_ID_Internet, So this DNN can be utilized again after step 7. |
| Summary of change | - Updated DNN with value of pc_APN_ID_Internet at step10, step 15, step21 and step 26. These changes made as per agreed CR R5-242958 during RAN5#103 |
| TTCN module | GSM_NR5GC |
| MCC160 Comment | Accepted. Additional change from TF160: As a result of Step 26, at Step 27 instead of using tsc_RandomAPN, pc_APN_ID_Internet should be used. |

Before Change

function fl_TC_10_1_8_2_Body() runs on NR5GC_PTC { -----------Skip code---- //@siclog "Step 9a1 - 9a2" siclog@ // @sic R5-233405 sic@ //Void // @sic R5-237375 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; //@siclog "Step 10" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Step 11" siclog@ // @sic R5-231545 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, ?, //S-NSSAI present ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 12" siclog@ // send PDU Session Reject with cause "insufficient resources for specific slice" v_MsgToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '01000101'B), v_BackOffTimer_Deactivated); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 14" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } //@siclog "Step 15" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Step 16" siclog@ // @sic R5-231545 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; t_Wait.stop; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, ?, //S-NSSAI present ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 17" siclog@ // send PDU Session Reject with cause "insufficient resources for specific slice" v_MsgToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '01000101'B), v_BackOffTimer_Deactivated); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); v_SecurityParams := f_NR5GC_Security_Get(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo := v_PDUSessionInfoList[f_NR5GC_MobileInfo_GetNoOfSessions() - 1]; // @sic R5s230782 sic@ if (pc_5GC_PDU_EAP_AKA_Prime) { // @sic R5-237375 sic@ // @sic R5s230782 sic@ //@siclog "Steps 18 - 19" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); //@siclog "Steps 20" siclog@ v_MsgToSend := f_Get_PDUSessionAuthenticationResult(v_GSM_MobilityInfo, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)))); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 21" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Step 22" siclog@ // @sic R5-231545 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, ?, //S-NSSAI present ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 23" siclog@ // send PDU Session Reject with cause "insufficient resources for specific slice" v_MsgToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '01000101'B), v_BackOffTimer_Deactivated); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); } v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionIdToRelease := v_PDUSessionInfoList[f_NR5GC_MobileInfo_GetNoOfSessions() - 1].SessionId; // @sic R5s230782 sic@ //@siclog "Steps 24 - 25" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease); //@siclog "Steps 26" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Steps 27 - 28" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI,-,-, cs_DNN(-, f_DomainName_Encode(valueof(tsc_RandomAPN)))); // @sic R5s240163 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27 - 28"); } //End of test body of TC_10_1_8_2

After Change

function fl_TC_10_1_8_2_Body() runs on NR5GC_PTC { -----------Skip code---- //@siclog "Step 9a1 - 9a2" siclog@ // @sic R5-233405 sic@ //Void // @sic R5-237375 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; //@siclog "Step 10" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, pc_APN_ID_Internet, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Step 11" siclog@ // @sic R5-231545 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, ?, //S-NSSAI present ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 12" siclog@ // send PDU Session Reject with cause "insufficient resources for specific slice" v_MsgToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '01000101'B), v_BackOffTimer_Deactivated); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 14" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } //@siclog "Step 15" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, pc_APN_ID_Internet, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Step 16" siclog@ // @sic R5-231545 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; t_Wait.stop; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, ?, //S-NSSAI present ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 17" siclog@ // send PDU Session Reject with cause "insufficient resources for specific slice" v_MsgToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '01000101'B), v_BackOffTimer_Deactivated); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); v_SecurityParams := f_NR5GC_Security_Get(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo := v_PDUSessionInfoList[f_NR5GC_MobileInfo_GetNoOfSessions() - 1]; // @sic R5s230782 sic@ if (pc_5GC_PDU_EAP_AKA_Prime) { // @sic R5-237375 sic@ // @sic R5s230782 sic@ //@siclog "Steps 18 - 19" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); //@siclog "Steps 20" siclog@ v_MsgToSend := f_Get_PDUSessionAuthenticationResult(v_GSM_MobilityInfo, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)))); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 21" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, pc_APN_ID_Internet, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Step 22" siclog@ // @sic R5-231545 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, ?, //S-NSSAI present ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 23" siclog@ // send PDU Session Reject with cause "insufficient resources for specific slice" v_MsgToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '01000101'B), v_BackOffTimer_Deactivated); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); } v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionIdToRelease := v_PDUSessionInfoList[f_NR5GC_MobileInfo_GetNoOfSessions() - 1].SessionId; // @sic R5s230782 sic@ //@siclog "Steps 24 - 25" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease); //@siclog "Steps 26" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, pc_APN_ID_Internet, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Steps 27 - 28" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI,-,-, cs_DNN(-, f_DomainName_Encode(valueof(tsc_RandomAPN)))); // @sic R5s240163 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27 - 28"); } //End of test body of TC_10_1_8_2