3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240456

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3481 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Addition of Cell reselection test case 6.1.2.3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2024-07-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 6.1.2.3a to the 5G/NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.1.2.3a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	10

3	Branches executed	12

4	Execution Log Files	12

4.1	Mediatek MT6897 UE	12

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.2.3a which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendar Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.1.2.3a

ATS Version:	iwd-TTCN3-B2022-09_D24wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6897

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_6_1_2_3a_NR5GC() |
| --- | --- |
| Reason for change | Corrected maximum reference power level of all cells. |
| Summary of change | Corrected initial ref power values so that it wont throw invalid ref power once we set power level as per test procedure. |
| TTCN module | Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_6_1_2_3a_NR5GC() runs on NR5GC_PTC {// Intra frequency reselection not allowed f_NR5GC_Init(NR_4); // System Simulator: System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells. // Create and configure cell - NR Cell 1, NR Cell 11 and NR Cell 12 have different tracking areas according to TS 38.508-1 Table 4.4.2-3. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_CellInfo_SetMIB_CellBarred_intraFreqReselection(nr_Cell1, barred, notAllowed); // Table 6.1.2.3a.3.3-1: MIB for NR Cell 1 f_NR_TestBody_Set(true); f_TC_6_1_2_3a_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); }

After Change

function f_TC_6_1_2_3a_NR5GC() runs on NR5GC_PTC {// Intra frequency reselection not allowed f_NR5GC_Init(NR_4); // System Simulator: System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells. // Create and configure cell - NR Cell 1, NR Cell 11 and NR Cell 12 have different tracking areas according to TS 38.508-1 Table 4.4.2-3. f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_CellInfo_SetMIB_CellBarred_intraFreqReselection(nr_Cell1, barred, notAllowed); // Table 6.1.2.3a.3.3-1: MIB for NR Cell 1 f_NR_TestBody_Set(true); f_TC_6_1_2_3a_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); }

## Change 2

| Function name | f_TC_6_1_2_3a_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Without adding delay UE was not getting modified sib1 of cell1 and reselecting unexpected intra frequency cell. |
| Summary of change | Added f_NR_ModifySysinfo_WaitActivation() instead of f_NR_ModifySysinfo. |
| TTCN module | Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_6_1_2_3a_NR5GC_TestBody() runs on NR5GC_PTC { var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T0_Cell11_RS_EPRE_FR1 := -78; var integer v_T0_Cell12_RS_EPRE_FR1 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_NonSuitableCellSSS_EPRE_FR2; var integer v_T0_Cell11_RS_EPRE_FR2 := tsc_NR_ServingCellSSS_EPRE_FR2; var integer v_T0_Cell12_RS_EPRE_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T0_Cell11_RS_EPRE_FR1, v_T0_Cell11_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell12, v_T0_Cell12_RS_EPRE_FR1, v_T0_Cell12_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. // Bit 1 of Short Message field is set to 1 to indicate the SysInfo Modification and SS changes NR Cell 1 SS/PBCH EPRE level according to row "T0" in table 6.1.2.3a.3.2-1/2. f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17(nr_Cell1, notAllowed); // Table 6.1.2.3a.3.3-2: SIB1 for NR Cell 1 f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 2" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12? f_NR5GC_MobilityRegistration (nr_Cell12); }

After Change

function f_TC_6_1_2_3a_NR5GC_TestBody() runs on NR5GC_PTC { var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T0_Cell11_RS_EPRE_FR1 := -78; var integer v_T0_Cell12_RS_EPRE_FR1 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_NonSuitableCellSSS_EPRE_FR2; var integer v_T0_Cell11_RS_EPRE_FR2 := tsc_NR_ServingCellSSS_EPRE_FR2; var integer v_T0_Cell12_RS_EPRE_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T0_Cell11_RS_EPRE_FR1, v_T0_Cell11_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell12, v_T0_Cell12_RS_EPRE_FR1, v_T0_Cell12_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. // Bit 1 of Short Message field is set to 1 to indicate the SysInfo Modification and SS changes NR Cell 1 SS/PBCH EPRE level according to row "T0" in table 6.1.2.3a.3.2-1/2. f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17(nr_Cell1, notAllowed); // Table 6.1.2.3a.3.3-2: SIB1 for NR Cell 1 f_NR_ModifySysinfo_WaitActivation(nr_Cell1); //Modify Sysinfo in SS f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 2" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12? f_NR5GC_MobilityRegistration (nr_Cell12); }

# Branches executed

The test case was executed on

- NR primary band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6897 UE

The Mediatek MT6897 UE passed this test case on Platform Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_2_3a_LOG.xml

- PICS/PIXIT parameter file:

TC_6_1_2_3a_PIXIT.txt

# References

R5s240457: NR5GC FR1: Supporting information for addition of Cell reselection test case 6.1.2.3a