3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240461

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3484 | rev | - | Current version: | 17.11.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the resume procedure, SS begin to use DCI1_1 after msg4 sended. But DCI1_1 can not be detected by UE before RRCResume message being received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set DCI1_0 to SS before sending RRCResume message. Set DCI1_1 to SS after RRCResumeComplete message being received. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.NR5GC,6.4.2.3.NR5GC,6.6.1.2.NR5GC,6.6.2.3.NR5GC,6.6.2.4.NR5GC,7.1.1.1.16.NR5GC,7.1.3.6.6.NR5GC,8.1.1.4.1.NR5GC,8.1.3.1.23.NR5GC,8.1.5.13.1.NR5GC,8.1.5.13.2.NR5GC,8_1_6_1_2_2_NR5GC, 8.1.8.2.2.NR5GC, 11.3.1a.NR5GC, 11.3.3.NR5GC, 11.3.6.NR5GC,11.3.6a.NR5GC,11.3.11.NR5GC,11.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | f_NR_SendRRCResume_Def() |
| --- | --- |
| Reason for change | In the resume procedure, SS begin to use DCI1_1 after msg4 sended. But DCI1_1 can not be detected by UE before RRCResume message being received. |
| Summary of change | Set DCI1_0 to SS before sending RRCResume message. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted, see MCC160 proposed implementation below. The same change is applied in: - f_TC_8_1_5_8_2_x_NR5GC_TestBody after step Step6a4 - f_TC_8_1_5_8_2_x_NR5GC_TestBody before step 6a5 - fl_TC_8_2_7_2_1_TestBody before step 4 |

Before Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ //@sic R5-241544: for SDT added v_RrcResume sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs); // Switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); // Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters //@sic R5s210578 sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

After Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ //@sic R5-241544: for SDT added v_RrcResume sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_CellId, cs_TimingInfo_Now, tsc_CnfReq, -, cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters))); // Switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); // Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters //@sic R5s210578 sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

MCC160 proposed implementation

function f_NR_SendRRCResume_Def( NR_CellId_Type                    p_NR_CellId,

template (value) TimingInfo_Type  p_TimingInfo := cs_TimingInfo_Now,

RRC_TransactionIdentifier         p_RRC_TI := tsc_NR_RRC_TI_Def,

template (omit) CellGroupConfig   p_CellGroupConfig := omit,

template (omit) RadioBearerConfig p_RadioBearerConfig := omit,

template (omit) MeasConfig        p_MeasConfig := omit,

template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit

) runs on NR5GC_PTC

{ //@sic R5-201646: added p_RadioBearerConfig sic@

//@sic R5-204441: added p_MeasConfig sic@

//@sic R5-241544: for SDT added v_RrcResume sic@

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs);

var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs);

// Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume

//@sic R5s240461 sic@

f_NR_SS_ReconfigSStoCSS_DCI1_0 (p_NR_CellId);

// Switch on UL grant assignments

f_NR_ULGrantConfiguration_Start(p_NR_CellId);

// Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters

//@sic R5s210578 sic@

//Reconfigure DRB in SS - existing DRBs are not reconfigured during release

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo);

//Activate security on newly configured DRB @sic R5s200102 sic@

f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo);

//@sic R5s210578 sic@

SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume));  //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@

}

/*

* @desc      Function to configure SS: map DcchDtch on CSS with DCI format 1_0

*            e.g. to send RRCResume

*/

function f_NR_SS_ReconfigSStoCSS_DCI1_0 (NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC

{ //@sic R5s240461 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_CellId, cs_TimingInfo_Now, tsc_CnfReq,

-,

cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters)

));

}

## Change 2

| Function name | f_NR_RRCResumeComplete_Def() |
| --- | --- |
| Reason for change | In the resume procedure, SS begin to use DCI1_1 after msg4 sended. But DCI1_1 can not be detected by UE before RRCResume message being received. |
| Summary of change | Set DCI1_1 to SS after RRCResumeComplete message being received. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted, see MCC160 proposed implementation below. The same change needs to be applied on: - fl_TC_11_3_1a_TestBody after step 14 - fl_TC_11_3_3_TestBody end of the test body - fl_TC_11_3_6and6a_TestBody after step Step 14a17 - fl_TC_11_3_11_TestBody after step 6 - fl_TC_8_1_1_1_1_TestBody after step 11a7 - f_TC_8_1_1_4_x_NR5GC_TestBody after step 4 - f_TC_8_1_1_4_y_NR5GC_TestBody after step 4 - f_TC_8_1_5_8_1_NR5GC_TestBody after step 14a5Ab2 - f_TC_8_1_5_8_2_x_NR5GC_TestBody after step 6a7 - f_TC_8_1_5_9_3_NR5GC_TestBody before step 3 - f_TC_8_1_5_9_3_NR5GC_TestBody before step 13 - fl_TC_8_2_7_2_1_TestBody after step 5 |

Before Change:

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId, template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete) runs on NR_BASE_PTC return template (omit) NG_NAS_MSG_Indication_Type { var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; //The UE transmits an RRCResumeComplete message. alt { // @sic R5s200403 sic@ [] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{} [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{ v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0]; } } return v_DedicatedNAS_Message; }

After Change:

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId, template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete) runs on NR_BASE_PTC return template (omit) NG_NAS_MSG_Indication_Type { var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); //The UE transmits an RRCResumeComplete message. alt { // @sic R5s200403 sic@ [] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{} [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{ v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0]; } } f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_CellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag)); return v_DedicatedNAS_Message; }

MCC160 proposed implementation

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId,

template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete)

runs on NR_BASE_PTC return  template (omit) NG_NAS_MSG_Indication_Type

{

var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@

var NR_SRB_COMMON_IND v_ReceivedAsp;

//The UE transmits an RRCResumeComplete message.

alt { // @sic R5s200403 sic@

[] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{}

[] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{

v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0];

}

}

// Reconfigure SS - set DcchDtch on USS with DCI format 1_1

//@sic R5s240461 sic@

f_NR_SS_ReconfigSStoUSS_DCI1_1 (p_NR_CellId);

return v_DedicatedNAS_Message;

}

/*

* @desc      Function to configure SS: map DcchDtch on USS with DCI format 1_1

*/

function f_NR_SS_ReconfigSStoUSS_DCI1_1 (NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC

{ //@sic R5s240461 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_CellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag));

}