3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240504

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3491 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 9 the TAU REQ expects the IE UE Additional Security Capability to be present. However, for the situation where a device does not support ENDC, this IE may not be included. This needs to be addressed. In the current TTCN implementation, fl_TC_11_1_7_NR5GC_EUTRA_TestBodyf_EUTRA38_TAUReqFromN1v_UENWCap is initialized. For the flow of testcase 11.1.7, the IEs v_UENWCap.cIoT_proSeCap and v_UENWCap.cIoT_V2X are set as ?. A device that sets support of N1 mode as not supported, and does not support any other capabilities in Octet 9 of UE Network capability, may not include the Octet 9 in the TAU REQ. Current TTCN implementation will fail such a device, and needs to be handled. In the current TTCN implementation the template used for capturing the TAU REQ at Step 25a4 expects the IE ueStatus to be present. This is fine for a case when the UE is transitioning from N1S1. However, in the current case the UE has already been on Eutra cell for the emergency call, and after releasing the call the UE performs a TAU to gain full services on the Eutra cell. Thus, this is not a N1S1 transition, and hence the IE ueStatus may be omitted by a conformant UE. This needs to be accommodated. The current TTCN does not implement rrc release after Step 25a6, this needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented the TTCN to check for pc_EN_DC and based on that initialized v_UEAddSecurityCap, which is then passed to function f_EUTRA38_TAUReqFromN1. Declared a new variable v_UENWCap, initialized it to cr_UENetworkCap_Any and passed this to function f_EUTRA38_TAUReqFromN1. Reworked the check for the N1 mode support. Created a new template cr_TAU_Request_UEStatus_N1for Step 25a4. Called function f_EUTRA_RRC_ConnectionRelease after Step 25a6 Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | These changes are in addition to R5s240502 and R5s240439. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	9

2.3	Change 3	10

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_11_1_7_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 9 the TAU REQ expects the IE UE Additional Security Capability to be present. However, for the situation where a device does not support ENDC, this IE may not be included. This needs to be addressed. In the current TTCN implementation, fl_TC_11_1_7_NR5GC_EUTRA_TestBodyf_EUTRA38_TAUReqFromN1v_UENWCap is initialized. For the flow of testcase 11.1.7, the IEs v_UENWCap.cIoT_proSeCap and v_UENWCap.cIoT_V2X are set as ?. A device that sets support of N1 mode as not supported, and does not support any other capabilities in Octet 9 of UE Network capability, may not include the Octet 9 in the TAU REQ. Current TTCN implementation will fail such a device, and needs to be handled. In the current TTCN implementation the template used for capturing the TAU REQ at Step 25a4 expects the IE ueStatus to be present. This is fine for a case when the UE is transitioning from N1S1. However, in the current case the UE has already been on Eutra cell for the emergency call, and after releasing the call the UE performs a TAU to gain full services on the Eutra cell. Thus, this is not a N1S1 transition, and hence the IE ueStatus may be omitted by a conformant UE. This needs to be accommodated. The current TTCN does not implement rrc release after Step 25a6, this needs to be implemented. |
| Summary of change | Implemented the TTCN to check for pc_EN_DC and based on that initialized v_UEAddSecurityCap, which is then passed to function f_EUTRA38_TAUReqFromN1. Declared a new variable v_UENWCap, initialized it to cr_UENetworkCap_Any and passed this to function f_EUTRA38_TAUReqFromN1. Reworked the check for the N1 mode support. Created a new template cr_TAU_Request_UEStatus_N1for Step 25a4. Called function f_EUTRA_RRC_ConnectionRelease after Step 25a6 Please see below. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comments | 1. This requires a prose CR. Decision postponed until outcome of prose CR 2. Alternative solution implemented. See R5s240502_MCC160Comments 3. Alternative solution implemented. See R5s240439_MCC160Comments 4. Accepted and implemented under R5s240439 |

Before Change:

function fl_TC_11_1_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var NasCount_Type v_NasCount; var NAS_MSG_Indication_Type v_NasInd; var boolean v_N1disabled := false; timer t_T1 := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5C" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); //@sic R5-210133 sic@ // @siclog "Steps 7-10b2" siclog@ // The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. // @sic R5-204467, R5s211357 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_N1Disabled('58'O))){ // @sic R5-227443 sic@ v_N1disabled := true; } v_NasCount := v_NasInd.SecurityProtection.NasCount; // @siclog "Step 10A - 10D" siclog@ f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); // @sic R5s210341 sic@ // @sic R5s210616 sic@ // Steps 4a1 - 4b2 are not performed for mapped security context, so not included here // @siclog "Step 10E - 11" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, true); // @sic R5s210149 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Steps 12 - 16" siclog@ // Steps 9-15 of 4.5A.4.3-1 f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, v_N1disabled); // @sic R5s211357, R5-227443 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14"); // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 21" siclog@ f_Delay(1.0); // @siclog "Step 21A" siclog@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-241691 sic@ // @siclog "Step 22" siclog@ // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //IMS call has been released. f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @siclog "Step 23" siclog@ f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);// @sic R5-241691 sic@ // @siclog "Step 24" siclog@ t_T1.start; // @sic R5-241691 sic@ alt {// @sic R5-241691 sic@ // @siclog "Step 25a1" siclog@ [] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { // @siclog "Step 25a2" siclog@ t_T1.stop; // @siclog "Step 25a3-7" siclog@ // @sic R5-243591 sic@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1(('000'B, '001'B, '010'B), ?, cr_DRXparameter_Any ('5C'O) ifpresent, f_GetAdditionalUpdateType(NORMAL), -, -, -, ?, -, -, cr_UeRadioCap ifpresent, -, -, cr_UE_AddSecurityCap_Any ifpresent, cr_UEStatus_Any ifpresent))); // @sic R5s240439 sic@ f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1, f_EUTRA_CellInfo_GetGuti(eutra_Cell1), f_GetEPSTAUType(NORMAL), v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); // @sic R5s240439 sic@ } // @siclog "Step 25b1" siclog@ [] t_T1.timeout {} } }

After Change:

module EPS_Fallback_NR5GC_EUTRA { <<SKIPPED CODE>> import from EUTRA_NR_Parameters all; //WA#WI1310708 11_1_7 <<SKIPPED CODE>> function fl_TC_11_1_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var NasCount_Type v_NasCount; var NAS_MSG_Indication_Type v_NasInd; var boolean v_N1disabled := false; var template UE_AdditionalSecurityCapability v_UEAddSecurityCap; //WA#WI1310708 11_1_7 var template(present) UE_NetworkCap v_UENWCap := cr_UENetworkCap_Any ('58'O); //WA#WI1310708 11_1_7 timer t_T1 := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5C" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); //@sic R5-210133 sic@ if (pc_EN_DC == true ) { //WA#WI1310708 11_1_7 v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } // @siclog "Steps 7-10b2" siclog@ // The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. // @sic R5-204467, R5s211357 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, //WA#WI1310708 11_1_7 v_UEAddSecurityCap, //WA#WI1310708 11_1_7 v_UENWCap /*WA#WI1310708 11_1_7*/); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7"); /*WA#WI1310708 11_1_7 COMMENTED OUT if (match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_N1Disabled('58'O))){ // @sic R5-227443 sic@ v_N1disabled := true; */ //WA#WI1310708 11_1_7 : Start if (ispresent (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X)) { if ( substr(bit2str(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X),2,1) == "0"){ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_N1disabled := true; } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4 : N1 mode supported"); } } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } //WA#WI1310708 11_1_7 : Stop v_NasCount := v_NasInd.SecurityProtection.NasCount; // @siclog "Step 10A - 10D" siclog@ f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); // @sic R5s210341 sic@ // @sic R5s210616 sic@ // Steps 4a1 - 4b2 are not performed for mapped security context, so not included here // @siclog "Step 10E - 11" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, true); // @sic R5s210149 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Steps 12 - 16" siclog@ // Steps 9-15 of 4.5A.4.3-1 f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, v_N1disabled); // @sic R5s211357, R5-227443 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14"); // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 21" siclog@ f_Delay(1.0); // @siclog "Step 21A" siclog@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-241691 sic@ // @siclog "Step 22" siclog@ // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //IMS call has been released. f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @siclog "Step 23" siclog@ f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);// @sic R5-241691 sic@ // @siclog "Step 24" siclog@ t_T1.start; // @sic R5-241691 sic@ alt {// @sic R5-241691 sic@ // @siclog "Step 25a1" siclog@ [] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { // @siclog "Step 25a2" siclog@ t_T1.stop; // @siclog "Step 25a3-7" siclog@ // @sic R5-243591 sic@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_UEStatus_N1(('000'B, '001'B, '010'B), //WA#WI1310708 11_1_7 ?, cr_DRXparameter_Any ('5C'O) ifpresent, f_GetAdditionalUpdateType(NORMAL), -, -, -, ?, -, -, cr_UeRadioCap ifpresent, -, -, cr_UE_AddSecurityCap_Any ifpresent))); f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1, f_EUTRA_CellInfo_GetGuti(eutra_Cell1), f_GetEPSTAUType(NORMAL), v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); //WA#WI1310708 11_1_7 } // @siclog "Step 25b1" siclog@ [] t_T1.timeout {} } }

## Change 2

| Function name | f_EUTRA38_TAUReqFromN1 |
| --- | --- |
| Reason for change | In the current TTCN implementation, fl_TC_11_1_7_NR5GC_EUTRA_TestBodyf_EUTRA38_TAUReqFromN1v_UENWCap is initialized. For the flow of testcase 11.1.7, the IEs v_UENWCap.cIoT_proSeCap and v_UENWCap.cIoT_V2X are set as ?. A device that sets support of N1 mode as not supported, and does not support any other capabilities in Octet 9 of UE Network capability, may not include the Octet 9 in the TAU REQ. Current TTCN implementation will fail such a device, and needs to be handled. |
| Summary of change | Within function f_EUTRA38_TAUReqFromN1, it is proposed to implement following code: if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { if((testcasename() == "TC_11_4_12_NR5GC") or (testcasename() == "TC_11_1_7_NR5GC")){ //WA#WI1306226 11_4_12 //WA#WI1310708 11_1_7 v_UENWCap := cr_UENetworkCap_Any('58'O); }else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); } //WA#WI1306226 11_4_12 v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency and (testcasename() != "TC_11_4_12_NR5GC") and (testcasename() != "TC_11_1_7_NR5GC")) { // @sic R5-217714 sic@ //WA#WI1306226 11_4_12 //WA#WI1310708 11_1_7 v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } Please note that this change is made in addition to proposed change in R5s240502 |
| TTCN module | Eutra38_NR5GCProcedures.ttcn |
| MCC160 Comments | Alternative solution implemented. See R5s240502_MCC160Comments |

Before Change:

<<SKIPPED CODE>> // If we need a new algorithm, have to store the new values before calling this // tell NAS EMU to check next message with 5G integrity f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(true)); if (not p_FirstN1_S1Transition) { // @sic R5s200583 sic@ v_UeRadioCap := cr_UeRadioCap ifpresent; } if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } /* Steps 1-3 */ //Receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // If we need a new algorithm, have to store the new values before calling this // tell NAS EMU to check next message with 5G integrity f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(true)); if (not p_FirstN1_S1Transition) { // @sic R5s200583 sic@ v_UeRadioCap := cr_UeRadioCap ifpresent; } if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { if((testcasename() == "TC_11_4_12_NR5GC") or (testcasename() == "TC_11_1_7_NR5GC")){ //WA#WI1306226 11_4_12 //WA#WI1310708 11_1_7 v_UENWCap := cr_UENetworkCap_Any('58'O); }else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); } //WA#WI1306226 11_4_12 v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency and (testcasename() != "TC_11_4_12_NR5GC") and (testcasename() != "TC_11_1_7_NR5GC")) { // @sic R5-217714 sic@ //WA#WI1306226 11_4_12 //WA#WI1310708 11_1_7 v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } /* Steps 1-3 */ //Receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): <<SKIPPED CODE>>

## Change 3

| Function name | cr_TAU_Request_UEStatus_N1 |
| --- | --- |
| Reason for change | In the current TTCN implementation the template used for capturing the TAU REQ at Step 25a4 expects the IE ueStatus to be present. This is fine for a case when the UE is transitioning from N1S1. However, in the current case the UE has already been on Eutra cell for the emergency call, and after releasing the call the UE performs a TAU to gain full services on the Eutra cell. Thus, this is not a N1S1 transition, and hence the IE ueStatus may be omitted by a conformant UE. This needs to be accommodated. |
| Summary of change | New template |
| TTCN module | Eutra38_NR5GCProcedures.ttcn |
| MCC160 Comments | Not needed |

template (present) NAS_UL_Message_Type cr_TAU_Request_UEStatus_N1 (template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue, template (present) NAS_KsiValue p_KsiValue, template DRXparameter p_DRXparameter, template AdditionalUpdateType p_AdditionalUpdateType, template (present) MobileIdentity p_MobileId := cr_MobileIdAny, template TrackingAreaId p_TAI := cr_TAI_Any ifpresent, template (present) B1_Type p_ActiveFlag := ?, // @sic R5-202667, R5-211371 sic@ template (present) B1_Type p_TSC := tsc_NasKsi_NativeSecurityContext, template NAS_KeySetIdentifier p_NonCurrentKSI := omit, template MobileIdentity p_AdditionalGUTI := cr_MobileIdAny ('50'O) ifpresent, // @sic R5-202563 sic@ template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap, template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_EPS_BearerContextStatus_Any, template (present) UE_NetworkCap p_UENWCap := cdr_NR5GC_UENetworkCap('58'O), template UE_AdditionalSecurityCapability p_AddSecurityCap := cr_UE_AddSecurityCap_Any) := { /* @status APPROVED (NR5GC_IRAT) */ tRACKING_AREA_UPDATE_REQUEST := { securityHeaderType := tsc_SHT_NoSecurityProtection, protocolDiscriminator := tsc_PD_EMM, messageType := tsc_MT_TauRequest, asmeNasKeySetId := cr_NAS_KeySetIdentifier(p_KsiValue, p_TSC), // @sic R5s200509 sic@ epsUpdateType := cr_EPS_UpdateType(p_ActiveFlag, p_EpsUpdate_TypeValue), // @sic R5-202667 sic@ oldGuti := p_MobileId, nonCurrentNativeNasKeySetId := p_NonCurrentKSI, /* @sic R5s100135, R5s200509 sic@ */ gprsCipheringKeySeqNum := cr_CiphKeySeqNum('8'H, ?) ifpresent, /* @sic R5s100135, R5-135073 sic@ */ oldPtmsiSignature := omit, additionalGuti := p_AdditionalGUTI, // @sic R5-201215 sic@ nonce := omit, ueNetworkCapability := p_UENWCap, // @sic R5s211353 sic@ lastVisitedRegisteredTai := p_TAI, drxParameter := p_DRXparameter, // @sic R5s100660, R5s120210 sic@ ueRadioCapNeeded := p_UeRadioCap, // @sic R5s200583 sic@ epsBearerContextStatus := p_EPS_BearerContextStatus, // @sic R5s200760 sic@ msNetworkCapability := cr_MS_NetworkCap_Any ifpresent, oldLai := cr_LAI_Any ifpresent, tmsiStatus := cr_TMSI_Status ifpresent, msClassmark2 := cr_MS_Clsmk2_Any_tlv ifpresent, // @sic R5s100662 sic@ msClassmark3 := cr_MSCLSMK3_Any ifpresent, supportedCodecList := cr_CodecListAny ifpresent, addUpdateType := p_AdditionalUpdateType, /* @sic R5-103681 sic@ */ voiceDomainPref := cr_VoiceDomainPref_Any ifpresent, /* @sic R5s110176 Baseline Moving sic@ */ oldGUTI_Type := cr_GUTI_Type_Native, deviceProperties := cr_DeviceProperties('D'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xD- @sic R5s120178 Baseline Moving sic@*/ msNetworkFeatureSupport := cr_MS_NetworkFeatureSupport('C'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xC- @sic R5s120178 Baseline Moving sic@*/ tmsiBasedNRIContainer := cr_NwkResourceIdAny ifpresent, // @sic R5s130195 Baseline Moving sic@ t3324Value := cr_GPRS_Timer2_3('6A'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ t3412ExtdValue := cr_GPRS_Timer2_3 ('5E'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ extendedDRXParams := cr_ExtdDRXParamsAny ifpresent, // @sic R5s160711 Baseline Moving sic@ ueAddSecurityCap := p_AddSecurityCap, // @sic R5-241528 sic@ ueStatus := cr_UEStatus_N1 ifpresent, //WA#WI1310708 11_1_7 addInfoReq := cr_AdditionalInfoReqAny ifpresent, /* cl. 9.9.3.55 O TLV 2 IEI=0x17 @sic R5s180552 Baseline Moving 2018 Phase 2 sic@ */ n1UENetworkCap := cr_N1_UENetworkCapAny ifpresent, /* cl. 9.9.3.57 O TLV 3-15 IEI=0x32 @sic R5s201386 Baseline Moving sic@ */ ueRadioCapIdAvailability := cr_UERadioCapIdAvailabilityAny ifpresent, /* cl. 9.9.3.58 O TV 3 IEI=0x34 @sic R5s201386 Baseline Moving sic@ */ requestedWUSAssistInfo := cr_WUSAssistInfoAny ifpresent, /* cl. 9.9.3.62 O TLV 3-n IEI=0x35 @sic R5s201386 Baseline Moving sic@ */ drxInNB_S1 := cr_DRXParamCommonAny ('36'O) ifpresent, /* cl. 9.9.3.63 O TLV 3 IEI=0x36 @sic R5s201386 Baseline Moving sic@ */ requestedIMSI_Offset := cr_IMSI_Offset ifpresent, /* cl. 9.9.3.64 O TLV 4 IEI=0x38 Sep22 @sic R5s221178 Baseline Moving sic@ */ ueRequestType := cr_UE_RequestType ifpresent, /* cl. 9.9.3.65 O TLV 3 IEI=0x29 Sep22 @sic R5s221178 Baseline Moving sic@ */ pagingRestriction := cr_PagingRestriction ifpresent /* cl. 9.9.3.66 O TLV 3-5 IEI=0x28 Sep22 @sic R5s221178 Baseline Moving sic@ */ } };