3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240517

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3495 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Addition of NR5GC Rel-16 RRC test case 8.1.1.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 8.1.1.3.10 to the 5G/NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.1.3.10 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	5

4	Execution Log Files	5

4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE UE	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.1.3.10 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.1.3.10

ATS Version:	iwd-TTCN3-B2022-09_D24wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function fl_TC_8_1_1_3_10_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | Current implementation is receiving and checking RRC CONNECTION REQUEST twice. This needs to be corrected. This can be done by using f_EUTRA38_TAUReqFromN1() which should check for TP just once. Additionally, LastVisitedRegisteredTAI should not be mandated as the Preamble doesn’t ensure EUTRA cell attach. This is in line with TS 38.508 Table 4.9.7.2.3-1: TRACKING AREA UPDATE REQUEST. |
| Summary of change | Use f_EUTRA38_TAUReqFromN1() to handle Steps 3-6, with p_CellIdPreviouslyCampedOn set to OMIT. Then continue Steps 7-11 with f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(). |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA.ttcn |
| MCC160 Comment | 1. Rejected. The RRC Connection Request is checked once (SRB.check) and then received once it’s inside the function (SRB.receive) 2. Accepted. Passed as a parameter to f_EUTRA38_TrackingAreaUpdateFromN1 |

Before Change

function fl_TC_8_1_1_3_10_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 3" siclog@ SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_HighPriorityAccess)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Power Level")); // @siclog "Step 5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def); }

After Change

import from EPS_NAS_MsgContainers all; function fl_TC_8_1_1_3_10_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 3" siclog@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, -, omit, cr_EstablishmentCause_HighPriorityAccess); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Power Level")); // @siclog "Step 5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_NasInd); }

## Change 2

| Function name | function fl_TC_8_1_1_3_10_TestBody() |
| --- | --- |
| Reason for change | To align the coordination messages for the exchange of GUTI parameters and authentication parameters to EUTRA PTC, f_NR5GC_TrackingAreaUpdateFromN1() needs to be moved before T2 Power Level change. |
| Summary of change | f_NR5GC_TrackingAreaUpdateFromN1() moved to before Power Level change |
| TTCN module | RRC_ConnRel_NR5GC_NR.ttcn |
| MCC160 Comment | Not needed as order of functions is not changed on EUTRA side |

Before Change

function fl_TC_8_1_1_3_10_TestBody() runs on NR5GC_PTC { var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message (IE mpsPriorityIndication-r16 and redirectionInformation including ARFCN-ValueEUTRA of EUTRA cell 1). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo, -, -, cs_RRCConnectionRelease_All_IEs_v1540(-, cs_RRCRelease_v1610_IEs(cs_RRCRelease_v1650_IEs(true_))) ))); //@siclog "Step 4" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

After Change

function fl_TC_8_1_1_3_10_TestBody() runs on NR5GC_PTC { var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message (IE mpsPriorityIndication-r16 and redirectionInformation including ARFCN-ValueEUTRA of EUTRA cell 1). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo, -, -, cs_RRCConnectionRelease_All_IEs_v1540(-, cs_RRCRelease_v1610_IEs(cs_RRCRelease_v1650_IEs(true_))) ))); //@siclog "Step 4" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble");

# Branches executed

The test case was executed on

- NR primary band N41 using NR ciphering nea1 and integrity algorithm nia1

- EUTRA primary band B3 using EUTRA ciphering eea1 and integrity algorithm eia1

# Execution Log Files

## Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE UE

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE UE passed this test case on Platform Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_1_3_10_LOG.xml

- PICS/PIXIT parameter file:

TC_8_1_1_3_10_PIXIT.txt

# References

R5s24518: NR5GC FR1: Supporting information for addition of NR5GC Rel-16 RRC test case 8.1.1.3.10