3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240521

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3497 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_IRAT_S1ToN1_SessionModification() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Inter RAT test cases may fail when the test case runs with IMS and the UE sends a SIP de-register message during the postamble. The current TTCN implementation changes the IMS state from deregister to reregister during the PDU session modification procedure by sending coordination messages to the IMS. If the UE does not send any SIP message after this then the IMS state will be re-registered, causing the test case to fail in the postramble when UE is sending SIP de-register message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted the TTCN code which is sending co-ordination messages to the IMS to change the IMS registration status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformance device may fail the inter-rat test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1.NR5GC, 6.2.3.2.NR5GC, 6.2.3.6.NR5GC, 6.2.3.8.NR5GC, 6.2.3.10.NR5GC, 6.2.3.11.NR5GC, 8.1.5.5.1.NR5GC, 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | f_NR5GC_IRAT_S1ToN1_SessionModification() |
| --- | --- |
| Reason for change | The Inter RAT test cases may fail when the test case runs with IMS and the UE sends a SIP de-register message during the postamble. The current TTCN implementation changes the IMS state from deregister to reregister during the PDU session modification procedure by sending coordination messages to the IMS. If the UE does not send any SIP message after this then the IMS state will be re-registered, causing the test case to fail in the postramble when UE is sending SIP de-register message. |
| Summary of change | Deleted the TTCN code which is sending co-ordination messages to the IMS to change the IMS registration status. |
| TTCN module | NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment | Alternative solution implemented: stop waiting for reregistration after the function has finished |

Before Change:

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_NR_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection +1), // @sic R5s220577 sic@ boolean p_DRBsAlreadyConfigured := false) runs on NR5GC_PTC return boolean { <<< Skipped >>> for (i := 0; i < N; i := i + 1) { if (p_PduSessionsFromLTE[i].PDU_SessionId == oct2int(v_GSM_MobilityInfo.SessionId)) { // f_IP_ChangeDrbMappingNR(IP, p_NR_CellId, cs_IP_QosFlowId(p_PduSessionsFromLTE[i].PDU_SessionId, p_PduSessionsFromLTE[i].QFI), p_PduSessionsFromLTE[i].PdnIndex); v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduSessionsFromLTE[i].EPS_Bearer); // @sic R5s210408 sic@ v_GSM_MobilityInfo.PdnIndex := valueof(p_PduSessionsFromLTE[i].PdnIndex); // @sic R5s210408 sic@ v_ReregisterIMS := v_ReregisterIMS or p_PduSessionsFromLTE[i].IsIMS; break; } } f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); // @sic R5s201413, R5s210408 sic@ // At the moment no DRBs are configured in the prose - so this will only work for HO TCs if (v_ReregisterIMS) { // @sic R5-205942 sic@ // Tell IMS to start waiting for the Register message. f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(NR)); } v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo); f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_RadioBearerList, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, v_SRBs); // @sic R5-213442 sic@ <<< Skipped >>> }

After Change:

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_NR_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection +1), // @sic R5s220577 sic@ boolean p_DRBsAlreadyConfigured := false) runs on NR5GC_PTC return boolean { <<< Skipped >>> for (i := 0; i < N; i := i + 1) { if (p_PduSessionsFromLTE[i].PDU_SessionId == oct2int(v_GSM_MobilityInfo.SessionId)) { // f_IP_ChangeDrbMappingNR(IP, p_NR_CellId, cs_IP_QosFlowId(p_PduSessionsFromLTE[i].PDU_SessionId, p_PduSessionsFromLTE[i].QFI), p_PduSessionsFromLTE[i].PdnIndex); v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduSessionsFromLTE[i].EPS_Bearer); // @sic R5s210408 sic@ v_GSM_MobilityInfo.PdnIndex := valueof(p_PduSessionsFromLTE[i].PdnIndex); // @sic R5s210408 sic@ v_ReregisterIMS := v_ReregisterIMS or p_PduSessionsFromLTE[i].IsIMS; break; } } f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); // @sic R5s201413, R5s210408 sic@ v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo); f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_RadioBearerList, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, v_SRBs); // @sic R5-213442 sic@ <<< Skipped >>> }

#### MCC160 Implementation:

function f_NR5GC_IRAT_S1ToN1_Steps14(NR_CellId_Type p_NR_CellId,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

boolean p_FirstTimeS1ToN1 := true,

InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED)  runs on NR5GC_PTC return NR_RadioBearerId_Type

{

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList;

//@sic R5s201060 sic@

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var boolean v_FailWhenK_LessN := false;

var template NG_Request_Type v_RequestType := cr_NG_Request_Type('001'B);

var boolean v_MsgRxd := false;

var integer i;

var boolean v_ReregisterIMS := false;

var default v_MyDefaultVar := null;

timer  t_Wait := 8.0; // @sic R5-204399, R5s210077 sic@

if (p_IwkN26 == SUPPORTED) { // @sic R5-204401 sic@

v_FailWhenK_LessN := true; // The only difference between 14a1 and 14b1 branch

v_RequestType := cr_NG_Request_Type('010'B); // @sic R5-206290 sic@

}

// always wait for PDU info, even if not 1st time

v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA);

// @sic R5s210520 sic@

// Clear store of msgs received in parallel

f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@

// Then activate store for any new msgs received - this works for Modification Reqs too

v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId));

t_Wait.start;  //  @sic R5-204399 sic@

// make sure all the PDNs configured on EUTRA are changed in routing table @sic R5s210149, R5s210520 sic@

for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) {

f_NR_ChangeDrbMappingForExistingPDNs(p_NR_CellId, v_PduSessionInfoList[i]);  // @sic R5s221175 sic@

}

// @sic R5s210520 sic@

// has a NAS msg already been received? if so, stop waiting

if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > 0) { // @sic R5s220174 sic@

v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); // @sic R5s220174 sic@

v_MsgRxd := true;

t_Wait.stop;

} else {

alt { //  @sic R5-204401 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp  // @sic R5s201413 sic@

{

f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp); // @sic R5s210687 sic@

v_MsgRxd := true;

t_Wait.stop;

}

[] t_Wait.timeout {}

}

}

if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request)) { // @sic R5s210520 sic@

if (p_FirstTimeS1ToN1 and p_IwkN26 == NOT_SUPPORTED) {

v_ReregisterIMS := f_NR5GC_IRAT_S1ToN1_SessionModification(p_NR_CellId, v_PduSessionInfoList, false, v_SRB); // @sic R5s201413 sic@

v_SRB := tsc_NR_RbId_SRB2; // @sic R5s210687 sic@

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not expecting PDU Session Modification message");

}

// reset flag as 1st msg now been processed

v_MsgRxd := false;

t_Wait.start; // restart timer in case there are now PDU Establishment messages

alt { //  @sic R5-204401 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp  // @sic R5s201413 sic@

{

// update v_ReceivedAsp and msg store if received again  @sic R5s210520 sic@

f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp);

v_MsgRxd := true;

t_Wait.stop;

}

[] t_Wait.timeout {

// Do nothing as no (more) PDU Modification/Establishment messages received so carry on

}

}

}

// don't need this anymore so deactivate it before it needs reactivating

deactivate(v_MyDefaultVar);  // @sic R5s210520 sic@

if (v_ReregisterIMS){ // @sic R5s240521 sic@

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStop);

}

if (v_MsgRxd) {  // @sic R5s210672 sic@

if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)) { // @sic R5s210520 sic@

f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, "", v_FailWhenK_LessN, -, v_PduSessionInfoList, -, 1); // @sic R5-206290, R5s210149, R5s210520 sic@

v_SRB := tsc_NR_RbId_SRB2; // @sic R5-204401 sic@

//  @sic R5-204401 sic@

if (pc_noOf_PDUsNewConnection >0 ){

f_NR_RRCRelease(p_NR_CellId);

f_NR5GC_RRC_Idle_Extension(p_NR_CellId, -, -, -, -, -, -, -, -, -, -, true);  // @sic R5-237315 sic@

}

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received");

}

}

// in case there's no PDU messages at all, don't release the connection in the above functions, but do it here instead

if (p_ReleaseConnection == rrcConnectionRelease) {

// Release RRC to enter RRC_IDLE

f_NR_RRCRelease(p_NR_CellId);

}

return v_SRB; // @sic R5-204401 sic@

}