3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240525

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3503 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.1.4.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN Implementation of 24wk24 - Inside the function f_TC_8_1_1_4_x_NR_Common - f_NR5GC_Preamble is done without test loop mode B activation. this should be done in order to achieve the data loop back involved in this case ( it requires Prose Change ) - In the function f_TC_8_1_1_4_x_NR5GC_TestBody - v_SCGConfigCA should be used only to assign v_CellGroupConfig.sCellToAddModList. Otherwise it over writes the values of mac_CellGroupConfig, physicalCellGroupConfig and spCellConfig - When sending the resume message as part of the function f_NR_SendRRCResume_Def , mac bearer routing field to be introduced as this is CA scenario - At step 5 , when sending the data activate scell boolean flag has to be set as false in the function f_NR5GC_CheckDataPath_CA . since the Scell activation is done using the peer message configuration in this test case - Additional steps are introduced to release the scell before postamble , to be in line with other CA cases ( it requries prose change ) - In the function definition f_NR_SendRRCResume_Def , new field p_MacBearerRoutingCA needs to be introduced to send the resume message in CA specific scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Inside the function f_TC_8_1_1_4_x_NR_Common - test loop mode B activation is done in f_NR5GC_Preamble - In the function f_TC_8_1_1_4_x_NR5GC_TestBody - v_SCGConfigCA is used only to assign v_CellGroupConfig.sCellToAddModList, to avoid overwritting the values of mac_CellGroupConfig, physicalCellGroupConfig and spCellConfig - When sending the resume message as part of the function f_NR_SendRRCResume_Def , mac bearer routing field is introduced - At step 5 , when sending the data activate scell boolean flag is set as false in the function f_NR5GC_CheckDataPath_CA . - Additional steps are introduced to release the scell before postamble , to be in line with other CA cases ( it requries prose change ) In the function definition f_NR_SendRRCResume_Def , new field p_MacBearerRoutingCA is introduced to send the resume message in CA specific scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

1.1	Verification Test Summary	4

2	Corrections required	4

2.1	Correction to the Function f_TC_8_1_1_4_x_NR_Common()	4

2.2	Correction to the Function f_TC_8_1_1_4_x_NR5GC_TestBody	6

2.3	Correction to the Function  f_NR_SendRRCResume_Def	9

3	Branches executed	11

4	Execution Log Files	11

4.1	MediaTek MT6983	11

4.2	Hisilicon Balong 5000	11

5	References	12

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.1.4.4 which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D24wk24 TTCN-3

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.1.4.4.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D24wk24 TTCN-3

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UEs used:	MediaTek MT6983 and Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Correction to the Function f_TC_8_1_1_4_x_NR_Common()

| Function name | f_TC_8_1_1_4_x_NR_Common |
| --- | --- |
| Reason for change | f_NR5GC_Preamble is done without test loop mode B activation. this should be done in order to achieve the data loop back involved in this case ( it requires Prose Change ) |
| Summary of change | test loop mode B activation is done in f_NR5GC_Preamble |
| TTCN module | develop\NR5GC\8_1_1\RRC_Resume_CA_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#104. |

Before Change:

function f_TC_8_1_1_4_x_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={ cs_NR_CellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2 )}; f_NR5GC_Init_CA(NR_1, {p_PCell, p_SCell}, p_NR_CA_Tested); // Create and configure cell f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell, Scell_Active); // Preamble: according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-2 f_NR5GC_Preamble (p_PCell, STATE_INACTIVE_2A); f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_TestBody_Set(true); f_TC_8_1_1_4_x_NR5GC_TestBody(p_PCell, p_SCell); f_NR_TestBody_Set(false); //Switch/Power off UE f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_1_1_4_x_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={ cs_NR_CellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2 )}; f_NR5GC_Init_CA(NR_1, {p_PCell, p_SCell}, p_NR_CA_Tested); // Create and configure cell f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell, Scell_Active); // Preamble: according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-2 f_NR5GC_Preamble (p_PCell, STATE_INACTIVE_2A, PING_Or_TEST_LOOPModeB_ON); //WA#WI=1289070 f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_TestBody_Set(true); f_TC_8_1_1_4_x_NR5GC_TestBody(p_PCell, p_SCell); f_NR_TestBody_Set(false); //Switch/Power off UE f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); }

## Correction to the Function f_TC_8_1_1_4_x_NR5GC_TestBody

| Function name | f_TC_8_1_1_4_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | - v_SCGConfigCA should be used only to assign v_CellGroupConfig.sCellToAddModList. Otherwise it over writes the values of mac_CellGroupConfig, physicalCellGroupConfig and spCellConfig - When sending the resume message as part of the function f_NR_SendRRCResume_Def , mac bearer routing field to be introduced as this is CA scenario - At step 5 , when sending the data activate scell boolean flag has to be set as false in the function f_NR5GC_CheckDataPath_CA . since the Scell activation is done using the peer message configuration in this test case - Additional steps are introduced to release the scell before postamble , to be in line with other CA cases ( it requries prose change ) |
| Summary of change | - v_SCGConfigCA is used only to assign v_CellGroupConfig.sCellToAddModList, to avoid overwritting the values of mac_CellGroupConfig, physicalCellGroupConfig and spCellConfig - When sending the resume message as part of the function f_NR_SendRRCResume_Def , mac bearer routing field is introduced - At step 5 , when sending the data activate scell boolean flag is set as false in the function f_NR5GC_CheckDataPath_CA . - Additional steps are introduced to release the scell before postamble , to be in line with other CA cases ( it requries prose change ) |
| TTCN module | develop\NR5GC\8_1_1\RRC_Resume_CA_NR5GC.ttcn |
| MCC160 Comment | - Accepted. - Rejected. The code pasted here is not the latest version. - Accepted. - Accepted conditional to prose CR agreement at RAN5#104. |

Before Change:

function f_TC_8_1_1_4_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1, -, -, -, -, activated); var template (value) CellGroupConfig v_CellGroupConfig; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo; //Prepare variables v_CellGroupConfig := f_NR_GetSCGConfigResume(p_PCell); // condition RESUME v_CellGroupConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig; // condition SCell_Add v_CellGroupConfig.physicalCellGroupConfig := v_SCGConfigCA.physicalCellGroupConfig; // condition SCell_Add v_CellGroupConfig.spCellConfig := v_SCGConfigCA.spCellConfig; // condition SCell_Add v_CellGroupConfig.sCellToAddModList := v_SCGConfigCA.sCellToAddModList; // condition SCell_Add //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(p_PCell, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(p_PCell); //Configure SS with CA v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); f_NR_ConfigPCell_AddRelSCells (p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_CellGroupConfig, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message f_NR_SendRRCResume_Def(p_PCell, cs_TimingInfo_NR(v_TimingInfo), -, v_CellGroupConfig); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCResumeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and SCell? f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 5", true ); }

After Change:

import from NR_ASP_Templates_CommonPart all; << SKIPPED CODE >> function f_TC_8_1_1_4_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1, -, -, -, -, activated); var template (value) CellGroupConfig v_CellGroupConfig; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo; //Prepare variables v_CellGroupConfig := f_NR_GetSCGConfigResume(p_PCell); // condition RESUME // v_CellGroupConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig; // condition SCell_Add //WA#WI=1289070 //v_CellGroupConfig.physicalCellGroupConfig := v_SCGConfigCA.physicalCellGroupConfig; // condition SCell_Add //WA#WI=1289070 //v_CellGroupConfig.spCellConfig := v_SCGConfigCA.spCellConfig; // condition SCell_Add //WA#WI=1289070 v_CellGroupConfig.sCellToAddModList := v_SCGConfigCA.sCellToAddModList; // condition SCell_Add //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(p_PCell, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(p_PCell); //Configure SS with CA v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); f_NR_ConfigPCell_AddRelSCells (p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_CellGroupConfig, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message f_NR_SendRRCResume_Def(p_PCell, cs_TimingInfo_NR(v_TimingInfo), -, v_CellGroupConfig,-,-,-,-,crs_MacBearerRouting(p_PCell)); //WA#WI=1289070 //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCResumeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete,-,crs_MacBearerRouting(p_PCell))); //WA#WI=1289070 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and SCell? f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 5", false ); //WA#WI=1289070 f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //WA#WI=1289070 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

## Correction to the Function  f_NR_SendRRCResume_Def

| Function name | f_NR_SendRRCResume_Def |
| --- | --- |
| Reason for change | In the function definition f_NR_SendRRCResume_Def , new field p_MacBearerRoutingCA needs to be introduced to send the resume message in CA specific scenarios |
| Summary of change | In the function definition f_NR_SendRRCResume_Def , new field p_MacBearerRoutingCA is introduced to send the resume message in CA specific scenarios |
| TTCN module | develop\Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit, boolean p_StartGrantPeriodic := true //WA#WI=1271929 ) runs on NR5GC_PTC { << SKIPPED CODE >> //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

After Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit, boolean p_StartGrantPeriodic := true, //WA#WI=1271929 template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit //WA#WI=1289070 ) runs on NR5GC_PTC { << SKIPPED CODE >> //@sic R5s210578 sic@ //Introducing Mac Bearer Routing for 8.1.1.4.4 //WA#WI=1289070 SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume,p_MacBearerRoutingCA)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

# Branches executed

## This NR TC was executed in NR5GC CA FR1 Intra-band Contiguous CA Bands CA_n78C (MT6983), CA_n41C (HiSilicon Balong 5000) with NEA1 ciphering and NIA1 integrity algorithms.

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_1_4_4_NR5GC MTK
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## Hisilicon Balong 5000

The Hisilicon Balong 5000 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_1_4_4_NR5GC HiSi
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240526: Supporting information for addition of NR5GC testcase 8.1.1.4.4 |
| --- | --- |