3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240547

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3517 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional handling of PICS settings in NR RRC test case 8.1.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation does not consider the case when a PICS item is wrongly enabled for a feature the UE does not support and this IE is contained in an omitted data structure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes implemented in test case 8.1.5.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE with invalid parameter configuration may wrongly pass the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	2

1	Changes to NR RRC test case 8.1.5.1.1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk32 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Changes to NR RRC test case 8.1.5.1.1

Module NR_CapabilityFunctions.

NOTE: update marked with label “//Moved inconclusive case from Dual mode to Dual and Single” is also done in functions:

fl_Check_eventA_MeasAndReport, fl_Check_csi_RSRP_AndRSRQ_MeasWithSSB, fl_Check_skipUplinkTxDynamic, fl_Check_logicalChannelSR_DelayTimer, fl_Check_longDRX_Cycle, fl_Check_shortDRX_Cycle, fl_Check_ss_SINR_Meas, fl_Check_csi_RSRP_AndRSRQ_MeasWithoutSSB, fl_Check_multipleConfiguredGrants, fl_Check_sftd_MeasNR_Neigh, fl_Check_voiceOverNR¸ fl_Check_idleInactiveNR_MeasReport_FRX_Diff, fl_Check_drx_Adaptation_r16, fl_Check_directMCG_SCellActivation_r16, fl_Check_directMCG_SCellActivationResume_r16, fl_Check_directSCG_SCellActivationResume_r16, fl_Check_directSCG_SCellActivation_r16 and f_Check_twoPUCCH_AnyOthersInSlot.

Additional comments from MCC160

Additional checks to first level of IEs in UE-NR-Capability when they are restricted with ‘ifpresent’ as they were not present in the previous version of this document.

function fl_Check_intraAndInterF_MeasAndReport(UE_NR_Capability p_ReceivedCapabilityMsg)

…

// check indication of TDD IE

if (ispresent(v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport.intraAndInterF_MeasAndReport)) {

v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD := true;

}

if (not (f_NR_AnyOfThree_SignaledbyUE(v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport, v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD, v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD)))  //NOTE: Moved inconclusive case from Dual mode to Dual and Single

{ // inconclusive case: UE does not report any IE for intraAndInterF_MeasAndReport but pc_intraAndInterF_MeasAndReport is set

f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_intraAndInterF_MeasAndReport to true is inconsistent with reported capabilities");

}

// Error cases if the feature is set

if ((not f_NR_DualModeCheck()) and (not v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport) or v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD or v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD) { // fail cases on Single mode

f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-CapabilityAddXDD IEs is inconsistent with pc_intraAndInterF_MeasAndReport on Single Mode");

}

else if(f_NR_DualModeCheck()) // case dual mode

{

if (not v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport)

{

if (v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD and v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD) // fail case: both FDD and TDD capabilities are reported and IE on IntraAndInterF_MeasAndReport_Common is not

{

f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs is inconsistent");

}

//NOTE: Moved inconclusive case from Dual mode to Dual and Single

}

else

if  (v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD or v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD)  // fail cases: if IE measAndMobParameters.measAndMobParametersXDD_Diff.intraAndInterF_MeasAndReport is reported but FDD and/or TDD capabilities are also reported

{

f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent");

}

}

}

else {  // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE

if (ispresent(v_IntraAndInterF_MeasAndReport_Common.intraAndInterF_MeasAndReport) or

ispresent(v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport.intraAndInterF_MeasAndReport) or

ispresent(v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport.intraAndInterF_MeasAndReport)) {

f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_intraAndInterF_MeasAndReport to false is inconsistent with reported capabilities for a Rel-15 and above UE");

}

}

…

}

/*

* @desc      Function to check PICS within optional groups of fields matching with ifpresent in Rel-15.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

*/

function fl_Check_PICSinOptionalGroups_NR_r15 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

fl_Check_RootPICSinOptionalGroups_NR_r15(v_Received_NRCapabilityMsg);

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension)) {  // UE_NR_Capability_v1530

if (pc_inactiveState) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1530, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.interRAT_Parameters)) {  // UE_NR_Capability_v1530.interRAT_Parameters

if (pc_RS_SINR_MeasEUTRA or pc_interRAT_NR_ToENDC) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1530.interRAT_Parameters, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) {  // UE_NR_Capability_v1560

if (pc_reducedCP_Latency) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1560, no PICS shall be set in a branch when parent is not present");

}

}

}; //end of fl_Check_PICSinOptionalGroups_NR_r15

/*

* @desc      Function to check PICS within optional groups of fields matching with ifpresent in Rel-16.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

*/

function fl_Check_PICSinOptionalGroups_NR_r16 (UE_NR_Capability p_NRCapability) {

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

fl_Check_RootPICSinOptionalGroups_NR_r16(v_Received_NRCapabilityMsg);

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) {  // UE_NR_Capability_v1610

if (pc_NR_dl_DedicatedMessageSegmentation or pc_resumeWithSCG_Config_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1610, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.ue_BasedPerfMeas_Parameters_r16)) {  // UE_NR_Capability_v1610.ue_BasedPerfMeas_Parameters_r16

if (pc_barometer_r16 or pc_immMeasBT_r16 or pc_immMeasWLAN_r16 or pc_loggedMeasBT_r16 or pc_loggedMeasurements_r16 or pc_loggedMeasWLAN_r16 or pc_orientation_r16 or pc_speed_r16 or pc_GNSS_location_r16 or pc_PDCP_Delay_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1610.ue_BasedPerfMeas_Parameters_r16, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.son_Parameters_r16)) {  // UE_NR_Capability_v1610.son_Parameters_r16

if (pc_rachReport_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1610.son_Parameters_r16, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) {  // UE_NR_Capability_v1650

if (pc_NR_mpsPriorityIndication_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1650, no PICS shall be set in a branch when parent is not present");    // UE_NR_Capability_v1650

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) {  // UE_NR_Capability_v1690

if (pc_NR_UL_Segmentation) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1690, no PICS shall be set in a branch when parent is not present");    // UE_NR_Capability_v1690

}

}

}; //end of fl_Check_PICSinOptionalGroups_NR_r16

/*

* @desc      Function to check PICS within optional groups of fields matching with ifpresent in Rel-17.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

*/

function fl_Check_PICSinOptionalGroups_NR_r17 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

fl_Check_RootPICSinOptionalGroups_NR_r17(v_Received_NRCapabilityMsg);

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.ue_BasedPerfMeas_Parameters_r16)) {  // UE_NR_Capability_v1610.ue_BasedPerfMeas_Parameters_r16

if (pc_multiple_CEF_Report_r17 or pc_earlyMeasLog_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1610.ue_BasedPerfMeas_Parameters_r16, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.son_Parameters_r16)) {  // UE_NR_Capability_v1610.son_Parameters_r16

if (pc_rlfReportCHO_r17 or pc_rlfReportDAPS_r17 or pc_success_HO_Report_r17 or pc_twoStepRACH_Report_r17 or pc_onDemandSI_Report_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1610.son_Parameters_r16, no PICS shall be set in a branch when parent is not present");

}

}

if (not isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) {  // UE_NR_Capability_v1700

if (pc_supportOfRedCap_r17 or pc_supportOf16DRB_RedCap_r17 or pc_ra_SDT_r17 or pc_srb_SDT_r17 or pc_musim_GapPreference_r17 or pc_musimLeaveConnected_r17 or pc_sliceInfoforCellReselection_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling UE_NR_Capability_v1700, no PICS shall be set in a branch when parent is not present");    // UE_NR_Capability_v1700

}

}

}; //end of fl_Check_PICSinOptionalGroups_NR_r17

/*

* @desc      Function to check some Rel 15 UE NR capabilities. Prints out all the PICS wrongly set specific within r15 v30 and v40

* @param     p_NRCapability

* @param     p_Configuration_Type

*/

function fl_NRCap_CommonChecks_r15(UE_NR_Capability p_NRCapability,

NR_Configuration_Type p_Configuration_Type)

{ // @sic R5-213468 R5-213482 R5-227571 R5-227572 R5-227573 R5-231410 sic@

var UE_NR_Capability v_Received_NRCapabilityMsg;

v_Received_NRCapabilityMsg := p_NRCapability;

fl_Check_Rf_Parameters_NRBand_PICS_Supp(v_Received_NRCapabilityMsg, p_Configuration_Type);  //rf-Parameters and NR supported bands fully checked. Note 8, 9, 20 and 21 of Table UE-NR-Capability

fl_Check_MeasAndMobParametersCommon(v_Received_NRCapabilityMsg, p_Configuration_Type);      //check conditions as per TS 38.306 clause 4.2.9

f_Check_featureSets(v_Received_NRCapabilityMsg, p_Configuration_Type);            // @sic R5-225384 sic@ featureSets IEs

fl_Check_NR_Notes(v_Received_NRCapabilityMsg);               // @sic R5-197070 sic@ Notes 1 to 18 on Table UE-NR-Capability

fl_Check_NR_Capability_1540(v_Received_NRCapabilityMsg);     // UE-NR-Capability-1540 IEs checked

fl_Check_PICSinOptionalGroups_NR_r15(v_Received_NRCapabilityMsg);     // PICS set in optional group of IEs checked in Rel-15

} // end of fl_NRCap_CommonChecks_r15

/*

* @desc      Function to check applicable Rel 16 UE NR capabilities plus legacy releases

* @param     p_NRCapability

* @param     p_Configuration_Type

*/

function fl_NRCap_CommonChecks_r16(UE_NR_Capability p_NRCapability,

NR_Configuration_Type p_Configuration_Type)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

fl_NRCap_CommonChecks_r15(v_Received_NRCapabilityMsg, p_Configuration_Type);

fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg); // Note 22 and Input(s) in field UE-NR-Capability-v1610 for Note 19, 23, 24, 25

if (p_Configuration_Type == NR_SA) { //@sic R5-225384 sic@

fl_Check_sidelinkParameters_r16(v_Received_NRCapabilityMsg);  // Only in Test Case 8.1.5.1.1.NR5GC

}

fl_Check_PICSinOptionalGroups_NR_r16(v_Received_NRCapabilityMsg);     // PICS set in optional group of IEs checked in Rel-16

} // end of fl_NRCap_CommonChecks_r16

/*

* @desc      Function to check applicable Rel 17 UE NR capabilities plus earlier releases

* @param     p_NRCapability

* @param     p_Configuration_Type

*/

function fl_NRCap_CommonChecks_r17(UE_NR_Capability p_NRCapability,

NR_Configuration_Type p_Configuration_Type)

{ //@sic R5-227571 R5-231410 sic@

var UE_NR_Capability v_Received_NRCapabilityMsg;

v_Received_NRCapabilityMsg := p_NRCapability;

fl_NRCap_CommonChecks_r16(v_Received_NRCapabilityMsg, p_Configuration_Type);

f_PEI_SubgroupingSupportBandList_r17(v_Received_NRCapabilityMsg);

if (p_Configuration_Type == NR_SA) { //@sic R5-225384 sic@

fl_SupportOfRedCap_IEs(v_Received_NRCapabilityMsg);  // Only in Test Case 8.1.5.1.1.NR5GC

}

fl_Check_PICSinOptionalGroups_NR_r17(v_Received_NRCapabilityMsg);     // PICS set in optional group of IEs checked in Rel-17

} // end of fl_NRCap_CommonChecks_r17

/*

* @desc      Function to check PICS within PDCP_Parameters matching with ifpresent in Rel-15.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_PDCP_Parameters_NR_r15 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.outOfOrderDelivery)) and pc_outOfOrderDelivery) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.outOfOrderDelivery but pc_outOfOrderDelivery is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.shortSN)) and pc_shortSN) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.shortSN but pc_shortSN is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.pdcp_DuplicationSRB)) and pc_pdcp_DuplicationSplitSRB) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.pdcp_DuplicationSRB but pc_pdcp_DuplicationSplitSRB is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.pdcp_DuplicationMCG_OrSCG_DRB)) and  pc_pdcp_DuplicationMCG_OrSCG_DRB) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.pdcp_DuplicationMCG_OrSCG_DRB but pc_pdcp_DuplicationMCG_OrSCG_DRB is set");

}

}

/*

* @desc      Function to check PICS within PDCP_Parameters matching with ifpresent in Rel-16.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_PDCP_Parameters_NR_r16 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.ehc_r16)) and pc_NR_ehc_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.ehc_r16 but pc_NR_ehc_r16 is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.pdcp_DuplicationMoreThanTwoRLC_r16)) and pc_pdcp_DuplicationMoreThanTwoRLC_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.pdcp_DuplicationMoreThanTwoRLC_r16 but pc_pdcp_DuplicationMoreThanTwoRLC_r16 is set");

}

}

/*

* @desc      Function to check PICS within PDCP_Parameters matching with ifpresent in Rel-17.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_PDCP_Parameters_NR_r17 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.longSN_RedCap_r17)) and pc_longSN_RedCap_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.longSN_RedCap_r17 but pc_longSN_RedCap_r17 is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.pdcp_Parameters.udc_r17)) and pc_NR_udc_continueUDC_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability pdcp_Parameters.udc_r17 but pc_NR_udc_continueUDC_r17 is set");

}

}

/*

* @desc      Function to check PICS within RLC_Parameters matching with ifpresent in Rel-15.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_RLC_Parameters_NR_r15 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.rlc_Parameters.am_WithShortSN)) and pc_am_WithShortSN) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability rlc_Parameters.am_WithShortSN but pc_am_WithShortSN is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.rlc_Parameters.um_WithShortSN)) and pc_um_WithShortSN) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability rlc_Parameters.um_WithShortSN but pc_um_WithShortSN is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.rlc_Parameters.um_WithLongSN)) and pc_um_WithLongSN) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability rlc_Parameters.um_WithLongSN but pc_um_WithLongSN is set");

}

}

/*

* @desc      Function to check PICS within RLC_Parameters matching with ifpresent in Rel-17.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_RLC_Parameters_NR_r17 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.rlc_Parameters.am_WithLongSN_RedCap_r17)) and (pc_supportOfRedCap_r17 and pc_am_WithLongSN)) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability rlc_Parameters.am_WithLongSN_RedCap_r17 but pc_supportOfRedCap_r17 and pc_am_WithLongSN are both set");

}

}

/*

* @desc      Function to check PICS within RF_Parameters matching with ifpresent in Rel-16.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_RF_Parameters_NR_r16 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.rf_Parameters.extendedBand_n77_r16)) and pc_extendedBand_n77_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability rf_Parameters.extendedBand_n77_r16 but pc_extendedBand_n77_r16 is set");

}

}

/*

* @desc      Function to check PICS within RF_Parameters matching with ifpresent in Rel-17.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_RF_Parameters_NR_r17 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.rf_Parameters.extendedBand_n77_2_r17)) and pc_extendedBand_n77_2_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability rf_Parameters.extendedBand_n77_2_r17 but pc_extendedBand_n77_2_r17 is set");

}

}

/*

* @desc      Function to check PICS within MeasAndMobParametersCommon matching with ifpresent in Rel-15.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_MeasAndMobParametersCommon_NR_r15 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.eventB_MeasAndReport)) and pc_eventB_MeasAndReport) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.eventB_MeasAndReport but pc_eventB_MeasAndReport is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.eutra_CGI_Reporting)) and pc_eutra_CGI_Reporting) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.eutra_CGI_Reporting but pc_eutra_CGI_Reporting is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.nr_CGI_Reporting)) and pc_nr_CGI_Reporting) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.nr_CGI_Reporting but pc_nr_CGI_Reporting is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.independentGapConfig)) and pc_independentGapConfig) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.independentGapConfig but pc_independentGapConfig is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.periodicEUTRA_MeasAndReport)) and pc_periodicEUTRA_MeasAndReport) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.periodicEUTRA_MeasAndReport but pc_periodicEUTRA_MeasAndReport is set");

}

}

/*

* @desc      Function to check PICS within MeasAndMobParametersCommon matching with ifpresent in Rel-16.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_MeasAndMobParametersCommon_NR_r16 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.nr_CGI_Reporting_NPN_r16)) and pc_nr_CGI_Reporting_NPN_r16) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.nr_CGI_Reporting_NPN_r16 but pc_nr_CGI_Reporting_NPN_r16 is set");

}

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.idleInactiveEUTRA_MeasReport_r16)) and pc_idleInactiveEUTRA_MeasReport) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.idleInactiveEUTRA_MeasReport_r16 but pc_idleInactiveEUTRA_MeasReport is set");

}

}

/*

* @desc      Function to check PICS within MeasAndMobParametersCommon matching with ifpresent in Rel-17.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_MeasAndMobParametersCommon_NR_r17 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if ((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.rrm_RelaxationRRC_ConnectedRedCap_r17)) and pc_rrm_RelaxationRRC_ConnectedRedCap_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability measAndMobParametersCommon.rrm_RelaxationRRC_ConnectedRedCap_r17 but pc_rrm_RelaxationRRC_ConnectedRedCap_r17 is set");

}

}

/*

* @desc      Function to check handoverInterF PICS within MeasAndMobParameters matching with ifpresent. This completes Note 14 and Note 15.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_OptionalPICS_MeasAndMobParametersCommon_handoverInterF_NR_r15 (UE_NR_Capability p_NRCapability)

{

var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability;

if (((not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff.handoverInterF)) and (not isvalue(v_Received_NRCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.handoverInterF))  and (not isvalue(v_Received_NRCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.handoverInterF))  and (not isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersXDD_Diff.handoverInterF)) and (not isvalue(v_Received_NRCapabilityMsg.fdd_Add_UE_NR_Capabilities.measAndMobParametersXDD_Diff.handoverInterF)) and (not isvalue(v_Received_NRCapabilityMsg.tdd_Add_UE_NR_Capabilities.measAndMobParametersXDD_Diff.handoverInterF))) and (pc_handoverInterF))

{

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling Capability handoverInterF but pc_handoverInterF is set");

}

}

/*

* @desc      Function to wrap up all checks of fields matching with ifpresent in Rel-15.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_RootPICSinOptionalGroups_NR_r15 (UE_NR_Capability p_NRCapability)

{

fl_Check_OptionalPICS_PDCP_Parameters_NR_r15(p_NRCapability);

fl_Check_OptionalPICS_RLC_Parameters_NR_r15(p_NRCapability);

fl_Check_OptionalPICS_MeasAndMobParametersCommon_NR_r15(p_NRCapability);

fl_Check_OptionalPICS_MeasAndMobParametersCommon_handoverInterF_NR_r15(p_NRCapability);

}

/*

* @desc      Function to wrap up all checks of fields matching with ifpresent in Rel-16.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_RootPICSinOptionalGroups_NR_r16 (UE_NR_Capability p_NRCapability)

{

fl_Check_OptionalPICS_PDCP_Parameters_NR_r16(p_NRCapability);

fl_Check_OptionalPICS_RF_Parameters_NR_r16(p_NRCapability);

fl_Check_OptionalPICS_MeasAndMobParametersCommon_NR_r16(p_NRCapability);

}

/*

* @desc      Function to wrap up all checks of fields matching with ifpresent in Rel-17.

* @param     p_NRCapability

* @status    APPROVED (ENDC, NR5GC)

*/

function fl_Check_RootPICSinOptionalGroups_NR_r17 (UE_NR_Capability p_NRCapability)

{

fl_Check_OptionalPICS_PDCP_Parameters_NR_r17(p_NRCapability);

fl_Check_OptionalPICS_RLC_Parameters_NR_r17(p_NRCapability);

fl_Check_OptionalPICS_RF_Parameters_NR_r17(p_NRCapability);

fl_Check_OptionalPICS_MeasAndMobParametersCommon_NR_r17(p_NRCapability);

}

Module NR_AuxiliaryCapCheckFunctions

/*

* @desc      Function that returns whether one of the three inputs is true. Used in capability check test cases to get whether an optional feature under a Note in the specs is signaled in one its three possible branches.

* @param     p_Rootfield

* @param     p_LeftField

* @param     p_RightField

* @return    boolean

*/

function f_NR_AnyOfThree_SignaledbyUE(boolean p_Rootfield, boolean p_LeftField, boolean p_RightField) return boolean

{

return (p_Rootfield or p_LeftField or p_RightField);

};