3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240565

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3525 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the case of FDD, the SR Resource Config is set as srPeriodicity = sl10 and srOffset = 9 which means every 10ms at SF = 9, UE is allowed to send an SR. In response to the SR, if an UL Grant is provided, the UE tries and loopbacks data leading to TC failure. The config for SR intersects with the reception of the HARQ ACK which is also set as SF = 9. In this TC, DL SPS is tested, hence, the default onSR UL grant config needs to be stopped to avoid any SR being entertained when the Tesbody is being executed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The default onSR UL Grant Config is stopped in function f_TC_7_1_1_6_1_NR_TestBody1 and enabled before the RRC Reconfiguration procedure at Step 29-30 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case for FDD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.NR5GC, 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | For the case of FDD, the SR Resource Config is set as srPeriodicity = sl10 and srOffset = 9 which means every 10ms at SF = 9, UE is allowed to send an SR. In response to the SR, if an UL Grant is provided, the UE tries and loopbacks data leading to TC failure. The config for SR intersects with the reception of the HARQ ACK which is also set as SF = 9. In this TC, DL SPS is tested, hence, the default onSR UL grant config needs to be stopped to avoid any SR being entertained when the Tesbody is being executed. |
| Summary of change | The default onSR UL Grant Config is stopped in function f_TC_7_1_1_6_1_NR_TestBody1 and enabled before the RRC Reconfiguration procedure at Step 29-30 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comments | Accepted as improvement |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> timer t_Watchdog := 10.0; // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> timer t_Watchdog := 10.0; //Stop UL Grant WA# f_NR_ULGrantConfiguration_Stop(nr_Cell1); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); <<SKIPPED CODE>> }

## Change 2

| Template name | f_TC_7_1_1_6_1_NR5GC |
| --- | --- |
| Reason for change | For the case of FDD, the SR Resource Config is set as srPeriodicity = sl10 and srOffset = 9 which means every 10ms at SF = 9, UE is allowed to send an SR. In response to the SR, if an UL Grant is provided, the UE tries and loopbacks data leading to TC failure. The config for SR intersects with the reception of the HARQ ACK which is also set as SF = 9. In this TC, DL SPS is tested, hence, the default onSR UL grant config needs to be stopped to avoid any SR being entertained when the Tesbody is being executed. |
| Summary of change | The default onSR UL Grant Config is stopped in function f_TC_7_1_1_6_1_NR_TestBody1 and enabled before the RRC Reconfiguration procedure at Step 29-30 |
| TTCN module | MAC_SPS_NR5GC.ttcn |
| MCC160 Comments | Accepted as improvement, but implemented in function f_TC_7_1_1_6_1_NR_TestBody1 before return statement |

Before Change:

function f_TC_7_1_1_6_1_NR5GC() runs on NR5GC_PTC { // Correct handling of DL assignment / Semi-persistent case <<SKIPPED CODE>> //@sic R5s240268 sic@ v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR5GC() runs on NR5GC_PTC { // Correct handling of DL assignment / Semi-persistent case <<SKIPPED CODE>> //@sic R5s240268 sic@ v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //Start default UL Grant WA# f_NR_ULGrantConfiguration_Start(nr_Cell1); //@siclog steps 29-30 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; <<SKIPPED CODE>>

## Additional TF160 Change Required

| Template name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | At Step 17, there is a transmission of MAC PDU with CRC invalid. This is then followed by 3 re-transmissions at Step 19 with CRC Valid. In total, there are 4 transmissions to the UE. The reception of the HARQ NACK / ACK at Step 18 and Step 21 should consider all the transmissions to the UE |
| Summary of change | The condition for checking the aggregation of the v_Ack_Count + v_Nack_Count should be < 4 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comments | Accepted |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> //@sic R5s240563 sic@ // @siclog "Step 18 & 21" siclog@ alt { []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_Any )) { v_Ack_Count := v_Ack_Count +1; if ((v_Ack_Count + v_Nack_Count) < 3) { repeat; } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_Any )) { v_Nack_Count := v_Nack_Count + 1; if ((v_Ack_Count + v_Nack_Count) < 3) { repeat; } else if (v_Ack_Count == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: UE did not send HARQ ACK"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } } <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> //@sic R5s240563 sic@ // @siclog "Step 18 & 21" siclog@ alt { []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_Any )) { v_Ack_Count := v_Ack_Count +1; if ((v_Ack_Count + v_Nack_Count) < 4) { //WA# repeat; } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_Any )) { v_Nack_Count := v_Nack_Count + 1; if ((v_Ack_Count + v_Nack_Count) < 4) { //WA# repeat; } else if (v_Ack_Count == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: UE did not send HARQ ACK"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } } <<SKIPPED CODE>> }