3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240570

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3530 | rev | - | Current version: | 17.11.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_Check_idleInactiveNR_MeasReport_FRX_Diff |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint,TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In error cases, it should be if pc_idleInactiveNR_MeasReport is false: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use pc_idleInactiveNR_MeasReport instand of pc_handoverInterF in verdict. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Verdict may incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1	Corrections required	3

1.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: Ting Gao

gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | fl_Check_idleInactiveNR_MeasReport_FRX_Diff |
| --- | --- |
| Reason for change | In error cases, it should be if pc_idleInactiveNR_MeasReport is false: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE |
| Summary of change | Use pc_idleInactiveNR_MeasReport instand of pc_handoverInterF |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_Check_idleInactiveNR_MeasReport_FRX_Diff(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-232202 sic@ var boolean v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport := false; var boolean v_FR1_Add_idleInactiveNR_MeasReport := false; var boolean v_FR2_Add_Cap_idleInactiveNR_MeasReport := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersFRX_Diff v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common; var MeasAndMobParametersFRX_Diff v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport; var MeasAndMobParametersFRX_Diff v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport; if (ispresent(v_ReceivedCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff)) { v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common := v_ReceivedCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) { v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport := v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) { v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport := v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff; } if(pc_idleInactiveNR_MeasReport) { // check indication of MeasAndMobParametersFRX_Diff_Common if (ispresent(v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common.idleInactiveNR_MeasReport_r16)) { v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport := true; } // check indication of FR1 IE if (ispresent(v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16)) { v_FR1_Add_idleInactiveNR_MeasReport := true; } // check indication of FR2 IE if (ispresent(v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16)) { v_FR2_Add_Cap_idleInactiveNR_MeasReport := true; } if (not (f_NR_AnyOfThree_SignaledbyUE(v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport, v_FR1_Add_idleInactiveNR_MeasReport, v_FR2_Add_Cap_idleInactiveNR_MeasReport))) //@sic R5s240547 sic@ {// inconclusive case: UE does not report any IE for idleInactiveNR_MeasReport but pc_idleInactiveNR_MeasReport is set f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_idleInactiveNR_MeasReport to true is inconsistent with reported capabilities"); } if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport) or v_FR1_Add_idleInactiveNR_MeasReport or v_FR2_Add_Cap_idleInactiveNR_MeasReport)) { // fail cases on Single frequency range f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent with pc_idleInactiveNR_MeasReport on Single frequency range"); } else if(f_NR_FreqRangeFr1AndFR2()) //FR1&FR2 case { if (not v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport) { if (v_FR1_Add_idleInactiveNR_MeasReport and v_FR2_Add_Cap_idleInactiveNR_MeasReport) // fail case: both FR1 and FR2 capabilities are reported and no IE in measAndMobParameters.measAndMobParametersFRX_Diff { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } } else if (v_FR1_Add_idleInactiveNR_MeasReport or v_FR2_Add_Cap_idleInactiveNR_MeasReport) // fail cases: if IE in measAndMobParameters.measAndMobParametersFRX_Diff is reported but FR1 and/or FR2 capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE if (ispresent(v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common.idleInactiveNR_MeasReport_r16) or ispresent(v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16) or ispresent(v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16)) { f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_handoverInterF to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }

After Change

function fl_Check_idleInactiveNR_MeasReport_FRX_Diff(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-232202 sic@ var boolean v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport := false; var boolean v_FR1_Add_idleInactiveNR_MeasReport := false; var boolean v_FR2_Add_Cap_idleInactiveNR_MeasReport := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersFRX_Diff v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common; var MeasAndMobParametersFRX_Diff v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport; var MeasAndMobParametersFRX_Diff v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport; if (ispresent(v_ReceivedCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff)) { v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common := v_ReceivedCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) { v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport := v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) { v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport := v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff; } if(pc_idleInactiveNR_MeasReport) { // check indication of MeasAndMobParametersFRX_Diff_Common if (ispresent(v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common.idleInactiveNR_MeasReport_r16)) { v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport := true; } // check indication of FR1 IE if (ispresent(v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16)) { v_FR1_Add_idleInactiveNR_MeasReport := true; } // check indication of FR2 IE if (ispresent(v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16)) { v_FR2_Add_Cap_idleInactiveNR_MeasReport := true; } if (not (f_NR_AnyOfThree_SignaledbyUE(v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport, v_FR1_Add_idleInactiveNR_MeasReport, v_FR2_Add_Cap_idleInactiveNR_MeasReport))) //@sic R5s240547 sic@ {// inconclusive case: UE does not report any IE for idleInactiveNR_MeasReport but pc_idleInactiveNR_MeasReport is set f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_idleInactiveNR_MeasReport to true is inconsistent with reported capabilities"); } if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport) or v_FR1_Add_idleInactiveNR_MeasReport or v_FR2_Add_Cap_idleInactiveNR_MeasReport)) { // fail cases on Single frequency range f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent with pc_idleInactiveNR_MeasReport on Single frequency range"); } else if(f_NR_FreqRangeFr1AndFR2()) //FR1&FR2 case { if (not v_MeasAndMobParamFRX_Diff_idleInactiveNR_MeasReport) { if (v_FR1_Add_idleInactiveNR_MeasReport and v_FR2_Add_Cap_idleInactiveNR_MeasReport) // fail case: both FR1 and FR2 capabilities are reported and no IE in measAndMobParameters.measAndMobParametersFRX_Diff { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } } else if (v_FR1_Add_idleInactiveNR_MeasReport or v_FR2_Add_Cap_idleInactiveNR_MeasReport) // fail cases: if IE in measAndMobParameters.measAndMobParametersFRX_Diff is reported but FR1 and/or FR2 capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE if (ispresent(v_MeasAndMobParamFRX_idleInactiveNR_MeasReport_Common.idleInactiveNR_MeasReport_r16) or ispresent(v_FR1_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16) or ispresent(v_FR2_Add_UE_NR_Cap_idleInactiveNR_MeasReport.idleInactiveNR_MeasReport_r16)) { f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_idleInactiveNR_MeasReport to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }