3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240572

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4855 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to current TTCN implementation, rrcConnectionRelease with redirection to Ncell1 is transmitted at Step 11, and then at Step 12 the power level for Ncell23 is changed to non-suitable cell and Ncell 1 set to suitable cell. This results in two issues. First, if the NCell23 power is dropped immediately, the rrcConnectionRelease may not be received by UE. Second, the change of power levels will need some time, the test model allows up to 500ms for this. If the UE received the rrcConnectionRelease, then it may try to camp on Ncell 1 immediately, and this can happen while the SS is updating power levels at Step 12 (within 500ms). This results in testcase failure and needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of updating the power levels after transmitting rrcConnectionRelease, implemented the change of power levels after receiving the TAU REQ at Step 13. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | fl_TC_22_5_20_Body |
| --- | --- |
| Reason for change | According to current TTCN implementation, rrcConnectionRelease with redirection to Ncell1 is transmitted at Step 11, and then at Step 12 the power level for Ncell23 is changed to non-suitable cell and Ncell 1 set to suitable cell. This results in two issues. First, if the NCell23 power is dropped immediately, the rrcConnectionRelease may not be received by UE. Second, the change of power levels will need some time, the test model allows up to 500ms for this. If the UE received the rrcConnectionRelease, then it may try to camp on Ncell 1 immediately, and this can happen while the SS is updating power levels at Step 12 (within 500ms). This results in testcase failure and needs to be addressed. |
| Summary of change | Instead of updating the power levels after transmitting rrcConnectionRelease, implemented the change of power levels after receiving the TAU REQ at Step 13. Also commented out some code which is not needed anymore. A Prose CR will be raised for this. |
| TTCN module | ENBIOT_NAS.ttcn |
| MCC160 Comment | Accepted as this makes the prose and TTCN implementation simpler, subject to prose CR approval |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 10" siclog@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell23, cr_EstablishmentCause_NB_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_508_TAU_Request (tsc_EpsUpdate_TaUpdate, f_NBIOT_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, cr_AdditionalUpdateType_CIOT ifpresent))); if (not f_Check_UE_NetworkCap_CIOT (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, CP_Backoff)) { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Network Cap failed CIOT bit check"); } // @sic R5-245482 sic@ //@siclog "Step 10A" siclog@ //SS configures Ncell 1 as suitable cell and Ncell 23 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell1, tsc_SuitableCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); //@siclog "Step 11" siclog@ //SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1). v_TimingAtT := f_NBIOT_GetNextSearchSpace(nbiot_Cell23, 300); // @sic R5s230342 sic@ v_TimingInfoNCell1 := f_NBIOT_TimingOtherCell(nbiot_Cell23, cs_TimingInfo_EUTRA_NB(v_TimingAtT), nbiot_Cell1); v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList, v_TimingInfoNCell1, nbiot_Cell1 ); // @sic R5s230342 sic@ // @sic R5s23525 sic@ f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_EUTRA_NB(v_TimingAtT)); // @sic R5-206436, R5s210634, R5s230342 sic@ //@siclog "Step 12" siclog@ //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); // @sic R5s190574 sic@ //@siclog "Step 13" siclog@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell1, // @sic R5s190574 sic@ cr_EstablishmentCause_NB_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_508_TAU_Request (tsc_EpsUpdate_TaUpdate, // no specific message contents, so default value f_NBIOT_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, cr_AdditionalUpdateType_CIOT ifpresent))); if (not f_Check_UE_NetworkCap_CIOT (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, CP_Backoff)) { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Network Cap failed CIOT bit check"); } else{ f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 13"); } v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; // Send this back in the Accept //@siclog "Step 14" siclog@ // SS responds with a TRACKING AREA UPDATE ACCEPT message omitting T3448. SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_TAU_Accept_Common(tsc_EpsUpdate_TaUpdate, f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti_Params_Cell1), cds_TAIListNonConsecutive_tlv(v_PLMN_Cell1, { bit2oct(v_TAC_Cell1) }), v_EPS_ContextStatus, omit, omit, f_GetNBIOTAdditionalUpdateResult(v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType), -, -, -, f_GetNBIOTNetworkFeatureSupport(CONTROL_PLANE))))); //@siclog "Step 15" siclog@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 10A" siclog@ //SS configures Ncell 1 as suitable cell and Ncell 23 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell1, tsc_SuitableCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); //@siclog "Step 11" siclog@ //SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1). /*WA#WA#WI1334077 WA#22_5_20 COMMENTED OUT v_TimingAtT := f_NBIOT_GetNextSearchSpace(nbiot_Cell23, 600); // @sic R5s230342 sic@ //WA#WI1334077 WA#22_5_20 v_TimingInfoNCell1 := f_NBIOT_TimingOtherCell(nbiot_Cell23, cs_TimingInfo_EUTRA_NB(v_TimingAtT), nbiot_Cell1); v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList, v_TimingInfoNCell1, nbiot_Cell1 ); // @sic R5s230342 sic@ // @sic R5s23525 sic@ f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_EUTRA_NB(v_TimingAtT)); // @sic R5-206436, R5s210634, R5s230342 sic@ */ f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL)); // @sic R5-206436, R5s210634, R5s230342 sic@ //WA#WI1334077 WA#22_5_20 //@siclog "Step 12" siclog@ //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. /*WA#WI1334077 WA#22_5_20 COMMENTED OUT v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); */ // @sic R5s190574 sic@ //@siclog "Step 13" siclog@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell1, // @sic R5s190574 sic@ cr_EstablishmentCause_NB_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_508_TAU_Request (tsc_EpsUpdate_TaUpdate, // no specific message contents, so default value f_NBIOT_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, cr_AdditionalUpdateType_CIOT ifpresent))); if (not f_Check_UE_NetworkCap_CIOT (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, CP_Backoff)) { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Network Cap failed CIOT bit check"); } else{ f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 13"); } //WA#WI1334077 WA#22_5_20 //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE), cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; // Send this back in the Accept //@siclog "Step 14" siclog@ // SS responds with a TRACKING AREA UPDATE ACCEPT message omitting T3448. SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_TAU_Accept_Common(tsc_EpsUpdate_TaUpdate, f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti_Params_Cell1), cds_TAIListNonConsecutive_tlv(v_PLMN_Cell1, { bit2oct(v_TAC_Cell1) }), v_EPS_ContextStatus, omit, omit, f_GetNBIOTAdditionalUpdateResult(v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType), -, -, -, f_GetNBIOTNetworkFeatureSupport(CONTROL_PLANE))))); //@siclog "Step 15" siclog@ <<SKIPPED CODE>>