3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240578

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3537 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 9.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The R5s240059 MCC alternative solution removes the UL grant start (steps 9-10) with the argument that it is not needed, because it is started in f_NR_RRC_ConnEst_DefWithNas. This is true, however the grant is started with ActTime=Now which may collide with GrantStop from f_NR_RRCRelease_Local which has explicit timing. Because in case the GrantStop gets activated after the grant start then the grant stays stopped. Therefore we need to ensure that such case does not happen and so the grant start needs to be eplicitly timed too with the same/later ActTime to avoid grant staying stopped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added start of UL grant after the f_NR_RRCRelease_Local during steps 9-10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 9.1.7.1 may occassionally fail on conformant UE due to collision of timings between UL grant start/stop during steps 9-10. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 TTCN-3 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_7_1_Body |
| --- | --- |
| Reason for change | The R5s240059 MCC alternative solution removes the UL grant start (steps 9-10) with the argument that it is not needed, because it is started in f_NR_RRC_ConnEst_DefWithNas. This is true, however the grant is started with ActTime=Now which may collide with GrantStop from f_NR_RRCRelease_Local which has explicit timing. Because in case the GrantStop gets activated after the grant start then the grant stays stopped. Therefore we need to ensure that such case does not happen and so the grant start needs to be eplicitly timed too with the same/later ActTime to avoid grant staying stopped. |
| Summary of change | Added start of UL grant after the f_NR_RRCRelease_Local during steps 9-10. |
| TTCN module | 9_1_7\Service_Request_NR5GC.ttcn |
| MCC160 Comment | Accepted as an improvement. Please see MCC160 implementation below. |

Before Change:

function fl_TC_9_1_7_1_Body() runs on NR5GC_PTC { … //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*, omit, omit)))) // @sic R5s221179 Baseline Moving sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA, tsc_NR_DelayBeforeIntraCellHO)); //@sic R5s200087, R5s200662 sic@, R5s220551 sic@ //@sic R5s240059 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); //@sic R5s200087 R5s240059 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } else { //@sic R5s240059 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } } } } //For loop //@siclog "Step 11" siclog@

After Change:

function fl_TC_9_1_7_1_Body() runs on NR5GC_PTC { … //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*, omit, omit)))) // @sic R5s221179 Baseline Moving sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA, tsc_NR_DelayBeforeIntraCellHO)); //@sic R5s200087, R5s200662 sic@, R5s220551 sic@ //@sic R5s240059 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); //@sic R5s200087 R5s240059 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } else { //@sic R5s240059 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } } } } //For loop //@siclog "Step 11" siclog@

## MCC160 implementation

<<skipped code>> for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*, omit, omit)))) // @sic R5s221179 Baseline Moving sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA, tsc_NR_DelayBeforeIntraCellHO)); //@sic R5s200087, R5s200662 sic@, R5s220551 sic@ //@sic R5s240059 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); //@sic R5s240578 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, -, -, -, -, -, false); //@sic R5s200087 R5s240059 R5s240578 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } else { //@sic R5s240059 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } } } } //For loop <<skipped code>>