3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240583

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3540 | rev | - | Current version: | 18.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 is modified according to R5s210149 for TC11.1. 1. But for test case without N26 interface(TC11.1.2, TC11.1.5 and TC11.1.6), multi PDN is required by UE, a completely new PDN connection is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the code to make sure a new PDN connection is handled all the time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2.NR5GC, 11.1.5.NR5GC, 11.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Functionname | f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 |
| --- | --- |
| Reason for change | This function is modified according to R5s210149 for TC11.1. But for test case without N26 interface(TC11.1.2, TC11.1.6 and TC11.1.6), multi PDN is required by UE, a completely new PDN connection is needed. |
| Summary of change | Delete the code to make sure a new PDN connection is handled all the time. |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment | Alternative solution implemented: add extra parameter to if statement to check if N26 is supported |

Before Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoFromUE, template (omit) AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, template (omit) UERadioCapId p_UERadioCapId:= omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@ var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var template (omit) ESM_Cause v_ESM_Cause := omit; var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; var boolean v_StartPDN := true; // @sic R5s210149 sic@ var template (value) ProtocolConfigOptions v_PcoToUE; var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1; // @sic R5s230279 sic@ var integer i; if (pc_IPv6 and pc_IPv4) { select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant case (IMS_DNN, Emergency_PDN) { // @sic R5s230279 sic@ v_IsIMS := true; v_BearerContextNumber := DEF_2; } } v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId, v_PDN_Type, v_IsIMS); v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PDN_Type, // @sic R5s220883 sic@ v_IsIMS, v_EpsDefaultBearerId, p_ExtdPcoFromUE, p_PcoFromUE, p_N1disabled)); // @sic R5-227443 sic@ if (isvalue (p_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (isvalue(p_PduInfoFromNR)) { // @sic R5s210149 sic@ // The UE has just moved from NR. If so, data from NR should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) { if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) { // Data from NR should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function break; } } } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4))); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex); f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, v_EpsDefaultBearerId, tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, v_APN_Tx, // @sic R5-206296 sic@ v_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, -, -, v_BearerContextNumber, p_EMM_Cause, omit, p_DCN_Id, omit, -, p_UERadioCapId); // @sic R5-216280 sic@ return v_EpsDefaultBearerId; }

After Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoFromUE, template (omit) AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, template (omit) UERadioCapId p_UERadioCapId:= omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@ var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var template (omit) ESM_Cause v_ESM_Cause := omit; var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; var boolean v_StartPDN := true; // @sic R5s210149 sic@ var template (value) ProtocolConfigOptions v_PcoToUE; var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1; // @sic R5s230279 sic@ var integer i; if (pc_IPv6 and pc_IPv4) { select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant case (IMS_DNN, Emergency_PDN) { // @sic R5s230279 sic@ v_IsIMS := true; v_BearerContextNumber := DEF_2; } } v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId, v_PDN_Type, v_IsIMS); v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PDN_Type, // @sic R5s220883 sic@ v_IsIMS, v_EpsDefaultBearerId, p_ExtdPcoFromUE, p_PcoFromUE, p_N1disabled)); // @sic R5-227443 sic@ if (isvalue (p_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4))); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex); f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, v_EpsDefaultBearerId, tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, v_APN_Tx, // @sic R5-206296 sic@ v_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, -, -, v_BearerContextNumber, p_EMM_Cause, omit, p_DCN_Id, omit, -, p_UERadioCapId); // @sic R5-216280 sic@ return v_EpsDefaultBearerId; }

#### MCC160 Implementation

function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId,

EUTRA_ATTACH_Type p_ForcedAttach,

template (omit) AdditionalUpdateType p_AdditionalUpdateType,

ProcedureTransactionIdentifier p_EPS_TI,

boolean p_IPv4AllocationViaNasFlag,

template (omit) ProtocolConfigOptions p_PcoFromUE,

template (omit) AccessPointName p_APN,

template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact,

template (omit) GPRS_Timer p_T3402 := omit,

template (omit) GPRS_Timer p_T3423 := omit,

template (omit) PLMN_List p_EquivalentPlmnList := omit,

integer p_NoOfEmergencyNumbers := 0,

template (omit) EMM_Cause p_EMM_Cause := omit,

template (omit) DCN_ID p_DCN_Id := omit,

template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit,

template (omit) UERadioCapId p_UERadioCapId:= omit,

boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity

{

var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed();

var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach);

var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId);

var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams);

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity);

var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach);

var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId);

var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN);

var template (omit) EmergNumList v_EmergNumList := omit;

var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach);

var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@

var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type);  // @sic R5-206296 sic@

var boolean v_IsIMS := false;

var EPS_BearerIdentity v_EpsDefaultBearerId;

var template (omit) ESM_Cause v_ESM_Cause := omit;

var template (value) PDN_Address v_PDN_Address;

var EmergencyNumList v_EmergencyNumList;

var boolean v_StartPDN := true; // @sic R5s210149 sic@

var template (value) ProtocolConfigOptions v_PcoToUE;

var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1;  // @sic R5s230279 sic@

var InterworkWithoutN26 v_IwkN26 := f_EUTRA38_GetNetworkSupport();

var integer i;

if (pc_IPv6 and pc_IPv4) {

select (v_PDN_TypeToBeUsed) {

case (forceIPv4only)          { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); }     // PDN type IPv4 only allowed

case (forceIPv6only)          { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); }     // PDN type IPv6 only allowed

case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; }

}

}

if (p_NoOfEmergencyNumbers > 0) {

v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList());

f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers

v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList);

}

select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant

case (IMS_DNN, Emergency_PDN) {  // @sic R5s230279 sic@

v_IsIMS := true;

v_BearerContextNumber := DEF_2;

}

}

v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId,

v_PDN_Type,

v_IsIMS);

v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId,

v_PDN_Type, // @sic R5s220883 sic@

v_IsIMS,

v_EpsDefaultBearerId,

p_ExtdPcoFromUE,

p_PcoFromUE,

p_N1disabled));  // @sic R5-227443 sic@

if (isvalue (p_ExtdPcoFromUE)) {

// FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present

}

if (isvalue(p_PduInfoFromNR and v_IwkN26 == NOT_SUPPORTED)) { // @sic R5s210149 sic@

// The UE has just moved from NR. If so, data from NR should be used, rather than created anew

for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) {

if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) {

// Data from NR should be used, rather than created anew

v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function

break;

}

}

}

if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@

f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4)));

}

v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex);

f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId,

v_EpsDefaultBearerId,

tsc_RRC_TI_Def,

p_EPS_TI,

v_AttachType,

v_PDN_Address,

v_Guti,

cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}),

v_LAI,

v_MSId,

v_ESM_Cause,

v_APN_Tx,  // @sic R5-206296 sic@

v_PcoToUE,

f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach),

p_T3412,

p_T3402,

p_T3423,

p_EquivalentPlmnList,

v_EmergNumList,

-,

-,

v_BearerContextNumber,

p_EMM_Cause,

omit,

p_DCN_Id,

omit,

-,

p_UERadioCapId); // @sic R5-216280 sic@

return v_EpsDefaultBearerId;

}