3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240584

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3541 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of this testcase, at Steps 29 - 47, the Registration REQ is expected to be integrity protected even when in earlier Steps 28a1-28a21 the USIM has been removed, and the re-inserted (Step 28a25). According to 33.501 cl 6.2.2.2 : <<skipped text>> The ME shall perform the generation of all other subsequent keys that are derived from the KAMF. Any 5G security context, KAUSF and KSEAF that are stored at the ME shall be deleted from the ME if: a) the USIM is removed from the ME when the ME is in power on state; b) the ME is powered up and the ME discovers that the USIM is different from the one which was used to create the 5G security context; <<skipped text>> Thus, at Steps 29-47, depending on whether USIM was removed earlier the Registration REQ might come unprotected. This needs to be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-worked TTCN implementation of Steps 29 – 47 to handle the USIM removal case. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Change 1

| Function name | fl_TC_9_1_10_6_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of this testcase, at Steps 29 - 47, the Registration REQ is expected to be integrity protected even when in earlier Steps 28a1-28a21 the USIM has been removed, and the re-inserted (Step 28a25). According to 33.501 cl 6.2.2.2 : <<skipped text>> The ME shall perform the generation of all other subsequent keys that are derived from the KAMF. Any 5G security context, KAUSF and KSEAF that are stored at the ME shall be deleted from the ME if: a) the USIM is removed from the ME when the ME is in power on state; b) the ME is powered up and the ME discovers that the USIM is different from the one which was used to create the 5G security context; <<skipped text>> Thus, at Steps 29-47, depending on whether USIM was removed earlier the Registration REQ might come unprotected. This needs to be handled. |
| Summary of change | Re-worked TTCN implementation of Steps 29 – 47 to handle the USIM removal case. Please see below. |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 29 - 47" //Steps 2 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA); } }//End of Module

After Change:

<<SKIPPED CODE>> //@siclog "Step 29 - 47" //Steps 2 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. if(pc_USIM_Removal) { f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, -, Initial_NoSecurity); //WA#WI1317003 WA#9_1_10_6 } else { f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA); } } }//End of Module