3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240588

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3545 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of function fl_TC_11_1_7_NR5GC_EUTRA_TestBody, following is initialized : if (match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_N1Disabled('58'O)) // @sic R5-227443 sic@ or not (isvalue(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X))) { // @sic R5-244663 sic@ v_N1disabled := true; } The issue here is that the template cdr_NR5GC_UENetworkCap_N1Disabled sets the N1 supported bit to 0, which means N1 is not supported. This means that when v_N1disabled = true, it means N1 is not supported. After this, following is implemented in TTCN : if (pc_EN_DC == true or v_N1disabled ) { // @sic R5-244663 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } This is not inline with R5-244663, which clearly indicates that for condition {ENDC or N1 mode supported} the IE UE Additional Security Capability is set as anyvalue. This needs to be corrected. Also, the current TTCN implementation sets v_UENWCap := cdr_NR5GC_UENetworkCap('58'O);, which results in v_UENWCap.cIoT_proSeCap being set to ('1'B & ? length (7)). This results in the ePCO support set as 1. This also needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of initializing : if (pc_EN_DC == true or v_N1disabled ) { // @sic R5-244663 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } Implemented if (pc_EN_DC == true or v_N1disabled == false ) { // @sic R5-244663 sic@ //WA#WI1336712 WA#11_1_7 v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } Also initialized v_UENWCap.cIoT_proSeCap := *; Please see below for detailed changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the tetstcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_11_1_7_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_11_1_7_NR5GC_EUTRA_TestBody, following is initialized : if (match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_N1Disabled('58'O)) // @sic R5-227443 sic@ or not (isvalue(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X))) { // @sic R5-244663 sic@ v_N1disabled := true; } The issue here is that the template cdr_NR5GC_UENetworkCap_N1Disabled sets the N1 supported bit to 0, which means N1 is not supported. This means that when v_N1disabled = true, it means N1 is not supported. After this, following is implemented in TTCN : if (pc_EN_DC == true or v_N1disabled ) { // @sic R5-244663 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } This is not inline with R5-244663, which clearly indicates that for condition {ENDC or N1 mode supported} the IE UE Additional Security Capability is set as anyvalue. This needs to be corrected. Also, the current TTCN implementation sets v_UENWCap := cdr_NR5GC_UENetworkCap('58'O);, which results in v_UENWCap.cIoT_proSeCap being set to ('1'B & ? length (7)). This results in the ePCO support set as 1. This also needs to be corrected. |
| Summary of change | Instead of initializing : if (pc_EN_DC == true or v_N1disabled ) { // @sic R5-244663 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } Implemented if (pc_EN_DC == true or v_N1disabled == false ) { // @sic R5-244663 sic@ //WA#WI1336712 WA#11_1_7 v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } Also initialized v_UENWCap.cIoT_proSeCap := *; |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | 1.Accepted 2. Accepted in principle, but it should be present, therefore value set to ? |

Before Change:

<<SKIPPED CODE>> v_UENWCap.cIoT_V2X := *; // Any allowed value for all bits, if present = * @sic R5-244725 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, cr_UE_AddSecurityCap_Any ifpresent, // @sic R5-244663 sic@ v_UENWCap); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_N1Disabled('58'O)) // @sic R5-227443 sic@ or not (isvalue(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X))) { // @sic R5-244663 sic@ v_N1disabled := true; } if (pc_EN_DC == true or v_N1disabled ) { // @sic R5-244663 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> v_UENWCap.cIoT_V2X := *; // Any allowed value for all bits, if present = * @sic R5-244725 sic@ v_UENWCap.cIoT_proSeCap := *; //WA#WI1336712 WA#11_1_7 v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, cr_UE_AddSecurityCap_Any ifpresent, // @sic R5-244663 sic@ v_UENWCap); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_N1Disabled('58'O)) // @sic R5-227443 sic@ or not (isvalue(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X))) { // @sic R5-244663 sic@ v_N1disabled := true; } if (pc_EN_DC == true or v_N1disabled == false ) { // @sic R5-244663 sic@ //WA#WI1336712 WA#11_1_7 v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; }<<SKIPPED CODE>>

## Additional Change

| Function name | fl_TC_11_1_7_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Step 25a4 specifies the UE network capability should be any allowed value |
| Summary of change | Pass cr_UENetworkCap_Any as a parameter to cr_TAU_Request_N1ToS1 at step 25a4 |

alt {// @sic R5-241691 sic@

// @siclog "Step 25a1" siclog@

[] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any))))

{

// @siclog "Step 25a2" siclog@

t_T1.stop;

// @siclog "Step 25a3-7" siclog@

// @sic R5-243591 sic@

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1,

tsc_RRC_TI_Def,

?,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_TAU_Request_N1ToS1(('000'B, '001'B, '010'B),

?,

cr_DRXparameter_Any ('5C'O) ifpresent,

f_GetAdditionalUpdateType(NORMAL),

-, -, -,

?,

-, -,

cr_UeRadioCap ifpresent,

-,

cr_UENetworkCap_Any('58'O),

cr_UE_AddSecurityCap_Any ifpresent,

cr_UEStatus_Any ifpresent)));  // @sic R5s240439 sic@

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1,

f_EUTRA_CellInfo_GetGuti(eutra_Cell1),

f_GetEPSTAUType(NORMAL),

v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus,

v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType,

NORMAL);

f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);  // @sic R5s240439 sic@

}

// @siclog "Step 25b1" siclog@

[] t_T1.timeout {}

}