3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240589

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4858 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For NB-IoT NTN, the Koffset = 550ms, so the f_Delay(0.5) is not enough to isolate the previous DL data and ACK procedure. So the ACK, which response to MAC TA, will be received at step 2. TC failed. - According to TC spec, the value of timeAlignmentTimerDedicated-r13 is sf5120. But for NB-IoT NTN, due to longer Koffset timer it takes more time for sending and receiving messages, timeAlignmentTimer will expire before Step 20 - The timing for NTN needs to be updated in order to avoid the expiry of timeAlignmentTimer at step 29 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Take the NTN Koffset into consider, enlarge the time delay according to Koffset. - Add step 8a and step 8b, SS transmits the extra Timing Advance Command indicates a TA index of 31 to UE. - Updated Step 26, 27 and Step 29 by 1 NPDCCH period, so that the Step 29 timing is within timeAlignmentTimer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the f_TC_22_3_1_2_NBIOT	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_2_NBIOT

| Function name | f_TC_22_3_1_2_NBIOT() |
| --- | --- |
| Reason for change | - For NB-IoT NTN, the Koffset = 550ms, so the f_Delay(0.5) is not enough to isolate the previous DL data and ACK procedure. So the ACK, which response to MAC TA, will be received at step 2. TC failed. - According to TC spec, the value of timeAlignmentTimerDedicated-r13 is sf5120. But for NB-IoT NTN, it takes more times to do message transmissions, so it is impossible to ensure timeAlignmentTimerDedicated not to time out before step 20 in UE side.So add extra Timing Advance Command at step 8a to ensure that. - The timing for NTN needs to be updated in order to avoid the expiry of timeAlignmentTimer at step 29 |
| Summary of change | - Take the NTN Koffset into consider, enlarge the time delay according to Koffset. - Add step 8a and step 8b, SS transmits the extra Timing Advance Command indicates a TA index of 31 to UE. - Updated Step 26, 27 and Step 29 by 1 NPDCCH period, so that the Step 29 timing is within timeAlignmentTimer |
| TTCN module | ttcn\develop\NBIOT\22_3\NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment | Accepted conditional prose CR approval In addition simplified assignment of v_NtnEnvironment using f_NBIOT_MobileInfo_IsNTN() function call |

Before Change:

function f_TC_22_3_1_2_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var EUTRA_ASN1_C_RNTI_Type v_C_RNTI; var template (value) TimingInfo_Type v_TimingInfo; var template (value) MAC_PDU_Type v_MAC_PDU; var template (value) NB_DciDlInfo_Type v_DciDlInfo; var MAC_SDUList_Type v_MAC_SDUList; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_TimingStart_Step21; var SubFrameTiming_Type v_TimingStart_Step24; var integer v_RlcSduSizeDL; var integer v_TBS; var integer v_Imcs; var integer v_Isf; var integer v_RepetitionNumber; var integer v_IncrementVTS; var integer v_TestLoopModeRepetitions := 0; // according to test case preamble var NB_SYSTEM_IND v_NB_SYSTEM_IND; var template (value) NB_RarList_Type v_NB_RarList; var template (value) NB_RachProcedureList_Type v_RachProcedureList; var integer v_RAPID; var integer v_RA_PreambleIndex; var boolean v_NtnEnvironment; var integer v_NpdcchBundleOffset; timer t_Timer; v_MAC_MainConfig.timeAlignmentTimerDedicated_r13 := sf5120; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup, v_TestLoopModeRepetitions); if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None) {//@sic R5s240285, R5-241564 sic@ v_NtnEnvironment := true; } else { v_NtnEnvironment := false; } v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@ f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, -, cs_MAC_PDU_TimingAdvance_0); /* @sic R5s170886 change 1: ensure timeAlignmentTimer(5.12s) not to time out before step 20 sic@ */ f_Delay(0.5); /* @sic R5s170886 change 1: to be sure that TA is acknowledged before HARQ_IndMode gets enabled sic@ */ f_NBIOT_SS_ConfigUL_HARQ_IndMode(v_CellId, enable); v_C_RNTI := f_NBIOT_CellInfo_GetC_RNTI(v_CellId); //------------------------------------ f_NBIOT_TestBody_Set(true, layer2); // build MAC PDU to be used at steps 1, 3, 5 v_RlcSduSizeDL := 38; // 38 bytes v_TBS := 328; // 41 bytes v_MAC_SDUList := { fl_NBIOT_MAC_EncodedRlcAmPdu_TX(v_RLC_CountsInfoListByRef, fl_ESM_DATA_TRANSPORT_UserData(v_RlcSduSizeDL)) }; v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList, v_TBS); //@siclog "Step 1" siclog@ f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, cs_TimingInfo_Now, cds_NB_NewC_RNTI_ConfigReq(tsc_C_RNTI_Def2)); t_Timer.start(0.5); f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, cs_TimingInfo_Now, v_MAC_PDU); //@siclog "Step 2" siclog@ alt { [] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId)) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_Timer.timeout { } } f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, cs_TimingInfo_Now, cds_NB_NewC_RNTI_ConfigReq(v_C_RNTI)); v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId); //@siclog "Step 3" siclog@ v_Imcs := 10; v_Isf := 1; v_RepetitionNumber := 2; v_DciDlInfo := cs_NB_DciDlInfo_Explicit(v_Imcs, v_Isf, v_RepetitionNumber, crcErrorWithRetransmission); f_NBIOT_SS_CommonCellConfig(v_CellId, cas_NB_CcchDcchDtchDL_Config_REQ(v_CellId, cs_TimingInfo_Now, -, v_DciDlInfo)); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, v_TimingInfo, v_MAC_PDU); // schedule change of DCI before start of the next search space v_RepetitionNumber := 1; v_DciDlInfo := cs_NB_DciDlInfo_Explicit(v_Imcs, v_Isf, v_RepetitionNumber); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 0.8)); f_NBIOT_SS_CommonCellConfig(v_CellId, cas_NB_CcchDcchDtchDL_Config_REQ(v_CellId, v_TimingInfo, -, v_DciDlInfo)); //@siclog "Step 4" siclog@ fl_NBIOT_MAC_ReceiveAckNack(v_CellId, nack, __FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ /* HARQ restransmission is done automatically by the SS; the SS shall apply the new DCI */ //@siclog "Step 6" siclog@ alt { /* @sic R5s170813 change 1.1: repeated receiption of nack until ack is received sic@ */ [] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId, ack)) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId, nack)) { repeat; } } //@siclog "Step 7" siclog@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170813 change 1.2 sic@ */ //@siclog "Step 8" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_ReceiveAndCheck(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 8"); //@siclog "Step 9" siclog@ … … … //@siclog "Step 24 - 25" siclog@ fl_TC_22_3_1_2_TimeAlignment(v_CellId, v_TimingInfo, cs_MAC_PDU_TimingAdvance_1Padding(tsc_MAC_CTRL_TimingAdvance_Max)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 25"); //@siclog "Step 26" siclog@ //The SS waits for 55 (6 for NTN) NPDCCH periods bundles.(NOTE 9, NOTE 4) v_TimingStart_Step24 := valueof(v_TimingInfo.SubFrame); //@sic R5-241564 sic@ if (v_NtnEnvironment) {//@sic R5s240266, R5-241564 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(7 * v_NpdcchBundleOffset))); // 6 for step 26 + 1 for step 27: In the 56th (7th for NTN)NPDCCH period bundle after the transmission at step 24 } else { v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(56 * v_NpdcchBundleOffset))); /* with TA timer of 5.12s: 56 * 64ms = 3.584s; i.e. 70% of 5.12s */ } //@siclog "Step 27" siclog@ //In the 56th (7th for NTN)NPDCCH period bundle after the transmission at step 24 the SS transmits MAC PDU containing one RLC SDU in an AMD PDU with polling field 'P' set to '1'.(NOTE 9, NOTE 4) v_RlcSduSizeDL := 16; /* 16 bytes => MAC SDU size = 18 resulting in TBS=176 @sic R5s170482 change 1.1: 16 bytes instead of 12 bytes sic@ */ v_MAC_SDUList := { fl_NBIOT_MAC_EncodedRlcAmPdu_TX(v_RLC_CountsInfoListByRef, fl_ESM_DATA_TRANSPORT_UserData(v_RlcSduSizeDL)) }; v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, v_TimingInfo, v_MAC_PDU); // schedule UL grant of step 29 in advance //In the 58th (8th for NTN)NPDCCH period bundle after the transmission at step 24 the SS allocates an UL grant for UE to send the RLC status PDU.(NOTE 3, NOTE 4) if (v_NtnEnvironment) {//@sic R5s240266, R5-241564 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(8 * v_NpdcchBundleOffset))); } else { v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(58 * v_NpdcchBundleOffset))); } f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant), v_TimingInfo); //@siclog "Step 28" siclog@ fl_NBIOT_MAC_ReceiveAckNack(v_CellId, ack, __FILE__, __LINE__, "Step 28"); //@siclog "Step 29" siclog@ /* UL grant as scheduled before step 28 */ //@siclog "Step 30" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_ReceiveAndCheck(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 30"); t_Timer.start(2.0); /* at step 30 we are in the 59th NPDCCH cycle after sending the TA command at step 24; UL grant at step 34 is scheduled for the 89th NPDCCH cycle => 2s should be sufficient */ //@siclog "Step 31" siclog@ //In the 88th (9th for NTN) NPDCCH period bundle after the transmission at step 24 SS transmits a downlink assignment to C-RNTI assigned to the UE on NPDCCH and transmits in the indicated downlink assignment a RLC PDU with polling field 'P' set to '0' in a MAC PDU on NPDSCH. (NOTE 10, NOTE 13, , NOTE 4) if (v_NtnEnvironment) {//@sic R5s240266, R5-241564, R5s240365, R5-242524 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(9 * v_NpdcchBundleOffset))); } else { v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(88 * v_NpdcchBundleOffset))); }

After Change:

| function f_TC_22_3_1_2_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var EUTRA_ASN1_C_RNTI_Type v_C_RNTI; var template (value) TimingInfo_Type v_TimingInfo; var template (value) MAC_PDU_Type v_MAC_PDU; var template (value) NB_DciDlInfo_Type v_DciDlInfo; var MAC_SDUList_Type v_MAC_SDUList; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_TimingStart_Step8a; var SubFrameTiming_Type v_TimingStart_Step21; var SubFrameTiming_Type v_TimingStart_Step24; var integer v_RlcSduSizeDL; var integer v_TBS; var integer v_Imcs; var integer v_Isf; var integer v_RepetitionNumber; var integer v_IncrementVTS; var integer v_TestLoopModeRepetitions := 0; // according to test case preamble var NB_SYSTEM_IND v_NB_SYSTEM_IND; var template (value) NB_RarList_Type v_NB_RarList; var template (value) NB_RachProcedureList_Type v_RachProcedureList; var integer v_RAPID; var integer v_RA_PreambleIndex; var boolean v_NtnEnvironment; var integer v_NpdcchBundleOffset; timer t_Timer; v_MAC_MainConfig.timeAlignmentTimerDedicated_r13 := sf5120; v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig); f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup, v_TestLoopModeRepetitions); if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None) {//@sic R5s240285, R5-241564 sic@ v_NtnEnvironment := true; } else { v_NtnEnvironment := false; } v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@ f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, -, cs_MAC_PDU_TimingAdvance_0); /* @sic R5s170886 change 1: ensure timeAlignmentTimer(5.12s) not to time out before step 20 sic@ */ //WA#WI if(v_NtnEnvironment){ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) }else { f_Delay(0.5); /* @sic R5s170886 change 1: to be sure that TA is acknowledged before HARQ_IndMode gets enabled sic@ */ } f_NBIOT_SS_ConfigUL_HARQ_IndMode(v_CellId, enable); v_C_RNTI := f_NBIOT_CellInfo_GetC_RNTI(v_CellId); //------------------------------------ f_NBIOT_TestBody_Set(true, layer2); // build MAC PDU to be used at steps 1, 3, 5 v_RlcSduSizeDL := 38; // 38 bytes v_TBS := 328; // 41 bytes v_MAC_SDUList := { fl_NBIOT_MAC_EncodedRlcAmPdu_TX(v_RLC_CountsInfoListByRef, fl_ESM_DATA_TRANSPORT_UserData(v_RlcSduSizeDL)) }; v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList, v_TBS); //@siclog "Step 1" siclog@ f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, cs_TimingInfo_Now, cds_NB_NewC_RNTI_ConfigReq(tsc_C_RNTI_Def2)); t_Timer.start(0.5); f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, cs_TimingInfo_Now, v_MAC_PDU); //@siclog "Step 2" siclog@ alt { [] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId)) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_Timer.timeout { } } f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, cs_TimingInfo_Now, cds_NB_NewC_RNTI_ConfigReq(v_C_RNTI)); v_TimingStart := f_NBIOT_GetNextSearchSpace(v_CellId); //@siclog "Step 3" siclog@ v_Imcs := 10; v_Isf := 1; v_RepetitionNumber := 2; v_DciDlInfo := cs_NB_DciDlInfo_Explicit(v_Imcs, v_Isf, v_RepetitionNumber, crcErrorWithRetransmission); f_NBIOT_SS_CommonCellConfig(v_CellId, cas_NB_CcchDcchDtchDL_Config_REQ(v_CellId, cs_TimingInfo_Now, -, v_DciDlInfo)); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, v_TimingInfo, v_MAC_PDU); // schedule change of DCI before start of the next search space v_RepetitionNumber := 1; v_DciDlInfo := cs_NB_DciDlInfo_Explicit(v_Imcs, v_Isf, v_RepetitionNumber); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 0.8)); f_NBIOT_SS_CommonCellConfig(v_CellId, cas_NB_CcchDcchDtchDL_Config_REQ(v_CellId, v_TimingInfo, -, v_DciDlInfo)); //@siclog "Step 4" siclog@ fl_NBIOT_MAC_ReceiveAckNack(v_CellId, nack, __FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ /* HARQ restransmission is done automatically by the SS; the SS shall apply the new DCI */ //@siclog "Step 6" siclog@ alt { /* @sic R5s170813 change 1.1: repeated receiption of nack until ack is received sic@ */ [] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId, ack)) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId, nack)) { repeat; } } //@siclog "Step 7" siclog@ f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant)); /* @sic R5s170813 change 1.2 sic@ */ //@siclog "Step 8" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_ReceiveAndCheck(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 8"); //WA#WI //@siclog "Step 8a - 8b" siclog@ if(v_NtnEnvironment){ v_TimingStart_Step8a := f_NBIOT_GetNextSearchSpace(v_CellId); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart_Step8a); fl_TC_22_3_1_2_TimeAlignment(v_CellId, v_TimingInfo, cs_MAC_PDU_TimingAdvance_0); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 8a"); } //@siclog "Step 9" siclog@ … … … //@siclog "Step 24 - 25" siclog@ fl_TC_22_3_1_2_TimeAlignment(v_CellId, v_TimingInfo, cs_MAC_PDU_TimingAdvance_1Padding(tsc_MAC_CTRL_TimingAdvance_Max)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 25"); //@siclog "Step 26" siclog@ //The SS waits for 55 (5 for NTN) NPDCCH periods bundles.(NOTE 9, NOTE 4) v_TimingStart_Step24 := valueof(v_TimingInfo.SubFrame); //@sic R5-241564 sic@ if (v_NtnEnvironment) {//@sic R5s240266, R5-241564 sic@ //WA#WIchanged to 6 as Step 29 shall use 7. using 8 in Step 29 may result in timer alignment expiry v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(6 * v_NpdcchBundleOffset))); // 5 for step 26 + 1 for step 27: In the 56th (7th for NTN)NPDCCH period bundle after the transmission at step 24 } else { v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(56 * v_NpdcchBundleOffset))); /* with TA timer of 5.12s: 56 * 64ms = 3.584s; i.e. 70% of 5.12s */ } //@siclog "Step 27" siclog@ //In the 56th (6th for NTN)NPDCCH period bundle after the transmission at step 24 the SS transmits MAC PDU containing one RLC SDU in an AMD PDU with polling field 'P' set to '1'.(NOTE 9, NOTE 4) v_RlcSduSizeDL := 16; /* 16 bytes => MAC SDU size = 18 resulting in TBS=176 @sic R5s170482 change 1.1: 16 bytes instead of 12 bytes sic@ */ v_MAC_SDUList := { fl_NBIOT_MAC_EncodedRlcAmPdu_TX(v_RLC_CountsInfoListByRef, fl_ESM_DATA_TRANSPORT_UserData(v_RlcSduSizeDL)) }; v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, v_TimingInfo, v_MAC_PDU); // schedule UL grant of step 29 in advance //In the 58th (7th for NTN)NPDCCH period bundle after the transmission at step 24 the SS allocates an UL grant for UE to send the RLC status PDU.(NOTE 3, NOTE 4) if (v_NtnEnvironment) {//@sic R5s240266, R5-241564 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(7 * v_NpdcchBundleOffset))); //WA#WI changed to 7 using 8 in Step 29 may result in timer alignment expiry } else { v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(58 * v_NpdcchBundleOffset))); } f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant), v_TimingInfo); //@siclog "Step 28" siclog@ fl_NBIOT_MAC_ReceiveAckNack(v_CellId, ack, __FILE__, __LINE__, "Step 28"); //@siclog "Step 29" siclog@ /* UL grant as scheduled before step 28 */ //@siclog "Step 30" siclog@ f_NBIOT_MAC_CPMode_RlcStatusPdu_ReceiveAndCheck(v_RLC_CountsInfoListByRef, v_CellId); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 30"); t_Timer.start(2.0); /* at step 30 we are in the 59th NPDCCH cycle after sending the TA command at step 24; UL grant at step 34 is scheduled for the 89th NPDCCH cycle => 2s should be sufficient */ //@siclog "Step 31" siclog@ //In the 88th (9th for NTN) NPDCCH period bundle after the transmission at step 24 SS transmits a downlink assignment to C-RNTI assigned to the UE on NPDCCH and transmits in the indicated downlink assignment a RLC PDU with polling field 'P' set to '0' in a MAC PDU on NPDSCH. (NOTE 10, NOTE 13, , NOTE 4) if (v_NtnEnvironment) {//@sic R5s240266, R5-241564, R5s240365, R5-242524 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(9 * v_NpdcchBundleOffset))); } else { v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(88 * v_NpdcchBundleOffset))); } |  |
| --- | --- |