3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240626

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3559 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC TC 8.1.5.9.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For testcase 8.1.5.9.3 at Steps 3 and 13, upon reception of rrcResume Request from UE, SS is set to transmit segmented rrcResume message. However, in current TTCN implementation, process of creating the rrcResume message (calculation of size, padding, etc.) is done once rrcResume is received, with this step included the time before NW is able to transmit the first rrcResume segment is greather than 1000ms (1 second) which the T319 timer. Therefore, UE initiates rrcSetupRequest due to T319 expiry leading to TC failure. By moving the process of preparing the segmented message before the reception of rrcResumeRequest, NW can immediately start transmitting the segments after reception of rrcResumeRequest to avoid T319 expiration issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the process of building the segmented rrcResume message (addition of padding, calculationg of size etc) to earlier steps, to reduce processing time after reception of rrcResumeRequest from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 TTCN-3 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_9_3_NR5GC_TestBody() |
| --- | --- |
| Reason for change | For testcase 8.1.5.9.3 at Steps 3 and 13, upon reception of rrcResume Request from UE, SS is set to transmit segmented rrcResume message. However, in current TTCN implementation, process of creating the rrcResume message (calculation of size, padding, etc.) is done once rrcResume is received, with this step included the time before NW is able to transmit the first rrcResume segment is greather than 1000ms (1 second) which the T319 timer. Therefore, UE initiates rrcSetupRequest due to T319 expiry leading to TC failure. By moving the process of preparing the segmented message before the reception of rrcResumeRequest, NW can immediately start transmitting the segments after reception of rrcResumeRequest to avoid T319 expiration issue. |
| Summary of change | Moved the process of preparing rrcResume segments earlier to reduce extra processing time. |
| TTCN module | 8_1_5\RRC_Others_NR5GC |
| MCC160 Comment | Accepted in principle but implemented differently. See proposed implementation below. The configuration of the SS before step 3 and step 13 has been re-organised to follow generic function f_NR_SendRRCResume_Def. r1: Added missing line in proposed implementation. |

Before Change:

function f_TC_8_1_5_9_3_NR5GC_TestBody() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo; var integer v_DRBId; var template (value) DL_DCCH_Message v_RRCResume; var octetstring v_RRCResumeEncoded; var integer v_PaddingLength; var octetstring v_LateNonCriticalExt; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); //@sic R5s240358 sic@ var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s240358 sic@ //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release @sic R5s240358 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB @sic R5s240358 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1); // Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume f_NR_SS_ReconfigSStoCSS_DCI1_0(nr_Cell1); //@sic R5s240461 sic@ //@siclog "Step 3" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message. //Get RRCResume message to segment and config SS v_RRCResume := f_NR_GetRRCResume_Def(nr_Cell1); //Add padding to get size of encoded RRCResume message < 3*maximum PDCP SDU size, so that it will fit in three segments -> e.g. total size = 2.5 x tsc_Max_NR_PDCP_SDU_size v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 25 / 10) - lengthof(v_RRCResumeEncoded); v_LateNonCriticalExt := ''O; while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0; v_PaddingLength := v_PaddingLength - 1000; } v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt; v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); // Send first segment f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 0, notLastSegment); //@siclog "Step 4" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 1, notLastSegment); //@siclog "Step 5" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 2, lastSegment); //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 7" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); if (isvalue(v_PDUSessionInfo[0])) { f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 7"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } //@siclog "Step 8" siclog@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 9" siclog@ //The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_); //@siclog "Step 10" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //@siclog "Step 11" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //@siclog "Step 12" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 12A" siclog@ //The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release @sic R5s240358 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB @sic R5s240358 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1); // Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume f_NR_SS_ReconfigSStoCSS_DCI1_0(nr_Cell1); //@sic R5s240461 sic@ //@siclog "Step 13" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message. //Get RRCResume message to segment and config SS v_RRCResume := f_NR_GetRRCResume_Def(nr_Cell1); //Add padding to get size of encoded RRCResume message < 2*maximum PDCP SDU size, so that it will fit in two segments -> e.g. total size = 1.5 x tsc_Max_NR_PDCP_SDU_size v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 15 / 10) - lengthof(v_RRCResumeEncoded); v_LateNonCriticalExt := ''O; while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0; v_PaddingLength := v_PaddingLength - 1000; } v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt; v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); // Send first segment f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 0, notLastSegment); //@siclog "Step 14" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the second/last segment of the 2 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 1, lastSegment); //@siclog "Step 15" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 16" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 16"); //@siclog "Step 17" siclog@ f_NR_RRCRelease(nr_Cell1); }//end of f_TC_8_1_5_9_3_NR5GC_TestBody

After Change:

function f_TC_8_1_5_9_3_NR5GC_TestBody() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo; var integer v_DRBId; var template (value) DL_DCCH_Message v_RRCResume; var octetstring v_RRCResumeEncoded; var integer v_PaddingLength; var octetstring v_LateNonCriticalExt; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); //@sic R5s240358 sic@ var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s240358 sic@ //Get RRCResume message to segment and config SS v_RRCResume := f_NR_GetRRCResume_Def(nr_Cell1); //Add padding to get size of encoded RRCResume message < 3*maximum PDCP SDU size, so that it will fit in three segments -> e.g. total size = 2.5 x tsc_Max_NR_PDCP_SDU_size v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 25 / 10) - lengthof(v_RRCResumeEncoded); v_LateNonCriticalExt := ''O; while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0; v_PaddingLength := v_PaddingLength - 1000; } v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt; v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release @sic R5s240358 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB @sic R5s240358 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1); // Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume f_NR_SS_ReconfigSStoCSS_DCI1_0(nr_Cell1); //@sic R5s240461 sic@ //@siclog "Step 3" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message. // Send first segment f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 0, notLastSegment); //@siclog "Step 4" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 1, notLastSegment); //@siclog "Step 5" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 2, lastSegment); //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 7" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); if (isvalue(v_PDUSessionInfo[0])) { f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 7"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } //@siclog "Step 8" siclog@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 9" siclog@ //The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_); //@siclog "Step 10" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //@siclog "Step 11" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //Get RRCResume message to segment and config SS v_RRCResume := f_NR_GetRRCResume_Def(nr_Cell1); //Add padding to get size of encoded RRCResume message < 2*maximum PDCP SDU size, so that it will fit in two segments -> e.g. total size = 1.5 x tsc_Max_NR_PDCP_SDU_size v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 15 / 10) - lengthof(v_RRCResumeEncoded); v_LateNonCriticalExt := ''O; while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0; v_PaddingLength := v_PaddingLength - 1000; } v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt; v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); //@siclog "Step 12" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 12A" siclog@ //The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release @sic R5s240358 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB @sic R5s240358 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1); // Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume f_NR_SS_ReconfigSStoCSS_DCI1_0(nr_Cell1); //@sic R5s240461 sic@ //@siclog "Step 13" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message. // Send first segment f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 0, notLastSegment); //@siclog "Step 14" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the second/last segment of the 2 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 1, lastSegment); //@siclog "Step 15" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 16" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 16"); //@siclog "Step 17" siclog@ f_NR_RRCRelease(nr_Cell1); }//end of f_TC_8_1_5_9_3_NR5GC_TestBody

MCC160 proposed implementation

function f_TC_8_1_5_9_3_NR5GC_TestBody() runs on NR5GC_PTC

{

var PDUSessionInfoList_Type v_PDUSessionInfo;

var integer v_DRBId;

var octetstring v_RRCResumeEncoded;

var integer v_PaddingLength;

var octetstring v_LateNonCriticalExt;

var integer v_ModificationPeriodFrames;

var float v_ModificationPeriod;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); //@sic R5s240358 sic@

var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s240358 sic@

var template (value) DL_DCCH_Message v_RRCResume := f_NR_38508_Get_RRCResume_Def(nr_Cell1); //@sic R5s240626 sic@

//Prepare segmented RRCResume message in advance so that it is ready to be sent at Step 3

//@sic R5s240626 sic@

//Add padding to get size of encoded RRCResume message < 3*maximum PDCP SDU size, so that it will fit in three segments -> e.g. total size = 2.5 x tsc_Max_NR_PDCP_SDU_size

v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume));

v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 25 / 10) - lengthof(v_RRCResumeEncoded);

v_LateNonCriticalExt := ''O;

while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining

v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0;

v_PaddingLength := v_PaddingLength - 1000;

}

v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated

v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt;

v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume));

//@siclog "Step 1" siclog@

//The SS transmits a Paging message including a matched identity (correct fullI-RNTI).

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

//@siclog "Step 2" siclog@

//The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value.

f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access);

//@sic R5s240626 sic@

// Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume

f_NR_SS_ReconfigSStoCSS_DCI1_0(nr_Cell1);        //@sic R5s240461 sic

// Switch on UL grant assignments

f_NR_ULGrantConfiguration_Start(nr_Cell1);

//Reconfigure DRB in SS - existing DRBs are not reconfigured during release @sic R5s240358 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config);

//Activate security on newly configured DRB @sic R5s240358 sic@

f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1);

//@siclog "Step 3" siclog@

//The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message.

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCResumeEncoded,

0,

notLastSegment);

//@siclog "Step 4" siclog@

//The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message.

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCResumeEncoded,

1,

notLastSegment);

//@siclog "Step 5" siclog@

//The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message.

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCResumeEncoded,

2,

lastSegment);

//@siclog "Step 6" siclog@

//The UE transmits an RRCResumeComplete message.

f_NR_RRCResumeComplete_Def(nr_Cell1);

//@siclog "Step 7" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session?

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList();

if (isvalue(v_PDUSessionInfo[0])) {

f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 7");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session");

}

//@siclog "Step 8" siclog@

//The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig.

f_NR_RRC_InactiveDef(nr_Cell1);

//@siclog "Step 9" siclog@

//The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True.

f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_);

//@siclog "Step 10" siclog@

//The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.

f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE);

//@siclog "Step 11" siclog@

//Wait for 2.1* modification period second for the UE to receive new system information.

v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1);

v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds

f_Delay(2.1 * v_ModificationPeriod);

//Get RRCResume message to segment and config SS

v_RRCResume := f_NR_38508_Get_RRCResume_Def(nr_Cell1); //@sic R5s240626 sic@

//Prepare segmented RRCResume message in advance so that it is ready to be sent at Step 3

//@sic R5s240626 sic@

//Add padding to get size of encoded RRCResume message < 2*maximum PDCP SDU size, so that it will fit in two segments -> e.g. total size = 1.5 x tsc_Max_NR_PDCP_SDU_size

v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume));

v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 15 / 10) - lengthof(v_RRCResumeEncoded);

v_LateNonCriticalExt := ''O;

while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining

v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0;

v_PaddingLength := v_PaddingLength - 1000;

}

v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated

v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt;

v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume));

//@siclog "Step 12" siclog@

//The SS transmits a Paging message including a matched identity (correct fullI-RNTI).

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

//@siclog "Step 12A" siclog@

//The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value?

SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1)));

//@sic R5s240626 sic@

// Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume

f_NR_SS_ReconfigSStoCSS_DCI1_0(nr_Cell1);        //@sic R5s240461 sic@

// Switch on UL grant assignments

f_NR_ULGrantConfiguration_Start(nr_Cell1);

//Reconfigure DRB in SS - existing DRBs are not reconfigured during release @sic R5s240358 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config);

//Activate security on newly configured DRB @sic R5s240358 sic@

f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1);

//@siclog "Step 13" siclog@

//The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message.

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCResumeEncoded,

0,

notLastSegment);

//@siclog "Step 14" siclog@

//The SS transmits a DLDedicatedMessageSegment containing the second/last segment of the 2 segment RRCResume message.

f_NR_TransmitDLSegment(nr_Cell1,

v_RRCResumeEncoded,

1,

lastSegment);

//@siclog "Step 15" siclog@

//The UE transmits an RRCResumeComplete message.

f_NR_RRCResumeComplete_Def(nr_Cell1);

//@siclog "Step 16" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session?

f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 16");

//@siclog "Step 17" siclog@

f_NR_RRCRelease(nr_Cell1);

}//end of f_TC_8_1_5_9_3_NR5GC_TestBody