3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240629

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3561 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To verify NR5GC eCall over IMS testcase 11.5.3 using IWD 24wk37 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC eCall over IMS testcase 11.5.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required:	3

2.1	Correction to function TC_11_5_3_NR5GC	3

2.2	New function f_TC_11_5_3_NR5GC_IMS1	4

2.3	Correction to function f_TC_11_5_3_NR5GC_IMS2	5

2.4	Correction to function f_TC_11_5_3_NR5GC	6

2.5	Correction to function fl_TC_11_5_3_Body	7

2.6	Correction to function f_TC_11_5_3_NR5GC_EUTRA	8

2.7	Correction to function fl_TC_11_5_3_NR5GC_EUTRA_TestBody	9

2.8	Correction to function f_NR_CellInfo_SetSIB1_ImsEmergencySupport	11

2.9	Correction to function f_EUTRA_CellInfo_SetSysInfo_EmergencySupport	12

3	Branches executed	13

4	Execution Log Files	13

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.	13

5	References	14

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC eCall over IMS testcase 11.5.3 which is part of the 	iwd-TTCN3-B2024-06_D24wk37.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Narendra Kalahasti

Narendra.kalahasti@anritsu.com

Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

  - Verification Test Summary

Test Case:	11.5.3.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Verification Status:	PASS

- Corrections required:

Change 2.4 of R5s230896 [2] is also required to pass the test case.

  - Correction to function TC_11_5_3_NR5GC

| Function name | testcase TC_11_5_3_NR5GC() |
| --- | --- |
| Reason for change | Initial IMS registration, if done by UE, will occur in LTE. In current implementation, IMS default handler will expect IMS messages over NR and not LTE. |
| Summary of change | Create new IMS1 PTC so that IMS messages will be received over LTE. |
| TTCN module | NR5GC_IRAT_Testsuite |
| MCC160 | Accepted |

Before Change:

testcase TC_11_5_3_NR5GC() runs on MTC_NR5GC_IRAT system SYSTEM_NR5GC_IRAT { // @purpose // eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available var NR5GC_PTC v_NR5GC := null; var EUTRA_Multi5GS_PTC v_EUTRA_Multi5GS := null; var UTRAN_NR5GC_PTC v_UTRAN_NR5GC := null; var GERAN_NR5GC_PTC v_GERAN_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; var WLAN_PTC v_WLAN := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_EUTRA_Multi5GS := EUTRA_Multi5GS_PTC.create("EUTRA_Multi5GS") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC_IRAT(system, v_NR5GC, v_EUTRA_Multi5GS, v_UTRAN_NR5GC, v_GERAN_NR5GC, v_IMS1, v_IMS2, v_WLAN); v_NR5GC.start(f_TC_11_5_3_NR5GC()); v_EUTRA_Multi5GS.start(f_TC_11_5_3_NR5GC_EUTRA()); v_IMS2.start(f_TC_11_5_3_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC_IRAT(t_GuardTimer); }

After Change:

testcase TC_11_5_3_NR5GC() runs on MTC_NR5GC_IRAT system SYSTEM_NR5GC_IRAT { // @purpose // eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available var NR5GC_PTC v_NR5GC := null; var EUTRA_Multi5GS_PTC v_EUTRA_Multi5GS := null; var UTRAN_NR5GC_PTC v_UTRAN_NR5GC := null; var GERAN_NR5GC_PTC v_GERAN_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; var WLAN_PTC v_WLAN := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_EUTRA_Multi5GS := EUTRA_Multi5GS_PTC.create("EUTRA_Multi5GS") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC_IRAT(system, v_NR5GC, v_EUTRA_Multi5GS, v_UTRAN_NR5GC, v_GERAN_NR5GC, v_IMS1, v_IMS2, v_WLAN); v_NR5GC.start(f_TC_11_5_3_NR5GC()); v_EUTRA_Multi5GS.start(f_TC_11_5_3_NR5GC_EUTRA()); v_IMS1.start(f_TC_11_5_3_NR5GC_IMS1()); v_IMS2.start(f_TC_11_5_3_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC_IRAT(t_GuardTimer); }

  - New function f_TC_11_5_3_NR5GC_IMS1

| Function name | f_TC_11_5_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Same as Change 2.1 |
| Summary of change | Create new IMS1 PTC so that IMS messages will be received over LTE. |
| TTCN module | ECall_IRAT_NR5GC_IMS |
| MCC160 | Accepted |

Before Change:

function f_TC_11_5_3_NR5GC_IMS1() runs on IMS_PTC { f_IMS_PTC_Init(PDN_1); f_IMS_Registration(OtherIPCAN_E); //Coord message when call is hanged f_IMS_IpcanReleaseWithOptionalDeregistrationOtherIPCAN(); } // End of f_TC_11_5_3_IRAT_IMS1()

  - Correction to function f_TC_11_5_3_NR5GC_IMS2

| Function name | f_TC_11_5_3_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - As per Change 2.1, IMS messages will be expected in LTE RAT. Proper Access Network Info needs to be set. - Prose Step 4 indicates that UE initiates an automatic eCall. - Improvement (same as R5s240599 Change 2.3): Improve readability of coordination messages. (Change paired with correction 2.7.3) |
| Summary of change | - Used OtherIPCAN_E as IMS messages are expected in LTE. - Change to Automatic eCall to align with prose. - Added reasoning to IPCAN coord message. |
| TTCN module | ECall_IRAT_NR5GC_IMS |
| MCC160 | Accepted |

Before Change:

function f_TC_11_5_3_NR5GC_IMS2() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // @siclog "Steps 6-24" siclog@ //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in TS 36.508 subclause 4.5A.27 take place. f_IMS_EmergencyRegistration(); f_ECall_CallSetup_AnnexC47(MANUAL); // @siclog "Step 25" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33. v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); } // End of f_TC_11_5_3_IRAT_IMS2()

After Change:

function f_TC_11_5_3_NR5GC_IMS2() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // @siclog "Steps 6-24" siclog@ //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in TS 36.508 subclause 4.5A.27 take place. f_IMS_EmergencyRegistration(-,-,OtherIPCAN_E); // ecall WA f_ECall_CallSetup_AnnexC47(AUTOMATIC); // @siclog "Step 25" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33. v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); } // End of f_TC_11_5_3_IRAT_IMS2()

  - Correction to function f_TC_11_5_3_NR5GC

| Function name | f_TC_11_5_3_NR5GC |
| --- | --- |
| Reason for change | Connection and messages are expected to be received in LTE. It’s proposed to move UE configuration and initialisation (as well as corresponding MMIs) to LTE PTC. |
| Summary of change | Moved code to function f_TC_11_5_3_NR5GC_EUTRA, please check Change 2.6.2. |
| TTCN module | ECall_IRAT_NR5GC_NR |
| MCC160 | Accepted |

Before Change:

function f_TC_11_5_3_NR5GC() runs on NR5GC_PTC {//eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_SetSIB1_eCallOverIMS_Support(nr_Cell1,omit); f_NR_CellInfo_SetSIB1_ImsEmergencySupport(nr_Cell1,omit); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "USIM configuration 24 specified in table 6.4.1-24 TS 38.508-1"); // Preamble: The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_11_5_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); // Now tell EUTRA test is finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

After Change:

function f_TC_11_5_3_NR5GC() runs on NR5GC_PTC {//eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_SetSIB1_eCallOverIMS_Support(nr_Cell1,omit); f_NR_CellInfo_SetSIB1_ImsEmergencySupport(nr_Cell1,omit); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); <<MOVED CODE>> //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. //f_UT_USIM_Insert(UT, "USIM configuration 24 specified in table 6.4.1-24 TS 38.508-1"); <<MOVED CODE>> // Preamble: The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_11_5_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); // Now tell EUTRA test is finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

  - Correction to function fl_TC_11_5_3_Body

| Function name | fl_TC_11_5_3_Body |
| --- | --- |
| Reason for change | Same as Change 2.4 |
| Summary of change | Moved code to function f_TC_11_5_3_NR5GC_EUTRA, please check Correction 2.7.1. |
| TTCN module | ECall_IRAT_NR5GC_NR |
| MCC160 | Accepted |

Before Change:

function fl_TC_11_5_3_NR5GC_TestBody() runs on NR_BASE_PTC { //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);//@sic R5-235303 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) f_UT_InitiateECall(UT, AUTOMATIC); //@siclog "Step 29" siclog@ //The UE is switched on. f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Change Power Level"); f_UT_SwitchOnUE(UT); //@siclog "Step 30" siclog@ //Wait for 60s. f_Delay(60.0); //@siclog "Step 31" siclog@ //An automatic eCall is initiated. (Note 1) f_UT_InitiateECall(UT, AUTOMATIC, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ // @siclog "Step 32" siclog@ // Check: Does the UE transmit an RRCConnectionRequest message in the next 30 sec?// prose CR needed. f_NR_CheckNoRRCSetupReq (nr_Cell1, 30.0,-,"step 32 failed"); }

After Change:

function fl_TC_11_5_3_NR5GC_TestBody() runs on NR_BASE_PTC { //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);//@sic R5-235303 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); <<MOVED CODE>> // @siclog "Step 2" siclog@ // The UE is Switched ON //f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. //f_Delay(60.0); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) //f_UT_InitiateECall(UT, AUTOMATIC); <<MOVED CODE>> //@siclog "Step 29" siclog@ //The UE is switched on. f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Change Power Level"); f_UT_SwitchOnUE(UT); //@siclog "Step 30" siclog@ //Wait for 60s. f_Delay(60.0); //@siclog "Step 31" siclog@ //An automatic eCall is initiated. (Note 1) f_UT_InitiateECall(UT, AUTOMATIC, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ // @siclog "Step 32" siclog@ // Check: Does the UE transmit an RRCConnectionRequest message in the next 30 sec?// prose CR needed. f_NR_CheckNoRRCSetupReq (nr_Cell1, 30.0,-,"step 32 failed"); }

  - Correction to function f_TC_11_5_3_NR5GC_EUTRA

| Function name | f_TC_11_5_3_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - RTP loopback needs to be enabled for the eCall established during this test case. - Connection and messages are expected to be received in LTE. It’s proposed to move UE configuration and initialisation (as well as corresponding MMIs) to LTE PTC. |
| Summary of change | - Added HANDLE_UM_DATA as input parameter to activate RtpRtcpLoopback. - Moved code to function f_TC_11_5_3_NR5GC_EUTRA, please check Change 2.4 |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 | Accepted |

Before Change:

function f_TC_11_5_3_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available f_EUTRA38_IRAT_Init(c31); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. f_EUTRA_CellInfo_SetSysInfo_EmergencySupport(eutra_Cell1,true_); f_EUTRA_CellInfo_SetSysInfo_EcallOverIMS(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_5_3_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // take down the cell now // take down the cell now f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); } //End of f_TC_11_5_3_NR5GC_EUTRA()

After Change:

function f_TC_11_5_3_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. f_EUTRA_CellInfo_SetSysInfo_EmergencySupport(eutra_Cell1,true_); f_EUTRA_CellInfo_SetSysInfo_EcallOverIMS(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "configuration 24 specified in table 6.4.1-24 TS 38.508-1"); f_EUTRA_TestBody_Set(true); fl_TC_11_5_3_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // take down the cell now // take down the cell now f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); } //End of f_TC_11_5_3_NR5GC_EUTRA()

  - Correction to function fl_TC_11_5_3_NR5GC_EUTRA_TestBody

| Function name | fl_TC_11_5_3_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - Connection and messages are expected to be received in LTE. It’s proposed to move UE configuration and initialisation (as well as corresponding MMIs) to LTE PTC. - Existing function f_EUTRA_508_IMSeCallOnly_Steps3_25 can be used in this test case to avoid code duplication. - Improvement (same as R5s240599 Change 2.3): Improve readability of coordination messages. (Change paired with correction 2.3.3) - Current coordination message text does not match NR PTC counterpart. |
| Summary of change | - Moved code to function f_TC_11_5_3_NR5GC_EUTRA_TestBody, please check Change 2.5. - Used common function for eCall over LTE. - Added reasoning to IPCAN coord message. - Fix coordination message text. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 | Accepted |

Before Change:

function fl_TC_11_5_3_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; //@siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Suitable cell". f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"Change Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, -91, -96);//@sic R5-235303 sic@ // @siclog "Step 5" siclog@ //Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? f_EUTRA_MobileInfo_SetNetworkFeatureSupport(f_GetDefaultValueNetworkFeatureSupport()); v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (eutra_Cell1, PREAMBLE, Combined, DEFAULT); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 5"); //@siclog "Step 6-24" siclog@ //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in clause 4.5A.27 of TS 36.508 [6] take place. f_EUTRA_InitialRegistration_Step5_16c1 (eutra_Cell1, STATE2_IDLEUPDATE, Combined, v_NAS_Ind, omit, 0); f_EUTRA_508_EstablishIMSEmergencyCall_Steps9_15(eutra_Cell1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // @siclog "Step 25" siclog@ //Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 // @siclog "Step 26" siclog@ //The SS releases the RRC connection. //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); //@siclog "Step 27" siclog@ //The UE is Switched Off. f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell1, RRC_IDLE); //@siclog "Step 28" siclog@ //The SS configures:- E-UTRA Cell 1 " Non-suitable "Off" cell" f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); } // End of fl_TC_11_5_3_NR5GC_EUTRA_TestBody()

After Change:

import from UpperTesterFunctions all; function fl_TC_11_5_3_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; //@siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Suitable cell". f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"Change Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, -91, -96);//@sic R5-235303 sic@ // @siclog "Step 1" siclog@ f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); // @siclog "Step 2" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state f_Delay(60.0); // @siclog "Step 3" siclog@ // Note 1: The request to originate an automatic eCall may be performed by MMI or AT command. f_UT_InitiateECall(UT, AUTOMATIC); //Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in clause 4.5A.27 of TS 36.508 [6] take place. // @siclog "Steps 5" siclog@ // @siclog "Steps 6-24" siclog@ f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //IMS call has been established, wait for trigger from IMS PTC to release rrc connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS Call Release Msgs Sent"); // @siclog "Step 25" siclog@ //Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 // @siclog "Step 26" siclog@ //The SS releases the RRC connection. //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); //@siclog "Step 27" siclog@ //The UE is Switched Off. f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell1, RRC_IDLE); //@siclog "Step 28" siclog@ //The SS configures:- E-UTRA Cell 1 " Non-suitable "Off" cell" f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Change Power Level")); } // End of fl_TC_11_5_3_NR5GC_EUTRA_TestBody()

  - Correction to function f_NR_CellInfo_SetSIB1_ImsEmergencySupport

| Function name | f_NR_CellInfo_SetSIB1_ImsEmergencySupport |
| --- | --- |
| Reason for change | Function needs to be updated to handle input of ‘omit’ properly. This has already been done for many similar functions. |
| Summary of change | Add proper handling of ‘omit’ for via ispresent(p_IMS_EmergencySupport). |
| TTCN module | NR_CellInfo |
| MCC160 | Accepted, implemented with isvalue. |

Before Change:

function f_NR_CellInfo_SetSIB1_ImsEmergencySupport(NR_CellId_Type p_NR_CellId, template (omit) SIB1.ims_EmergencySupport p_IMS_EmergencySupport) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.ims_EmergencySupport := valueof(p_IMS_EmergencySupport); } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

After Change:

function f_NR_CellInfo_SetSIB1_ImsEmergencySupport(NR_CellId_Type p_NR_CellId, template (omit) SIB1.ims_EmergencySupport p_IMS_EmergencySupport) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (ispresent (p_IMS_EmergencySupport)) { // ecall WA v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.ims_EmergencySupport := valueof(p_IMS_EmergencySupport); } else { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.ims_EmergencySupport := omit; } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

  - Correction to function f_EUTRA_CellInfo_SetSysInfo_EmergencySupport

| Function name | f_EUTRA_CellInfo_SetSysInfo_EmergencySupport |
| --- | --- |
| Reason for change | Improvement: Function needs to be updated to handle input of ‘omit’ properly. This has already been done for many similar functions. |
| Summary of change | Add proper handling of ‘omit’ for via ispresent(p_IMS_EmergencySupport). |
| TTCN module | EUTRA_CellInfo |
| MCC160 | Accepted, see proposed implementation below. |

Before Change:

function f_EUTRA_CellInfo_SetSysInfo_EmergencySupport(EUTRA_CellId_Type p_CellId, template (omit) Ims_EmergencySupport_r9 p_Ims_EmergencySupport) runs on EUTRA_PTC // @sic R5s140731 R5-144741 sic@ { var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-171540 sic@ v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= p_Ims_EmergencySupport; if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= p_Ims_EmergencySupport; } f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); }

After Change:

function f_EUTRA_CellInfo_SetSysInfo_EmergencySupport(EUTRA_CellId_Type p_CellId, template (omit) Ims_EmergencySupport_r9 p_Ims_EmergencySupport) runs on EUTRA_PTC // @sic R5s140731 R5-144741 sic@ { var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-171540 sic@ if (ispresent (p_Ims_EmergencySupport)) { // ecall WA v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= p_Ims_EmergencySupport; if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= p_Ims_EmergencySupport; } } else { v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= omit; } f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); }

MCC160 proposed implementation

function f_EUTRA_CellInfo_SetSysInfo_EmergencySupport(EUTRA_CellId_Type p_CellId,

template (omit) Ims_EmergencySupport_r9 p_Ims_EmergencySupport) runs on EUTRA_PTC  // @sic R5s140731 R5-144741 sic@

{

var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId);

var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-171540 sic@

if (isvalue(p_Ims_EmergencySupport)) { //@sic R5s240629 sic@

v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= p_Ims_EmergencySupport;

if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= p_Ims_EmergencySupport;

}

} else {

v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= omit;

if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.ims_EmergencySupport_r9:= omit;

}

}

f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo);

}

- Branches executed

The test case was executed on

-	NR primary band N41 using NR ciphering nea1 and integrity algorithm nia1

-	EUTRA primary band B3 using EUTRA ciphering eea1 and integrity algorithm eia1

- Execution Log Files

  - Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

Snapdragon Auto 5G Modem-RF Gen 2 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/ TC_11_5_3_LOG.xml
RnS/ TC_11_5_3_LOG.log

- PICS/PIXIT parameter file:

Anritsu/ TC_11_5_3_PIXIT.txt
RnS/ TC_11_5_3_LOG.log

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s240630: Supporting information for addition of NR5GC eCall over IMS testcase 11.5.3 in FR1 |
| --- | --- |
| [2] | R5s240599: Addition of NR5GC eCall over IMS testcase 11.5.12 |