3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240634

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3563 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.1.2.3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2024-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, the function f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17 is called to set the IE intraFreqReselectionRedCap_r17 as barred. However, for a RedCap UE, the IE RedCap_ConfigCommonSIB_r17 should also be set correctly, within which the IE cellBarredRedCap_r17should be set as barred. This needs to be implemented. After updating the SIB1 information, it is proposed to call steps to broadcast updated sysinfo, and wait for 2.1* modification periods for the UEs to be able to read this information. This is done in many other testcases that have sysinfo modification. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN implementation of function f_TC_6_1_2_3a_NR5GC_TestBody. Please see below for detailed changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_6_1_2_3a_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17 is called to set the IE intraFreqReselectionRedCap_r17 as barred. However, for a RedCap UE, the IE RedCap_ConfigCommonSIB_r17 should also be set correctly, within which the IE cellBarredRedCap_r17should be set as barred. This needs to be implemented. After updating the SIB1 information, it is proposed to call steps to broadcast updated sysinfo, and wait for 2.1* modification periods for the UEs to be able to read this information. This is done in many other testcases that have sysinfo modification. A Prose CR will be raised for this. |
| Summary of change | Updated the TTCN implementation of function f_TC_6_1_2_3a_NR5GC_TestBody. Please see below for detailed changes. |
| TTCN module | Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#105. |

Before Change:

function f_TC_6_1_2_3a_NR5GC_TestBody() runs on NR5GC_PTC { var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T0_Cell11_RS_EPRE_FR1 := -78; var integer v_T0_Cell12_RS_EPRE_FR1 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_NonSuitableCellSSS_EPRE_FR2; var integer v_T0_Cell11_RS_EPRE_FR2 := tsc_NR_ServingCellSSS_EPRE_FR2; var integer v_T0_Cell12_RS_EPRE_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T0_Cell11_RS_EPRE_FR1, v_T0_Cell11_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell12, v_T0_Cell12_RS_EPRE_FR1, v_T0_Cell12_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. // Bit 1 of Short Message field is set to 1 to indicate the SysInfo Modification and SS changes NR Cell 1 SS/PBCH EPRE level according to row "T0" in table 6.1.2.3a.3.2-1/2. f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17(nr_Cell1, notAllowed); // Table 6.1.2.3a.3.3-2: SIB1 for NR Cell 1 f_NR_ModifySysinfo_WaitActivation(nr_Cell1); //@sic R5s240456 sic@ - Modify Sysinfo in SS and wait until it is activated f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 2" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12? f_NR5GC_MobilityRegistration (nr_Cell12); }//end of f_TC_6_1_2_3a_NR5GC_TestBody

After Change:

import from Idle_CellSelection_NR5GC all; //WA#WI1248684 WA#6_1_2_3a . . . function f_TC_6_1_2_3a_NR5GC_TestBody() runs on NR5GC_PTC { var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T0_Cell11_RS_EPRE_FR1 := -78; var integer v_T0_Cell12_RS_EPRE_FR1 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_NonSuitableCellSSS_EPRE_FR2; var integer v_T0_Cell11_RS_EPRE_FR2 := tsc_NR_ServingCellSSS_EPRE_FR2; var integer v_T0_Cell12_RS_EPRE_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T0_Cell11_RS_EPRE_FR1, v_T0_Cell11_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell12, v_T0_Cell12_RS_EPRE_FR1, v_T0_Cell12_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. // Bit 1 of Short Message field is set to 1 to indicate the SysInfo Modification and SS changes NR Cell 1 SS/PBCH EPRE level according to row "T0" in table 6.1.2.3a.3.2-1/2. f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17(nr_Cell1, notAllowed); // Table 6.1.2.3a.3.3-2: SIB1 for NR Cell 1 if (pc_supportOfRedCap_r17){ //WA#WI1248684 WA#6_1_2_3a f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17(nr_Cell1, barred, barred); } //WA#WI1248684 WA#6_1_2_3a COMMENTED OUT f_NR_ModifySysinfo_WaitActivation(nr_Cell1); //@sic R5s240456 sic@ - Modify Sysinfo in SS and wait until it is activated f_NR_ModifySysinfo(nr_Cell1); //WA#WI1248684 WA#6_1_2_3a f_NR_WaitModificationPeriods(nr_Cell1); //WA#WI1248684 WA#6_1_2_3a f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 2" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12? f_NR5GC_MobilityRegistration (nr_Cell12); }//end of f_TC_6_1_2_3a_NR5GC_TestBody