3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240646

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3572 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC RRC MPS test case 8.1.1.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current implementation of the test case SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_HighPriorityAccess)))); is called at step 3 to check the reception of RRCConnectionrequest Due to above implementation rrcConnectionRequest PDU shall remain in queue without getting handled until we reach f_EUTRA38_TrackingAreaUpdateFromN1() -> f_EUTRA38_TAUReqFromN1 -> f_EUTRA_RRC_ConnEst_DefWithNas -> f_EUTRA_RRC_ConnectionRequest_Common () -> where we wait for the message as below: [] SRB.receive(car_SRB0_RrcPdu_IND(p_CellId, cr_RRCConnectionRequest(p_EstablishmentCause, p_InitialUE_Identity))) However, due to wait for coordination from NR proc in f_EUTRA38_TrackingAreaUpdateFromN1() -> f_EUTRA38_TAUReqFromN1() to receive PDUSessionInfoList and GUTI parameters, EUTRA proc shall be paused waiting for the co-ordination messages from NR. Test case may fail this time, since the message in queue (rrcConnectionSetupRequest) may get handled in default. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handled the connection request/SetupComplete/TAU etc along with Step 3 by calling function f_EUTRA38_TAUReqFromN1() At Step 5, f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq() is called to handle the remaining procedures of TAU. Moved call to function f_NR5GC_TrackingAreaUpdateFromN1() before waiting for co-ordination message f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | fl_TC_8_1_1_3_10_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current implementation of the test case SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_HighPriorityAccess)))); is called at step 3 to check the reception of RRCConnectionrequest Due to above implementation rrcConnectionRequest PDU shall remain in queue without getting handled until we reach f_EUTRA38_TrackingAreaUpdateFromN1() -> f_EUTRA38_TAUReqFromN1 -> f_EUTRA_RRC_ConnEst_DefWithNas -> f_EUTRA_RRC_ConnectionRequest_Common () -> where we wait for the message as below: [] SRB.receive(car_SRB0_RrcPdu_IND(p_CellId, cr_RRCConnectionRequest(p_EstablishmentCause, p_InitialUE_Identity))) However, due to wait for coordination from NR proc in f_EUTRA38_TrackingAreaUpdateFromN1() -> f_EUTRA38_TAUReqFromN1() to receive PDUSessionInfoList and GUTI parameters, EUTRA proc shall be paused waiting for the co-ordination messages from NR. Test case may fail this time, since the message in queue (rrcConnectionSetupRequest) may get handled in default. |
| Summary of change | Handled the connection request/SetupComplete/TAU etc along with Step 3 by calling function f_EUTRA38_TAUReqFromN1() At Step 5, f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq() is called to handle the remaining procedures of TAU. |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_1_3_10_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 3" siclog@ SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_HighPriorityAccess)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Power Level")); // @siclog "Step 5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, omit); // @sic R5s240517 sic@ }

After Change:

function fl_TC_8_1_1_3_10_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 3" siclog@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, -, omit, cr_EstablishmentCause_HighPriorityAccess); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Power Level")); // @siclog "Step 5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_NasInd); }

## Change 2

| Function name | fl_TC_8_1_1_3_10_TestBody |
| --- | --- |
| Reason for change | For the same reason as in change 2.1, there is change also needed in NR proc to send the Co-ordination messages from NR proc before receiving the Step 4 co-ordination message from EUTRA proc to change the NR cell Power level. |
| Summary of change | Moved call to function f_NR5GC_TrackingAreaUpdateFromN1() before waiting for co-ordination message f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"). |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_1_3_10_TestBody() runs on NR5GC_PTC { var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message (IE mpsPriorityIndication-r16 and redirectionInformation including ARFCN-ValueEUTRA of EUTRA cell 1). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo, -, -, cs_RRCConnectionRelease_All_IEs_v1540(-, cs_RRCRelease_v1610_IEs(cs_RRCRelease_v1650_IEs(true_))) ))); //@siclog "Step 4" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

After Change:

function fl_TC_8_1_1_3_10_TestBody() runs on NR5GC_PTC { var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message (IE mpsPriorityIndication-r16 and redirectionInformation including ARFCN-ValueEUTRA of EUTRA cell 1). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo, -, -, cs_RRCConnectionRelease_All_IEs_v1540(-, cs_RRCRelease_v1610_IEs(cs_RRCRelease_v1650_IEs(true_))) ))); //@siclog "Step 4" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }