3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240658

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3582 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Before Step 9, the reception of the UL HARQ is enabled. However, this causes issues when the RLC ACK for the RRC Reconfiguration complete at Steps 7 – 8 is transmitted to the UE to which the UE responds with an HARQ ACK. This, then, gets sent to TTCN where it is absorbed by the default handler causing the TC to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the enabling of the reception of UL HARQ after the delay timer of 1.28s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_1_12_3_NR_Steps9_11 |
| --- | --- |
| Reason for change | Before Step 9, the reception of the UL HARQ is enabled. However, this causes issues when the RLC ACK for the RRC Reconfiguration Complete at Steps 7 – 8 is transmitted to the UE to which the UE responds with an HARQ ACK. This, then, gets sent to TTCN where it is absorbed by the default handler causing the TC to fail. |
| Summary of change | Move the enabling of the reception of UL HARQ after the delay timer of 1.28s |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |
| MCC160 Comments | Accepted |

Before Change:

function f_TC_7_1_1_12_3_NR_Steps9_11(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var NR_SchedulingParameters_Type v_SchedulingParameters := f_NR_SchedulingParameters_DL(); //@sic R5s230001 sic@ //to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@sic R5s230001 sic@ //@siclog Step 9 siclog@ //Wait 1280ms to ensure UE is out DRX active time f_Delay(1.28); //@sic R5-233467 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_3_NR_Steps9_11(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var NR_SchedulingParameters_Type v_SchedulingParameters := f_NR_SchedulingParameters_DL(); //@sic R5s230001 sic@ //@siclog Step 9 siclog@ //Wait 1280ms to ensure UE is out DRX active time f_Delay(1.28); //@sic R5-233467 sic@ //WA#MOVED to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@sic R5s230001 sic@ <<SKIPPED CODE>>