3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250007

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3606 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.12.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some changes mentioned in submission TTCN CR R5s240686 were not implemented in 24wk50. However, these changes are needed for the successful execution of the TC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-implemented some of the changes mentioned in TTCN CR R5s240686 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	8

2.4	Change 4	10

2.5	Change 5	13

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1 |
| --- | --- |
| Reason for change | There are several changes required in this function to initialise the CellGroupConfig: - To initialise the DRX parameters, the MAC cell group needs to be extracted from the TTCN database and the DRX parameters initialised - The spCellConfigDedicated on the PCell needs to be configured with the DCI 2_6 search space along with searchSpacesToAddModListExt_r16 where the second entry belongs to the DCI 2_6 search space. - The additional BWP (bwp = 1) parameters of the SCell, BWP Common and BWP Dedicated need to be initialised which includes the searchSpacesToAddModList for USS and CSS |
| Summary of change | - Initialised the DRX parameters for the peer signalling to the UE via the RRC Reconfiguration message - TTCN code modifications to PCell to configure DCI 2_6 search space as well as added the entries searchSpacesToAddModListExt_r16 - Initialised the additional BWP parameters for the SCell |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, CellGroupId p_CellGroupId ) runs on NR_BASE_PTC return template (value) CellGroupConfig {//DRX adaptation / SCell dormancy indication var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SCellConfig v_SCellConfig; var template (omit) ServingCellConfigCommon v_ServingCellConfigCommon; var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated; var template (value) BWP_Downlink v_BWP_Downlink; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) ControlResourceSet v_ControlResourceSet; var template (value) ServingCellConfig v_SCellConfigDedicated; var template (value) DormantBWP_Config_r16 v_DormantBWP_Config; var SearchSpace v_SearchSpaceUSS; var template (omit) PDCCH_Config.controlResourceSetToAddModList v_ControlResourceSetToAddModList; var template (omit) BWP_DownlinkDedicated.pdcch_Config v_PDCCH_Config; //Table 7.1.1.12.4.1.3.3-2 v_CellGroupConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, p_CellGroupId); //configure DRX if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_CellGroupConfig.mac_CellGroupConfig); //compiler warning may be ignored //configure DCP v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_PCell); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); f_NR_SS_ServingCellConfig(p_PCell, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_CellGroupConfig.mac_CellGroupConfig, v_PhysicalCellGroupConfig))); v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; //configure PDCCH in SpCellConfig as per Table 7.1.1.12.4.1.3.3-3 v_SpCellConfig := f_NR_GetSpCellConfigDef(p_PCell); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(p_PCell, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := tsc_NR_ControlResourceSetId; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet}; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_Dci_Format2_6}; v_CellGroupConfig.spCellConfig := v_SpCellConfig; //configure ServingCellConfigCommon as per Table 7.1.1.12.4.1.3.3-4 v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon_CA(p_SCell); //configure BPW to add as per Table 7.1.1.12.4.1.3.3-6 v_SearchSpaceUSS := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_SCell, tsc_NR_SearchSpaceId_USS, tsc_NR_BWP_Id); v_ControlResourceSetToAddModList := f_NR_GetPDCCH_ConfigControlResourceSet(p_SCell); v_PDCCH_Config := {setup := cs_38508_PDCCH_Config (v_SearchSpaceUSS, -, v_ControlResourceSetToAddModList)}; v_BWP_DownlinkDedicated := cs_BWP_DownlinkDedicated_Def(v_PDCCH_Config); v_BWP_Downlink := cs_BWP_Downlink_Def (tsc_NR_BWP_Id1, omit, v_BWP_DownlinkDedicated); v_BWP_Downlink.bwp_Common.pdcch_ConfigCommon := omit; //@sic R5-235285 sic@ //configure serving cell as per Table 7.1.1.12.4.1.3.3-5 v_SCellConfigDedicated := f_NR_GetServingCellConfig(p_SCell); v_SCellConfigDedicated.uplinkConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.initialDownlinkBWP.radioLinkMonitoringConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.downlinkBWP_ToAddModList := {v_BWP_Downlink}; v_DormantBWP_Config := cs_NR_DormantBWP_Config_r16_Setup_OutsideActiveTime(tsc_NR_BWP_Id1); v_SCellConfigDedicated.dormantBWP_Config_r16.setup := v_DormantBWP_Config; //v_SCellConfigDedicated.pdsch_ServingCellConfig.setup := {pucch_Cell := 1}; //@sic R5-225297, R5-235285 sic@ if (ispresent (v_SCellConfigDedicated)) //compiler warning may be ignored due to check { v_SCellConfig := cs_SCellConfig_Def(v_ServingCellConfigCommon,v_SCellConfigDedicated, tsc_NR_SCellIndex_1); v_CellGroupConfig.sCellToAddModList := {v_SCellConfig}; } return v_CellGroupConfig } //f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1

After Change:

function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, CellGroupId p_CellGroupId ) runs on NR_BASE_PTC return template (value) CellGroupConfig {//DRX adaptation / SCell dormancy indication var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SCellConfig v_SCellConfig; var template (omit) ServingCellConfigCommon v_ServingCellConfigCommon; var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated; var template (value) BWP_Downlink v_BWP_Downlink; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) ControlResourceSet v_ControlResourceSet; var template (value) ServingCellConfig v_SCellConfigDedicated; var template (value) DormantBWP_Config_r16 v_DormantBWP_Config; var template (value) SearchSpace v_SearchSpaceUSS; //WA# var template (omit) PDCCH_Config.controlResourceSetToAddModList v_ControlResourceSetToAddModList; //WA# var template (omit) ServingCellConfig v_ServingCellConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template(value) SearchSpace v_SearchSpace; var template (value) DMRS_DownlinkConfig v_DMRS_DownlinkConfig; var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig; var template (value) BWP_DownlinkCommon v_BWP_DownlinkCommon; var SubcarrierSpacing v_SubcarrierSpacing; var BWP.locationAndBandwidth v_LocationAndBandwidth; var template (value) SearchSpace v_SearchSpaceCSS; var B45_Type v_Coreset_FrequencyDomain := '111100000000000000000000000000000000000000000'B; var template (value) PDCCH_Config v_PDCCH_Config; var template (value) PDSCH_Config v_PDSCH_Config; var SSB_Index v_SSB_Index := f_NR_CellInfo_GetSSB_Index(p_SCell); //Table 7.1.1.12.4.1.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell); //WA# //configure DRX if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); //WA# } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); //WA# } f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_MAC_CellGroupConfig); //WA# //WA# v_CellGroupConfig := cs_38508_CellGroupConfig (f_NR_CellInfo_Get_CellGroupId(p_PCell), omit, omit, omit, omit, omit ); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; //WA# //configure DCP v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_PCell); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); f_NR_SS_ServingCellConfig(p_PCell, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //WA# v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; //configure PDCCH in SpCellConfig as per Table 7.1.1.12.4.1.3.3-3 v_ServingCellConfig := f_NR_GetServingCellConfig(p_PCell); //WA# v_SpCellConfig := f_NR_GetSpCellConfigDef(p_PCell, v_ServingCellConfig); //WA# v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(p_PCell, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := tsc_NR_ControlResourceSetId; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet}; //WA# v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_PCell, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := tsc_NR_SearchSpaceId_DCI_2_6; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[1] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_AllOmit, cs_SearchSpaceExt_r16_Dci_Format2_6}; v_SpCellConfig.spCellConfigDedicated.tag_Id := 0; v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_PCell, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_AllOmit; v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(p_PCell, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR1; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR1; v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR2; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR2; v_NrofCandidates := cds_38508_NrofCandidates_FR2; } v_CellGroupConfig.spCellConfig := v_SpCellConfig; f_NR_SS_SearchSpaceConfig(p_PCell, -, v_DMRS_DownlinkConfig, v_PDSCH_TimeDomainResourceAllocationList, v_NrofCandidates, v_SearchSpace); v_PDSCH_Config := cs_38508_PDSCH_Config(v_DMRS_DownlinkConfig, -, v_SSB_Index); // Table 7.1.1.8.1.3.3-2C: PDSCH-Config v_PDSCH_Config.pdsch_TimeDomainAllocationList.setup := v_PDSCH_TimeDomainResourceAllocationList; //configure ServingCellConfigCommon as per Table 7.1.1.12.4.1.3.3-4 v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon_CA(p_SCell); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_SCell); v_LocationAndBandwidth := 1381; v_BWP_DownlinkCommon := cs_BWP_DownlinkCommon_GenericParametersOnly(cs_38508_BWP(v_SubcarrierSpacing, v_LocationAndBandwidth)); //configure BPW to add as per Table 7.1.1.12.4.1.3.3-6 v_ControlResourceSetToAddModList := { cs_38508_ControlResourceSet(9, -, v_Coreset_FrequencyDomain) }; v_SearchSpaceUSS := cs_38508_SearchSpace(37, 9, cs_NR_SearchSpace_SearchSpaceType_USS(formats0_1_And_1_1), cs_NR_SearchSpace_NrofCandidates_AggregationLevel2Only); v_PDCCH_Config := cs_38508_PDCCH_Config(v_SearchSpaceUSS, -, v_ControlResourceSetToAddModList); v_SearchSpaceCSS := cs_38508_SearchSpace(30, 9, cs_NR_SearchSpace_SearchSpaceType_CSS, cs_NR_SearchSpace_NrofCandidates_AggregationLevel2Only); v_PDCCH_Config.searchSpacesToAddModList[1]:= v_SearchSpaceCSS; v_BWP_DownlinkDedicated := cs_38508_BWP_DownlinkDedicated(v_PDCCH_Config, v_PDSCH_Config); v_BWP_Downlink := cs_BWP_Downlink_Def (tsc_NR_BWP_Id1, v_BWP_DownlinkCommon, v_BWP_DownlinkDedicated); v_BWP_Downlink.bwp_Common.pdcch_ConfigCommon := omit; //@sic R5-235285 sic@ //configure serving cell as per Table 7.1.1.12.4.1.3.3-5 v_SCellConfigDedicated := f_NR_GetServingCellConfig(p_SCell); v_SCellConfigDedicated.uplinkConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.initialDownlinkBWP.radioLinkMonitoringConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.downlinkBWP_ToAddModList := {v_BWP_Downlink}; v_DormantBWP_Config := cs_NR_DormantBWP_Config_r16_Setup_OutsideActiveTime(tsc_NR_BWP_Id1); v_SCellConfigDedicated.dormantBWP_Config_r16.setup := v_DormantBWP_Config; //v_SCellConfigDedicated.pdsch_ServingCellConfig.setup := {pucch_Cell := 1}; //@sic R5-225297, R5-235285 sic@ if (ispresent (v_SCellConfigDedicated)) //compiler warning may be ignored due to check { v_SCellConfig := cs_SCellConfig_Def(v_ServingCellConfigCommon,v_SCellConfigDedicated, tsc_NR_SCellIndex_1); v_CellGroupConfig.sCellToAddModList := {v_SCellConfig}; } return v_CellGroupConfig } //f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1

## Change 2

| Function name | f_NR_SS_SearchSpaceConfig |
| --- | --- |
| Reason for change | Refer to Change 2 |
| Summary of change | New Change |
| TTCN module | NR_ConfigurationSteps.ttcn |

function f_NR_SS_SearchSpaceConfig(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) DMRS_DownlinkConfig p_DMRS_DownlinkConfig, template (value) PDSCH_TimeDomainResourceAllocationList p_PDSCH_TimeDomainResourceAllocationList, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, template(value) SearchSpace p_SearchSpace) runs on NR_BASE_PTC { var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId); var NR_FrequencyBand_Type v_NR_FrequencyBand := f_NR_CellInfo_GetFrequencyBand(p_NR_CellId); var BWP.locationAndBandwidth v_LocationAndBandwidth := f_NR_InitLocationAndBandwidth(v_NR_FrequencyBand.FrequencyBandIndicator, v_NR_FrequencyBand.FrequencyInfoDL.scs_SpecificCarrierList[0].subcarrierSpacing, v_NR_FrequencyBand.ChannelBandwidth); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_SS_Config_REQ(p_NR_CellId, p_TimingInfo, p_DMRS_DownlinkConfig, p_PDSCH_TimeDomainResourceAllocationList, p_NrofCandidates, v_SubcarrierSpacing, v_NR_FrequencyBand,v_LocationAndBandwidth, p_SearchSpace)); }

## Change 3

| Template name | cads_NR_SS_Config_REQ |
| --- | --- |
| Reason for change | Refer to Change 2 |
| Summary of change | New Change |
| TTCN module | NR_CellCfg_Templates.ttcn |

template (value) NR_SYSTEM_CTRL_REQ cads_NR_SS_Config_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo, template (value) DMRS_DownlinkConfig p_DMRS_DownlinkConfig, template (value) PDSCH_TimeDomainResourceAllocationList p_PDSCH_TimeDomainResourceAllocationList, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, template (value) SubcarrierSpacing p_SubcarrierSpacing, template (value) NR_FrequencyBand_Type p_NR_FrequencyBand, BWP.locationAndBandwidth p_LocationAndBandwidth, template(value) SearchSpace p_SearchSpace ) modifies cas_NR_CellConfigAllOmit_REQ := { Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo), Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := { ActiveBWP := omit, BwpArray := { { Id := tsc_NR_BWP_Id, BWP := { R15 := cs_38508_BWP(p_SubcarrierSpacing, p_LocationAndBandwidth) }, Pdcch := { SearchSpaceArray := {cs_NR_BWP_SearchSpaceConfig_SearchSpaceZero(p_SubcarrierSpacing), cs_NR_BWP_SearchSpaceConfig_CSS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_USS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_SISS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(p_SearchSpace)}, CoresetArray := { cs_NR_ASN1_ControlResourceSet(cs_NR_ControlResourceSetZeroAtSS(0)), // CORESET#0 according to 38.211 clause 7.3.2.2 with shift index being the physical layer cell id @sic R5-190786, RP-182523 sic@ */ cs_NR_ASN1_ControlResourceSet(cs_38508_ControlResourceSet()) // default CORESET as according to 38.508-1 Table 4.6.3-17 }, RelativeTxPower := cs_NR_PDCCH_EPREs_Def, Coreset0_OffsetRBs := p_NR_FrequencyBand.Coreset0_OffsetRBs }, Pdsch := { ConfigCommon := { R15 := cs_38508_PDSCH_ConfigCommon(p_PDSCH_TimeDomainResourceAllocationList) }, ConfigDedicated := { R15 := cs_38508_PDSCH_Config(p_DMRS_DownlinkConfig, omit, tsc_NR_SSB_Index1) }, RelativeTxPower := cs_NR_PDSCH_EPREs_Def }, Sps := omit, SL_Pdcch := omit, NCD_SSPbchBlock := omit } }, InitialBWP_RedCap := omit }, CsiConfig := omit }, Uplink := omit } } } } };

## Change 4

| Function name | f_TC_7_1_1_12_4_NR_Steps3_9 |
| --- | --- |
| Reason for change | - As the sizeDCI2_6_r16 is setup as 6 entries, the local end SS configuration should have 6 entries when the Wakeup and SCellDormancyIndicationBitValue is configured - A delay of 11.0s needs to happen so that the complete long drx cycle of 10.240s is finished before Step 7 is started. - The DRB transmissions at Step 5 and Step 8 should happen on BWP#0 with data routed via the PDCCH of the SCell which does not necessitate the switching of the BWP. |
| Summary of change | - Used a new template called cs_NR_DciFormat_2_6_Block_List_6Items which matches the sizeDCI2_6_r16 - A delay of 11.0s is added to ensure the DRX long cycle is completed so that the DCI 2-6 is transmitted within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1 - Removed the configuration for the BWP switch. |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / SCell dormancy indication var UInt_Type v_LongDRXCycle; var UInt_Type v_DrxStartOffset; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_SendTiming; var SubFrameTiming_Type v_ConfigTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 5.0; //@siclog Step 3 siclog@ //The SS transmits a SCell Activation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(p_PCell); f_Delay(0.1); //@sic R5s240686 Ch 5.3 sic@ // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //f_NR_SS_SystemIndCtrlConfig(p_SCell, cds_NR_SystemIndCtrl_UL_HARQ(enable)); v_LongDRXCycle := 10240; //@sic R5s240686 Ch 5.5 sic@ v_DrxStartOffset := 0; v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 4 siclog@ //The SS transmits DCI 2-6 within ps-Offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 3) //The Wake-up indication is value 1 and the SCell dormancy indication is value 0 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_5Items('1'B, '0'B))); v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(0, p_DRBId); //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-225297 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Step 6 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 7 siclog@ //The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) //Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_5Items('1'B, '1'B))); //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Steps 9 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }//f_TC_7_1_1_12_4_NR_Steps3_9

After Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / SCell dormancy indication var UInt_Type v_LongDRXCycle; var UInt_Type v_DrxStartOffset; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_SendTiming; var SubFrameTiming_Type v_ConfigTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 5.0; //@siclog Step 3 siclog@ //The SS transmits a SCell Activation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(p_PCell); f_Delay(0.1); //@sic R5s240686 Ch 5.3 sic@ // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //f_NR_SS_SystemIndCtrlConfig(p_SCell, cds_NR_SystemIndCtrl_UL_HARQ(enable)); v_LongDRXCycle := 10240; //@sic R5s240686 Ch 5.5 sic@ v_DrxStartOffset := 0; v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 4 siclog@ //The SS transmits DCI 2-6 within ps-Offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 3) //The Wake-up indication is value 1 and the SCell dormancy indication is value 0 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items('1'B, '0'B))); v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(0, p_DRBId); //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_PCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Step 6 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } f_Delay(11.0); //WA# v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 7 siclog@ //The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) //Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_6Items('1'B, '1'B))); //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_PCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //WA# //@siclog Steps 9 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }//f_TC_7_1_1_12_4_NR_Steps3_9

## Change 5

| Function name | cs_NR_DciFormat_2_6_Block_List_6Items |
| --- | --- |
| Reason for change | Refer to change 5 |
| Summary of change | New Change |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

template (value) NR_DciFormat_2_6_Block_List_Type cs_NR_DciFormat_2_6_Block_List_6Items ( B1_Type p_WakeUP := '1'B, bitstring p_SCellDormacyIndicationBitValue := '0'B ) := { cs_NR_DciFormat_2_6_Block_Type(p_WakeUP, cs_NR_DciFormat_2_6_SCellDormancyIndication_Value(p_SCellDormacyIndicationBitValue)), cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type }