3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250015

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3492 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC testcase 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current test case implemenation presents following issues: - For the case of upto 2 BWPs, the number of BWPs are 2. Hence, the BWP Indicator must have a valid index rather than a NULL index - The template cads_NR_BWP_CONFIG_UL_REQ does not have the IEs SupplementaryUplink TimingAdvance PUSCH_ServingCellConfig PUSCH_ServingCellConfigSUL initialised - For the case of paired spectrum (=FDD), the dynamic switch at Step 5a has to be asymmetric as switching of the Downlink BWP and Uplink BWP should happen independently. - For the case of FDD and upto4 BWPs, it is necessary to bring the UL BWP to BWP #2 using a semi-static BWP switch so that the next dynamic BWP switch using UL DCI at Step 10b shifts the UL BWP to BWP #3 - For the case of FDD and upto2 BWPs at Step 10d, the shift of the UL BWP is to BWP #0 from BWP #0. Hence, here a semi-static BWP switch is required rather than a Dynamic BWP switch. - For the case of FDD and upto2 BWPs (Steps 11AABa1 – 11AABa4), the requirement is to shift the DL BWP from #0 to #1 using dynamic BWP switch using DL DCI and then at Step 11AABa2, the requirement is to provide an UL Grant on BWP #1. Hence, here a semi-static BWP switch is needed to switch the UL to BWP #1 - There is a redundant switch at the end of Steps 11AABa1 – 11AABa4 to switch the DL back to BWP#0. This is incorrect and should be removed as the PDCCH Order at Step 12d needs to be sent on BWP #1 - For the case of FDD and upto4 BWPs, a semi-static switch needs to be done to bring the DL BWP to BWP #3 as the last switch in this path had the UL BWP on BWP #3 and DL BWP on BWP #2 - For the case of paired spectrum (=FDD), the dynamic switch at Step 16 has to be asymmetric as switching of the Downlink BWP and Uplink BWP should happen independently. - For the case of FDD, The switch for the UL at Step 17 to BWP #0 is not needed as the UL BWP has remained on BWP#0 due to the DL only Dynamic BWP switch previously at Step 16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the condition to check the parameter p_NoPfBWPs >= 1 to set a valid index for the BWP indicator. - Modified template cads_NR_BWP_CONFIG_UL_REQ to have the IEs SupplementaryUplink TimingAdvance PUSCH_ServingCellConfig PUSCH_ServingCellConfigSUL initialised - Modified TTCN, for the case of FDD, to switch the BWP using Dynamic switch using DL DCI. - Modified TTCN, for the case of FDD and upto4 BWPs, a semi-static BWP switch to bring the UL BWP to BWP #2 - Modified TTCN, for the case of FDD and upto2 BWPs, a semi-static BWP switch is used rather than a Dynamic BWP switch. - Modified TTCN, for the case FDD and upto2 BWPs (Steps 11AABa1 – 11AABa4), to switch the BWP to BWP #1 using Dynamic switch using DL DCI and then used a semi-static BWP switch to bring the UL BWP to BWP #1 - Removed the redundant BWP switch at the end of Steps 11AABa1 – 11AABa4 - Modified TTCN, for the case of FDD and upto4 BWPs, a semi-static switch is used to bring the DL BWP to BWP #3 - Modified TTCN, for the case of FDD, to switch the BWP using Dynamic switch using DL DCI - Removed the BWP switch for UL before Step 17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC, 7.1.1.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	5

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:  Juan Garcia Martin
                juan.garciamartin@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_NR_DciCommon_BWPIndicator |
| --- | --- |
| Reason for change | For the case of upto 2 BWPs, the number of BWPs are 2. Hence, the BWP Indicator must have a valid index rather than a NULL index |
| Summary of change | Modified the condition to check the parameter p_NoPfBWPs >= 1 to set a valid index for the BWP indicator |
| TTCN module | MAC_BWP_Common_NR.ttcn |

Before Change:

function f_NR_DciCommon_BWPIndicator(integer p_NoPfBWPs, BWP_Id p_BwpId) return template (value) NR_DciCommon_BWPIndicator_Type { var template (value) NR_DciCommon_BWPIndicator_Type v_BWPIndicator; if (p_NoPfBWPs > 2) { v_BWPIndicator := cs_NR_DciCommon_BWPIndicator_MoreThanTwoBWPs(p_BwpId); } else if (p_NoPfBWPs > 1) { v_BWPIndicator := cs_NR_DciCommon_BWPIndicator_TwoBWPs(p_BwpId); } else { v_BWPIndicator := cs_NR_DciCommon_BWPIndicator_SingleBWP; } return v_BWPIndicator; }

After Change:

function f_NR_DciCommon_BWPIndicator(integer p_NoPfBWPs, BWP_Id p_BwpId) return template (value) NR_DciCommon_BWPIndicator_Type { var template (value) NR_DciCommon_BWPIndicator_Type v_BWPIndicator; if (p_NoPfBWPs > 2) { v_BWPIndicator := cs_NR_DciCommon_BWPIndicator_MoreThanTwoBWPs(p_BwpId); } else if (p_NoPfBWPs >= 1) { //WA# v_BWPIndicator := cs_NR_DciCommon_BWPIndicator_TwoBWPs(p_BwpId); } else { v_BWPIndicator := cs_NR_DciCommon_BWPIndicator_SingleBWP; } return v_BWPIndicator; }

## Change 2

| Template name | cads_NR_BWP_CONFIG_UL_REQ |
| --- | --- |
| Reason for change | The template cads_NR_BWP_CONFIG_UL_REQ does not have the IEs SupplementaryUplink TimingAdvance PUSCH_ServingCellConfig PUSCH_ServingCellConfigSUL initialised |
| Summary of change | Modified template cads_NR_BWP_CONFIG_UL_REQ to have the IEs SupplementaryUplink TimingAdvance PUSCH_ServingCellConfig PUSCH_ServingCellConfigSUL initialised |
| TTCN module | MAC_BWP_Common_NR.ttcn |

Before Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_BWP_CONFIG_UL_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ActiveUplinkBWP_Id_Type p_ActiveUplinkBWP_Id, template (value) NR_AssignedBWPs_Type p_AssignedBWPs, template (value) NR_DciUlInfo_Type p_DciUlInfo, template (omit) NR_UplinkBWP_List_Type p_UplinkBWP_List := omit) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ //@sic R5s210263 added IE AddOrReconfigure->PhysicalLayer->Common sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := omit, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := p_ActiveUplinkBWP_Id, BwpArray := p_UplinkBWP_List, InitialBWP_RedCap := omit }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } } } }, DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := { AssignedBWPs := p_AssignedBWPs, SearchSpaceType := omit, DciInfo := p_DciUlInfo }, PUCCH_Synch := omit, GrantConfig := omit, CoresetPoolIndex := omit //@sic R5w240003 sic@ }, DrxCtrl := omit, MeasGapCtrl := omit, DL2 := omit, //@sic R5w240003 sic@ UL2 := omit //@sic R5w240003 sic@ } } } } };

After Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_BWP_CONFIG_UL_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ActiveUplinkBWP_Id_Type p_ActiveUplinkBWP_Id, template (value) NR_AssignedBWPs_Type p_AssignedBWPs, template (value) NR_DciUlInfo_Type p_DciUlInfo, template (omit) NR_UplinkBWP_List_Type p_UplinkBWP_List := omit) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ //@sic R5s210263 added IE AddOrReconfigure->PhysicalLayer->Common sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := omit, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := p_ActiveUplinkBWP_Id, BwpArray := p_UplinkBWP_List, InitialBWP_RedCap := omit }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } }, SupplementaryUplink := omit, //WA# TimingAdvance := omit, PUSCH_ServingCellConfig := omit, PUSCH_ServingCellConfigSUL := omit } }, DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := { AssignedBWPs := p_AssignedBWPs, SearchSpaceType := omit, DciInfo := p_DciUlInfo }, PUCCH_Synch := omit, GrantConfig := omit, CoresetPoolIndex := omit //@sic R5w240003 sic@ }, DrxCtrl := omit, MeasGapCtrl := omit, DL2 := omit, //@sic R5w240003 sic@ UL2 := omit //@sic R5w240003 sic@ } } } } };

## Change 3

| Template name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | - For the case of paired spectrum (=FDD), the dynamic switch at Step 5a has to be asymmetric as switching of the Downlink BWP and Uplink BWP should happen independently. - For the case of FDD and upto4 BWPs, it is necessary to bring the UL BWP to BWP #2 using a semi-static BWP switch so that the next dynamic BWP switch using UL DCI at Step 10b shifts the UL BWP to BWP #3 - For the case of FDD and upto2 BWPs at Step 10d, the shift of the UL BWP is to BWP #0 from BWP #0. Hence, here a semi-static BWP switch is required rather than a Dynamic BWP switch. - For the case of FDD and upto2 BWPs (Steps 11AABa1 – 11AABa4), the requirement is to shift the DL BWP from #0 to #1 using dynamic BWP switch using DL DCI and then at Step 11AABa2, the requirement is to provide an UL Grant on BWP #1. Hence, here a semi-static BWP switch is needed to switch the UL to BWP #1 - There is a redundant switch at the end of Steps 11AABa1 – 11AABa4 to switch the DL back to BWP#0. This is incorrect and should be removed as the PDCCH Order at Step 12d needs to be sent on BWP #1 - For the case of FDD and upto4 BWPs, a semi-static switch needs to be done to bring the DL BWP to BWP #3 as the last switch in this path had the UL BWP on BWP #3 and DL BWP on BWP #2 - For the case of paired spectrum (=FDD), the dynamic switch at Step 16 has to be asymmetric as switching of the Downlink BWP and Uplink BWP should happen independently. - For the case of FDD, The switch for the UL at Step 17 to BWP #0 is not needed as the UL BWP has remained on BWP#0 due to the DL only Dynamic BWP switch previously at Step 16. |
| Summary of change | - Modified TTCN, for the case of FDD, to switch the BWP using Dynamic switch using DL DCI. - Modified TTCN, for the case of FDD and upto4 BWPs, a semi-static BWP switch to bring the UL BWP to BWP #2 - Modified TTCN, for the case of FDD and upto2 BWPs, a semi-static BWP switch is used rather than a Dynamic BWP switch. - Modified TTCN, for the case FDD and upto2 BWPs (Steps 11AABa1 – 11AABa4), to switch the BWP to BWP #1 using Dynamic switch using DL DCI and then used a semi-static BWP switch to bring the UL BWP to BWP #1 - Removed the redundant BWP switch at the end of Steps 11AABa1 – 11AABa4 - Modified TTCN, for the case of FDD and upto4 BWPs, a semi-static switch is used to bring the DL BWP to BWP #3 - Modified TTCN, for the case of FDD, to switch the BWP using Dynamic switch using DL DCI - Removed the BWP switch for UL before Step 17 |
| TTCN module | MAC_BWP_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 R5-244020 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var template (value) NR_PDCCH_Order_Type v_PDCCH_Order; var BWP_Id v_BWP_Id_PDCCH_Order; var SubFrameTiming_Type v_TimingInfoDL; var SubFrameTiming_Type v_TimingInfoSS; //@sic R5s240283 sic@ var SubFrameTiming_Type v_TimingInfoUL; var SubFrameTiming_Type v_TimingInfoConfig; var octetstring v_RLC_SDU1; var octetstring v_RLC_SDU2; var integer v_RA_PreambleIndex; var integer v_SN := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var SubFrameTiming_Type v_TimingInfoUL_SS; var BWP_Id v_BwpId_DL_DCI; //BWP on which the DL DCI is transmitted var BWP_Id v_BwpId_DL_DCI_IndicatorData; // Indicator inside DL BWP = BWP on which DL data is tranmsitted var BWP_Id v_BwpId_UL_DCI; //BWP on which the UL DCI is transmitted var BWP_Id v_BwpId_UL_DCI_IndicatorData; // Indicator inside UL BWP = BWP on which UL data is tranmsitted var boolean v_IsFDD := f_NR_CellInfo_GetIsFDD(p_NR_CellId); var integer v_MaxBWP := 3; if (not v_BWP_SameNumerology_upto4) { v_MaxBWP := 1; } // @siclog "Step 1" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 2" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 1 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; //@sic R5s240283 sic@ if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 5a - 6a - 7a //IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_2; } else { //Step 5b - 6b - 7b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } else { if (not(v_IsFDD)) { //Step 5c - 6c - 7c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD ) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } else { //Step 5d - 6d - 7d //IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); // @siclog "Step 5a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 6a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 5 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 7a/b/c/d-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 9.2 R5-2134599 sic@ // Reconfigure DL DCI to be transmitted on v_BwpId_DL_DCI_IndicatorData f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, v_MaxBWP); // @siclog "Step 9a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ //Initialise variable for step 10a/b/c/d if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 10a - 11a //IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_3; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } else { //Step 10b - 11b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_2; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } } else { if (not(v_IsFDD)) { //Step 10c - 11c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 10d - 11d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } } v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); } else { // For FDD reconfigure only UL BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } // @siclog "Step 10a/b/c/d" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11a/b/c/d-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if (v_IsFDD and not(v_BWP_SameNumerology_upto4)) { // for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1) v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_SN := v_SN + 1; //@siclog 11AABa1 siclog@ f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 11AABa2" siclog@ //After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 11AABa1. v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11AABa3 - 11AABa4" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, v_MaxBWP); } // configure CBRA CRNTI Based RACH procedure //@sic R5s230220 change 10 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure); v_SN := v_SN + 1; //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // @sic R5s230733 sic@ // @siclog "Step 11Aa/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @siclog "Step 12" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 9 v_RA_PreambleIndex := 0; // => contention-based random access v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order))); // @sic R5s230733 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); // @siclog "Step 13" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 13A - 13C" siclog@ // steps are done automatically according to configuration of the RACH procedure // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; if (not(v_IsFDD)) { //Step 16 - 18a/18c - 19a/c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 16 - 18b/d - 19b/d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } //@sic R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1); if (v_IsFDD) { // For FDD reconfigure only UL DCI BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } // @siclog "Step 17" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200); // 200ms after step 16 @sic R5s230220 R5-231578 sic@ v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 18a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200); // 200ms after step 17 @sic R5s230220 R5-231578 sic@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 19a/b/c/d-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20a/b/c/d" siclog@ //The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD). v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1200); // 1200ms after step 17 => 1000ms after step 18 @sic R5s240283 sic@ v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 21" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 20 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); //DL and UL BWP already configured on tsc_NR_BWP_Id_2/tsc_NR_BWP_Id_Initial @sic R5s230733 sic@ }

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 R5-244020 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var template (value) NR_PDCCH_Order_Type v_PDCCH_Order; var BWP_Id v_BWP_Id_PDCCH_Order; var SubFrameTiming_Type v_TimingInfoDL; var SubFrameTiming_Type v_TimingInfoSS; //@sic R5s240283 sic@ var SubFrameTiming_Type v_TimingInfoUL; var SubFrameTiming_Type v_TimingInfoConfig; var octetstring v_RLC_SDU1; var octetstring v_RLC_SDU2; var integer v_RA_PreambleIndex; var integer v_SN := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var SubFrameTiming_Type v_TimingInfoUL_SS; var BWP_Id v_BwpId_DL_DCI; //BWP on which the DL DCI is transmitted var BWP_Id v_BwpId_DL_DCI_IndicatorData; // Indicator inside DL BWP = BWP on which DL data is tranmsitted var BWP_Id v_BwpId_UL_DCI; //BWP on which the UL DCI is transmitted var BWP_Id v_BwpId_UL_DCI_IndicatorData; // Indicator inside UL BWP = BWP on which UL data is tranmsitted var boolean v_IsFDD := f_NR_CellInfo_GetIsFDD(p_NR_CellId); var integer v_MaxBWP := 3; if (not v_BWP_SameNumerology_upto4) { v_MaxBWP := 1; } // @siclog "Step 1" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 2" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 1 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; //@sic R5s240283 sic@ if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 5a - 6a - 7a //IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_2; } else { //Step 5b - 6b - 7b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } else { if (not(v_IsFDD)) { //Step 5c - 6c - 7c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD ) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } else { //Step 5d - 6d - 7d //IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //WA# if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); } else { // For FDD reconfigure only DL BWP f_NR_SS_ConfigureBWPs_DL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, -, -, v_MaxBWP); } // @siclog "Step 5a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 6a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 5 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 7a/b/c/d-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //WA# if (v_BWP_SameNumerology_upto4) { if (v_IsFDD) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_Now, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, v_MaxBWP); } } // @siclog "Step 9a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ //Initialise variable for step 10a/b/c/d if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 10a - 11a //IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_3; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } else { //Step 10b - 11b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_2; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } } else { if (not(v_IsFDD)) { //Step 10c - 11c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 10d - 11d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } } v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoUL_SS, 100); // 100ms after step 9 if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); } else { //WA# if (v_BWP_SameNumerology_upto4) { // For FDD reconfigure only UL BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } else { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); } } // @siclog "Step 10a/b/c/d" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11a/b/c/d-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if (v_IsFDD and not(v_BWP_SameNumerology_upto4)) { // for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1) v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_SN := v_SN + 1; //@siclog 11AABa1 siclog@ //WA# v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); f_NR_SS_ConfigureBWPs_DL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, -, -, v_MaxBWP); v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); //WA# f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, v_MaxBWP); // @siclog "Step 11AABa2" siclog@ //After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 11AABa1. v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11AABa3 - 11AABa4" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //WA# /*REMOVED v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_Now, v_BwpId_DL_DCI_IndicatorData, tsc_NR_BWP_Id_Initial, -, -, v_MaxBWP); */ } //WA# if(v_IsFDD and v_BWP_SameNumerology_upto4) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_Now, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, v_MaxBWP); } // configure CBRA CRNTI Based RACH procedure //@sic R5s230220 change 10 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure); v_SN := v_SN + 1; //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // @sic R5s230733 sic@ // @siclog "Step 11Aa/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @siclog "Step 12" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 9 v_RA_PreambleIndex := 0; // => contention-based random access v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order))); // @sic R5s230733 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); // @siclog "Step 13" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 13A - 13C" siclog@ // steps are done automatically according to configuration of the RACH procedure // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; if (not(v_IsFDD)) { //Step 16 - 18a/18c - 19a/c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 16 - 18b/d - 19b/d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } //@sic R5s240283 sic@ //@sic R5s230634 sic@ //WA# v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData); } else { // For FDD reconfigure only DL BWP f_NR_SS_ConfigureBWPs_DL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, -, -, v_MaxBWP); } // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1); /* REMOVED WA# if (v_IsFDD) { // For FDD reconfigure only UL DCI BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); }*/ // @siclog "Step 17" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200); // 200ms after step 16 @sic R5s230220 R5-231578 sic@ v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 18a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200); // 200ms after step 17 @sic R5s230220 R5-231578 sic@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 19a/b/c/d-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20a/b/c/d" siclog@ //The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD). v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1200); // 1200ms after step 17 => 1000ms after step 18 @sic R5s240283 sic@ v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 21" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 20 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); //DL and UL BWP already configured on tsc_NR_BWP_Id_2/tsc_NR_BWP_Id_Initial @sic R5s230733 sic@ }