3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250025

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0125 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Integrity protection added to NOTIFY for xcap-diff |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2024-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | C1-213597 has clarified XML security for xcap-diff but integrity protection is still missing in NOTIFY sent at initial registration |  |  |  |  |  |  |  |  |
| Summary of change: |  | f_MCX_SIP_ServiceAuthorization_SendNotify modified so that the NOTIFY sent at initial registration contains a MIME message with the xcap-diff document and its signature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing integrity protection |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MCX ATSs |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Extension of f_MCX_MIME_PartList_AddTX and f_MCX_MIME_MessageBodyTX	3

3	New common function f_MCX_SIP_SendNotify	6

4	Correction of f_MCX_SIP_ServiceAuthorization_SendNotify	7

5	Improvement of f_MCX_SIP_Notification	8

6	Improvement of f_MCX_NotifyRequest_MessageHeaderTX	9

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2024-06_D24wk37.

Contact:	Oliviers Genoud
	olivier.genoud@etsi.org

# Extension of f_MCX_MIME_PartList_AddTX and f_MCX_MIME_MessageBodyTX

| Functions | f_MCX_MIME_PartList_AddTX, f_MCX_MIME_MessageBodyTX |
| --- | --- |
| Reason for change | The XML security for NOTIFY messages sent from various servers at initial registration use different CSKs for each server |
| Summary of change | New parameter p_KeyIndex to specify the CSK to be used |
| TTCN modules | MCX_MIME_Functions |

function f_MCX_MIME_PartList_AddTX(MCX_ServiceType_Type p_ServiceType, MCX_KeyIndex_Type p_KeyIndex := CSK, template (value) MIME_Part_List p_MIME_Part_List, template (omit) SDP_Message p_SDP_Message := omit, template (omit) MCPTT_Info_Type p_McpttInfo := omit, template (omit) MCVideo_Info_Type p_McvideoInfo := omit, template (omit) MCData_Info_Type p_McdataInfo := omit, template (omit) MCPTT_LocationInfo_Type p_McpttLocationInfo := omit, template (omit) MCVideo_LocationInfo_Type p_McvideoLocationInfo := omit, template (omit) MCData_LocationInfo_Type p_McdataLocationInfo := omit, template (omit) ResourceLists_Type p_ResourceLists := omit, template (omit) MCX_Affiliation_CommandList_Type p_AffiliationCommand := omit, template (omit) MCX_XcapDiff_Type p_XcapDiff := omit, template (omit) MCX_PIDF_Type p_PIDF := omit, template (omit) PocSettings_Type p_PocSettings := omit, template (omit) ConferenceInfo_Type p_ConferenceInfo := omit, template (omit) McpttRegroup_Type p_McpttRegroup := omit) runs on MCX_IMS_PTC return template (value) MIME_Part_List { var template (value) MIME_Part_List v_MIME_Part_List := p_MIME_Part_List; var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var template (omit) MCX_SecurityKey_Type v_CSK := f_MCX_IMS_PTC_SecurityParameters_GetKey(p_KeyIndex); var charstring v_TypeAndSubtype; var XmlBody_Type v_XmlBody; if (isvalue(p_SDP_Message)) { v_MIME_Part_List := f_MIME_Part_List_AddSDP(v_MIME_Part_List, valueof(p_SDP_Message)); } if (isvalue(p_McpttInfo)) { v_XmlBody := f_MCX_MCPTTInfo_EncodeEncrypt(valueof(p_McpttInfo), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_McpttInfo, -, -, v_XmlBody); } if (isvalue(p_McvideoInfo)) { v_XmlBody := f_MCX_MCVideoInfo_EncodeEncrypt(valueof(p_McvideoInfo), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_McvideoInfo, -, -, v_XmlBody); } if (isvalue(p_McdataInfo)) { v_XmlBody := f_MCX_MCDataInfo_EncodeEncrypt(valueof(p_McdataInfo), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_McdataInfo, -, -, v_XmlBody); } if (isvalue(p_McpttLocationInfo)) { if (ischosen(p_McpttLocationInfo.choice.report)) { /* according to 24.379 F.3.3 "the contents of the subelements in the <CurrentLocation> subelement of the <Report> element can be encrypted" */ v_XmlBody := f_MCPTT_LocationInfoReport_EncodeEncrypt(valueof(p_McpttLocationInfo), v_MCServiceID, v_CSK); } else { v_XmlBody := f_MCPTT_LocationInfo_Encode(valueof(p_McpttLocationInfo)); } v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_McpttLocationInfo, -, -, v_XmlBody); } if (isvalue(p_McvideoLocationInfo)) { if (ischosen(p_McvideoLocationInfo.choice.report)) { /* according to 24.281 F.3.3 "the contents of the subelements in the <CurrentLocation> subelement of the <Report> element can be encrypted" */ v_XmlBody := f_MCVideo_LocationInfoReport_EncodeEncrypt(valueof(p_McvideoLocationInfo), v_MCServiceID, v_CSK); } else { v_XmlBody := f_MCVideo_LocationInfo_Encode(valueof(p_McvideoLocationInfo)); } v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_McvideoLocationInfo, -, -, v_XmlBody); } if (isvalue(p_McdataLocationInfo)) { if (ischosen(p_McdataLocationInfo.choice.report)) { /* according to 24.281 F.3.3 "the contents of the subelements in the <CurrentLocation> subelement of the <Report> element can be encrypted" */ v_XmlBody := f_MCData_LocationInfoReport_EncodeEncrypt(valueof(p_McdataLocationInfo), v_MCServiceID, v_CSK); } else { v_XmlBody := f_MCData_LocationInfo_Encode(valueof(p_McdataLocationInfo)); } v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_McdataLocationInfo, -, -, v_XmlBody); } if (isvalue(p_ResourceLists)) { v_XmlBody := f_MCX_ResourceLists_EncodeEncrypt(valueof(p_ResourceLists), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, c_xmlresourcelistApplication, -, -, v_XmlBody); } if (isvalue(p_AffiliationCommand)) { select (p_ServiceType) { case (MCPTT) { v_TypeAndSubtype := tsc_MimeType_McpttAffiliationCommand; } case (MCVideo) { v_TypeAndSubtype := tsc_MimeType_McvideoAffiliationCommand; } case (MCData, MCDataSDS, MCDataFD, MCDataIPCONN) { v_TypeAndSubtype := tsc_MimeType_McdataAffiliationCommand; } case else { FatalError(__FILE__, __LINE__, "Invalid service type"); } } v_XmlBody := f_MCX_AffiliationCommand_Encode(valueof(p_AffiliationCommand)); /* according to 24.379 F.4 there is no encryption for a affiliate-command and the group URIs are no attribute URIs */ v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, v_TypeAndSubtype, -, -, v_XmlBody); } if (isvalue(p_XcapDiff)) { v_XmlBody := f_MCX_XcapDiff_EncodeEncrypt(valueof(p_XcapDiff), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_XcapDiff, -, -, v_XmlBody); } if (isvalue(p_PIDF)) { v_XmlBody := f_MCX_PIDF_EncryptEncode(valueof(p_PIDF), v_MCServiceID, v_CSK); /* attribute URI encryption */ v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_Pidf, -, -, v_XmlBody); } if (isvalue(p_PocSettings)) { v_XmlBody := f_MCX_PocSettings_Encode(valueof(p_PocSettings)); /* PoC Settings are not encrypted */ v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_pocSettings, -, -, v_XmlBody); } if (isvalue(p_ConferenceInfo)) { v_XmlBody := f_MCX_ConferenceInfo_EncodeEncrypt(valueof(p_ConferenceInfo), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_ConferenceInfo, -, -, v_XmlBody); } if (isvalue(p_McpttRegroup)) { v_XmlBody := f_MCX_Mcpttregroup_EncodeEncrypt(valueof(p_McpttRegroup), v_MCServiceID, v_CSK); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_Mcpttregroup, -, -, v_XmlBody); } v_MIME_Part_List := f_MCX_MIME_Part_List_AddSignatures(p_ServiceType, v_MIME_Part_List, v_CSK); return v_MIME_Part_List; }

function f_MCX_MIME_MessageBodyTX(MCX_ServiceType_Type p_ServiceType, MCX_KeyIndex_Type p_KeyIndex := CSK, template (omit) SDP_Message p_SDP_Message := omit, template (omit) MCPTT_Info_Type p_McpttInfo := omit, template (omit) MCVideo_Info_Type p_McvideoInfo := omit, template (omit) MCData_Info_Type p_McdataInfo := omit, template (omit) MCPTT_LocationInfo_Type p_McpttLocationInfo := omit, template (omit) MCVideo_LocationInfo_Type p_McvideoLocationInfo := omit, template (omit) MCData_LocationInfo_Type p_McdataLocationInfo := omit, template (omit) ResourceLists_Type p_ResourceLists := omit, template (omit) MCX_Affiliation_CommandList_Type p_AffiliationCommand := omit, template (omit) MCX_XcapDiff_Type p_XcapDiff := omit, template (omit) MCX_PIDF_Type p_PIDF := omit, template (omit) PocSettings_Type p_PocSettings := omit, template (omit) ConferenceInfo_Type p_ConferenceInfo := omit, template (omit) McpttRegroup_Type p_McpttRegroup := omit) runs on MCX_IMS_PTC return template (value) MessageBody { var template (value) MIME_Part_List v_MIME_Part_List := {}; v_MIME_Part_List := f_MCX_MIME_PartList_AddTX(p_ServiceType, p_KeyIndex, v_MIME_Part_List, p_SDP_Message, p_McpttInfo, p_McvideoInfo, p_McdataInfo, p_McpttLocationInfo, p_McvideoLocationInfo, p_McdataLocationInfo, p_ResourceLists, p_AffiliationCommand, p_XcapDiff, p_PIDF, p_PocSettings, p_ConferenceInfo, p_McpttRegroup); if (lengthof(v_MIME_Part_List) < 2) { FatalError(__FILE__, __LINE__, "Less than 2 MIME body parts for MIME message"); } return cs_MessageBody_MIME(cs_MIME_Message(tsc_MCX_MIME_Boundary, v_MIME_Part_List)); }

# New common function f_MCX_SIP_SendNotify

| Function | f_MCX_SIP_SendNotify |
| --- | --- |
| Reason for change | There is no case anymore where the NOTIFY has a message body different than MIME common function f_MCX_SIP_SendNotify can be used for initial registration and for the other cases |
| Summary of change | New common function f_MCX_SIP_SendNotify |
| TTCN modules | MCX_SIP_Registration |

function f_MCX_SIP_SendNotify(MCX_ServiceType_Type p_ServiceType, IMS_PTC_DialogIndex_Type p_DialogIndex, InternetProtocol_Type p_Protocol, MCX_KeyIndex_Type p_KeyIndex := CSK, template (omit) MCPTT_Info_Type p_McpttInfo := omit, template (omit) MCVideo_Info_Type p_McvideoInfo := omit, template (omit) MCData_Info_Type p_McdataInfo := omit, template (omit) MCX_XcapDiff_Type p_XcapDiff := omit, template (omit) MCX_PIDF_Type p_PIDF := omit, template (omit) PocSettings_Type p_PocSettings := omit, template (omit) ConferenceInfo_Type p_ConferenceInfo := omit) runs on MCX_IMS_PTC return template (value) MessageHeader { var SipUrl v_RequestURI := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog, p_DialogIndex); var SipUrl v_ContactURI := f_IMS_PTC_ImsInfo_GetContactUrlSS(p_DialogIndex); var template (value) MessageHeader v_MessageHeader_Notify := f_MCX_NotifyRequest_MessageHeaderTX(p_ServiceType, p_DialogIndex, v_ContactURI); var template (value) MessageBody v_MessageBody_Notify := f_MCX_MIME_MessageBodyTX(p_ServiceType, p_KeyIndex, -, p_McpttInfo, p_McvideoInfo, p_McdataInfo, -, -, -, -, -, p_XcapDiff, p_PIDF, p_PocSettings, p_ConferenceInfo); IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(p_Protocol), cs_NOTIFY_Request(v_RequestURI, v_MessageHeader_Notify, v_MessageBody_Notify))); return v_MessageHeader_Notify; }

# Correction of f_MCX_SIP_ServiceAuthorization_SendNotify

| Function | f_MCX_SIP_ServiceAuthorization_SendNotify |
| --- | --- |
| Reason for change | Integrity protection is needed for the xcap-diff document in the NOTIFY sent at initial registration |
| Summary of change | New common function f_MCX_SIP_SendNotify used which adds signature by using f_MCX_MIME_MessageBodyTX |
| TTCN modules | MCX_SIP_Registration |

function f_MCX_SIP_ServiceAuthorization_SendNotify(MCX_ServiceType_Type p_ServiceType, IMS_PTC_DialogIndex_Type p_DialogIndex, InternetProtocol_Type p_Protocol, template (value) MCX_XcapDiff_Type p_XcapDiff) runs on MCX_IMS_PTC return template (value) MessageHeader { // @sic R5w220111, R5-223945: global variable ContactUrlSS => p_ContactUrl removed sic@ var MCX_KeyIndex_Type v_KeyIndex := fl_MCX_XcapDiffDialog_GetKeyIndex(p_DialogIndex); var template (value) MessageHeader v_MessageHeader; v_MessageHeader := f_MCX_SIP_SendNotify(p_ServiceType, p_DialogIndex, p_Protocol, v_KeyIndex, -, -, -, p_XcapDiff); return v_MessageHeader; }

# Improvement of f_MCX_SIP_Notification

| Function | f_MCX_SIP_Notification |
| --- | --- |
| Reason for change | New common function f_MCX_SIP_SendNotify |
| Summary of change | Use of f_MCX_SIP_SendNotify |
| TTCN modules | MCX_SIP_Registration |

function f_MCX_SIP_Notification(MCX_ServiceType_Type p_ServiceType, IMS_PTC_DialogIndex_Type p_DialogIndex, template (omit) MCPTT_Info_Type p_McpttInfo := omit, template (omit) MCVideo_Info_Type p_McvideoInfo := omit, template (omit) MCData_Info_Type p_McdataInfo := omit, template (omit) MCX_PIDF_Type p_PIDF := omit, template (omit) PocSettings_Type p_PocSettings := omit, template (omit) ConferenceInfo_Type p_ConferenceInfo := omit) runs on MCX_IMS_PTC { /* NOTE: further MIME bodies may be added */ var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var template (value) MessageHeader v_MessageHeader_Notify; var template (present) MessageHeader v_MessageHeader_200OK; // Send NOTIFY v_MessageHeader_Notify := f_MCX_SIP_SendNotify(p_ServiceType, p_DialogIndex, v_Protocol, -, p_McpttInfo, p_McvideoInfo, p_McdataInfo, -, p_PIDF, p_PocSettings, p_ConferenceInfo); // Receive 200 OK v_MessageHeader_200OK := f_MCX_CommonResponse_200_MessageHeaderRX(v_MessageHeader_Notify); f_IMS_ReceiveResponse(v_MessageHeader_Notify, v_Protocol, cr_Response(c_statusLine200, v_MessageHeader_200OK)); }

# Improvement of f_MCX_NotifyRequest_MessageHeaderTX

| Function | f_MCX_NotifyRequest_MessageHeaderTX |
| --- | --- |
| Reason for change | There is no case anymore where the NOTIFY has a message body different than MIME |
| Summary of change | Parameter p_ContentType removed; Content-Type hard-code to MIME |
| TTCN modules | MCX_SIP_Registration |

function f_MCX_NotifyRequest_MessageHeaderTX(MCX_ServiceType_Type p_ServiceType, IMS_PTC_DialogIndex_Type p_DialogIndex, SipUrl p_ContactURI ) runs on IMS_PTC return template (value) MessageHeader { var EventType v_EventType := f_SIP_SUBSCRIBE_DialogIndex2EventType(p_DialogIndex); var charstring v_PcscfAddress := f_IMS_PTC_Pcscf_Get(); var template (value) ContentType v_ContentType := cs_ContentTypeMIME(tsc_MCX_MIME_Boundary); var integer v_MaxForwards := 67; // 36.579-1 Table 5.5.2.8-1 var charstring v_ServerName; var charstring v_PublicServiceId; var CharStringList_Type v_ViaAddressList; var template (value) Via v_Via; var template (value) MessageHeader v_NotifyHeader; select (p_DialogIndex) { case (xcapDiffDialogCMS) { v_ServerName := tsc_MCX_CMS_Hostname; // 36.579-1 Table 5.5.2.8-1 } case (xcapDiffDialogGMS1, xcapDiffDialogGMS2, xcapDiffDialogGMS3, xcapDiffDialogGMS4) { v_ServerName := tsc_MCX_GMS_Hostname; // 36.579-1 Table 5.5.2.8-1 } case else { v_ServerName := f_SIP_SipUrl_GetDomainName(p_ContactURI); } } v_ViaAddressList := {v_PcscfAddress, tsc_IMS_Scscf, v_ServerName}; v_Via := f_IMS_NotifyRequest_ViaTX(v_ViaAddressList); v_NotifyHeader := f_IMS_NotifyRequest_MessageHeader_Common(p_DialogIndex, v_Via, cs_Contact(p_ContactURI), v_EventType, cs_SubscriptionState_Active("7200"), v_ContentType, v_MaxForwards); if (p_DialogIndex == confEventDialog) { // 36.579-2 Table 6.1.3.1.3.3-9 and Table 6.1.3.1.3.3-12 v_PublicServiceId := f_MCX_MCServer_PublicServiceId(p_ServiceType); v_NotifyHeader.pAssertedID := cs_PAssertedId(f_SIP_BuildSipUri_TX(v_PublicServiceId)); v_NotifyHeader.expires := cs_Expires("3600"); v_NotifyHeader.subscriptionState := cs_SubscriptionState_Active("3600"); v_NotifyHeader.pPreferredService := cs_PPreferredService(tsc_MCX_FeatureTag_3gpp_service_ims_icsi_mcptt); } return v_NotifyHeader; }