3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250033

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3615 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC 5GSM test case 8.1.5.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current implementation at the step 2a1, UE capability RAT containers (nr and eutra_nr) are not decoded. It should be good to see also these containers decoded for better checking reported UE capabilities |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TTCN at the step 2a1 to decode also UE capability RAT containers like nr and eutra_nr) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Worse conditions to check reported UE capabilities |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

2.3	Change 3	11

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_9_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In current implementation at the step 2a1, UE capability RAT containers (nr and eutra_nr) are not decoded. It should be good to see also these containers decoded for better checking reported UE capabilities |
| Summary of change | Update TTCN at the step 2a1 to decode also UE capability RAT containers like nr and eutra_nr) |
| TTCN module | 8_1_5\RRC_Others_NR5GC.ttcn |

Before Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200933 R5-204456 R5-216276 sic@ var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var template (value) FreqBandList v_FreqBandList := f_NR_Build_FreqBandList_MaxCap(tsc_ListFreqBandIndicator); // Acc. Table 8.1.5.9.1.3.3-3 var octetstring v_EncodeCapRequestNR := bit2oct(encvalue(cs_38508_UECapabilityRequestFilterNR(v_FreqBandList))); var template (value) UE_CapabilityRAT_RequestList v_CapRAT_RequestList:= {cs_UE_CapabilityRAT_Request(nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestNR)}; //@sic R5-223425 sic@ var octetstring v_EncodeUECapabilityEnquiry_v1560_RACS := bit2oct(encvalue(cs_UECapabilityEnquiry_v1560_RACS)); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var SubFrameTiming_Type v_TimingInfoStep4; //Step 4 var SubFrameTiming_Type v_TimingInfoStep5; //Step 5 timer t_Watchdog := 5.0; //@siclog "Step 1" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ // IF UE's encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 will be performed alt { // EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment { if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive v_Completed_AllSegments := true; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact last message } else { // notLastSegment received if (v_MessageSegmentRx == 15) { f_NR_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2a1: Received a notLastSegment as 16th segment from the UE"); } v_MessageSegmentRx := v_MessageSegmentRx + 1; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact message repeat; } } //Step 2b1: Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UeCapabilityInformation(tsc_NR_RRC_TI_Def, ({cr_UE_CapabilityRAT_Container_NR(nr), cr_UE_CapabilityRAT_Container_NR(eutra_nr)}, {cr_UE_CapabilityRAT_Container_NR(nr)})))) -> value v_ReceivedAspForUeCapabilityInfo //@sic R5s221130, R5-245663 sic@ { if (v_MessageSegmentRx>0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE radio access capability information received after a segment"); // VerdictFail case: step 2b1 is mutually exclusive with 2ax } f_NR_8_1_5_9_1_Steps2b2_2b7(); //@sic R5s220330 sic@ } };//end alt // Decode and store the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments if (lengthof(v_Rx_SegmentOct) < tsc_Max_NR_PDCP_SDU_size) { //<9000 @sic R5s220330 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size"); } f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //@sic R5s220330 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a1"); } //@siclog "Step 3" siclog@ // SS stops regular grant transmission upon scheduling request. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 4" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. v_TimingInfoStep4 := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep4), cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ //@siclog "Step 5" siclog@ //The SS transmits periodic grant from (80*2microsec+1+TDL) slots after step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. v_TimingInfoStep5 := fl_CalculateTiming_8_1_5_9_1(v_TimingInfoStep4); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep5), valueof (cs_NR_ResourceAllocationDef), 10, 15); //15 periodic grants every 10ms @sic R5s221222 sic@ //@siclog "Step 6" siclog@ // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(0)))); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // stop periodic grant //@siclog "Step 7" siclog@ // The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.9.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded. f_NR_SetCellPowerList (v_CellPowerList_AtT1); //@siclog "Step 8" siclog@ // Wait for T310 to expire f_Delay(1.0); //@sic R5s220330 sic@ //@siclog "Step 9" siclog@ // The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. -> Done inside f_NR5GC_RRCReestablishment_Def @sic R5s230515 sic@ //@siclog "Step 10 - 12" siclog@ //@siclog "Step 12A - 12B" siclog@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, ?, -,-,-,-,-,-, v_CellPowerList_AtT2); //@sic R5s230515 sic@ //@siclog "Step 13" siclog@ // SS resumes regular UL grant transmission. f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, -, -, cs_UL_GrantConfig_OnSR); //@siclog "Step 14" siclog@ // Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and // does not send any ULDedicatedMessageSegment message in the next 5 seconds? t_Watchdog.start; alt { [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(?)))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected segment received"); } } }

After Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200933 R5-204456 R5-216276 sic@ var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var template (value) FreqBandList v_FreqBandList := f_NR_Build_FreqBandList_MaxCap(tsc_ListFreqBandIndicator); // Acc. Table 8.1.5.9.1.3.3-3 var octetstring v_EncodeCapRequestNR := bit2oct(encvalue(cs_38508_UECapabilityRequestFilterNR(v_FreqBandList))); var template (value) UE_CapabilityRAT_RequestList v_CapRAT_RequestList:= {cs_UE_CapabilityRAT_Request(nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestNR)}; //@sic R5-223425 sic@ var octetstring v_EncodeUECapabilityEnquiry_v1560_RACS := bit2oct(encvalue(cs_UECapabilityEnquiry_v1560_RACS)); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var SubFrameTiming_Type v_TimingInfoStep4; //Step 4 var SubFrameTiming_Type v_TimingInfoStep5; //Step 5 var UL_DCCH_Message v_UL_Dcch_Msg; var UE_NR_Capability v_NRCapability; var UE_MRDC_Capability v_NRCapability_MRDC; timer t_Watchdog := 5.0; //@siclog "Step 1" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ // IF UE's encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 will be performed alt { // EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment { if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive v_Completed_AllSegments := true; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact last message } else { // notLastSegment received if (v_MessageSegmentRx == 15) { f_NR_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2a1: Received a notLastSegment as 16th segment from the UE"); } v_MessageSegmentRx := v_MessageSegmentRx + 1; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact message repeat; } } //Step 2b1: Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UeCapabilityInformation(tsc_NR_RRC_TI_Def, ({cr_UE_CapabilityRAT_Container_NR(nr), cr_UE_CapabilityRAT_Container_NR(eutra_nr)}, {cr_UE_CapabilityRAT_Container_NR(nr)})))) -> value v_ReceivedAspForUeCapabilityInfo //@sic R5s221130, R5-245663 sic@ { if (v_MessageSegmentRx>0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE radio access capability information received after a segment"); // VerdictFail case: step 2b1 is mutually exclusive with 2ax } f_NR_8_1_5_9_1_Steps2b2_2b7(); //@sic R5s220330 sic@ } };//end alt // Decode and store the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments if (lengthof(v_Rx_SegmentOct) < tsc_Max_NR_PDCP_SDU_size) { //<9000 @sic R5s220330 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size"); } v_UL_Dcch_Msg := f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //@sic R5s220330 sic@ v_NRCapability := f_NR_DecodeNrCapMsg(v_UL_Dcch_Msg.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NRCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_UL_Dcch_Msg.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a1"); } //@siclog "Step 3" siclog@ // SS stops regular grant transmission upon scheduling request. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 4" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. v_TimingInfoStep4 := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep4), cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ //@siclog "Step 5" siclog@ //The SS transmits periodic grant from (80*2microsec+1+TDL) slots after step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. v_TimingInfoStep5 := fl_CalculateTiming_8_1_5_9_1(v_TimingInfoStep4); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep5), valueof (cs_NR_ResourceAllocationDef), 10, 15); //15 periodic grants every 10ms @sic R5s221222 sic@ //@siclog "Step 6" siclog@ // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(0)))); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // stop periodic grant //@siclog "Step 7" siclog@ // The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.9.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded. f_NR_SetCellPowerList (v_CellPowerList_AtT1); //@siclog "Step 8" siclog@ // Wait for T310 to expire f_Delay(1.0); //@sic R5s220330 sic@ //@siclog "Step 9" siclog@ // The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. -> Done inside f_NR5GC_RRCReestablishment_Def @sic R5s230515 sic@ //@siclog "Step 10 - 12" siclog@ //@siclog "Step 12A - 12B" siclog@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, ?, -,-,-,-,-,-, v_CellPowerList_AtT2); //@sic R5s230515 sic@ //@siclog "Step 13" siclog@ // SS resumes regular UL grant transmission. f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, -, -, cs_UL_GrantConfig_OnSR); //@siclog "Step 14" siclog@ // Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and // does not send any ULDedicatedMessageSegment message in the next 5 seconds? t_Watchdog.start; alt { [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(?)))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected segment received"); } } }

## Change 2

| Function name | f_NR_8_1_5_9_1_Steps2b2_2b7 |
| --- | --- |
| Reason for change | Following change1 |
| Summary of change | Following change1 |
| TTCN module | 8_1_5\RRC_Others_NR5GC.ttcn |

Before Change:

function f_NR_8_1_5_9_1_Steps2b2_2b7() runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment; var template (value) FreqBandList v_FreqBandList := { cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }; var octetstring v_EncodeCapRequestNR := bit2oct(encvalue(cs_38508_UECapabilityRequestFilterNR(v_FreqBandList))); var template (value) UE_CapabilityRAT_RequestList v_CapRAT_RequestList:= {cs_UE_CapabilityRAT_Request(nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestNR)}; //@sic R5-223425 sic@ var octetstring v_EncodeUECapabilityEnquiry_v1560_RACS := bit2oct(encvalue(cs_UECapabilityEnquiry_v1560_RACS)); var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..1) var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; //@siclog "Step 2b2-2b3" siclog@ // The SS transmits a SET UL MESSAGE REQUEST message. // The UE transmits a SET UL MESSAGE RESPONSE message. f_NR5GC_SetULMessage(nr_Cell1); //@siclog "Step 2b4" siclog@ // The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ //@siclog "Step 2b5" siclog@ //Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? //EXCEPTION: Step 2b5 is repeated a maximum of 2 times. alt { [] SRB.receive( car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment { if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive v_Completed_AllSegments := true; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact last message } else { // notLastSegment received if (v_MessageSegmentRx == 2) { f_NR_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2b5: Received a notLastSegment as 2nd segment from the UE"); } v_MessageSegmentRx := v_MessageSegmentRx + 1; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact message repeat; } } [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UeCapabilityInformation(tsc_NR_RRC_TI_Def, ?))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected UeCapabilityInformation received"); } };//end alt // Decode the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //@sic R5-220330 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2b5"); } //@siclog "Step 2b6-2b7" siclog@ // The SS transmits a SET UL MESSAGE REQUEST message. // The UE transmits a SET UL MESSAGE RESPONSE message. f_NR5GC_SetULMessage(nr_Cell1); }

After Change:

function f_NR_8_1_5_9_1_Steps2b2_2b7() runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment; var template (value) FreqBandList v_FreqBandList := { cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }; var octetstring v_EncodeCapRequestNR := bit2oct(encvalue(cs_38508_UECapabilityRequestFilterNR(v_FreqBandList))); var template (value) UE_CapabilityRAT_RequestList v_CapRAT_RequestList:= {cs_UE_CapabilityRAT_Request(nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestNR)}; //@sic R5-223425 sic@ var octetstring v_EncodeUECapabilityEnquiry_v1560_RACS := bit2oct(encvalue(cs_UECapabilityEnquiry_v1560_RACS)); var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..1) var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var UL_DCCH_Message v_UL_Dcch_Msg; var UE_NR_Capability v_NRCapability; var UE_MRDC_Capability v_NRCapability_MRDC; //@siclog "Step 2b2-2b3" siclog@ // The SS transmits a SET UL MESSAGE REQUEST message. // The UE transmits a SET UL MESSAGE RESPONSE message. f_NR5GC_SetULMessage(nr_Cell1); //@siclog "Step 2b4" siclog@ // The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ //@siclog "Step 2b5" siclog@ //Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? //EXCEPTION: Step 2b5 is repeated a maximum of 2 times. alt { [] SRB.receive( car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment { if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive v_Completed_AllSegments := true; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact last message } else { // notLastSegment received if (v_MessageSegmentRx == 2) { f_NR_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2b5: Received a notLastSegment as 2nd segment from the UE"); } v_MessageSegmentRx := v_MessageSegmentRx + 1; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact message repeat; } } [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UeCapabilityInformation(tsc_NR_RRC_TI_Def, ?))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected UeCapabilityInformation received"); } };//end alt // Decode the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments v_UL_Dcch_Msg := f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //@sic R5-220330 sic@ v_NRCapability := f_NR_DecodeNrCapMsg(v_UL_Dcch_Msg.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[0].ue_CapabilityRAT_Container); v_NRCapability_MRDC := f_NR_DecodeNrCapMsg_MRDC(v_UL_Dcch_Msg.message_.c1.ueCapabilityInformation.criticalExtensions.ueCapabilityInformation.ue_CapabilityRAT_ContainerList[1].ue_CapabilityRAT_Container); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2b5"); } //@siclog "Step 2b6-2b7" siclog@ // The SS transmits a SET UL MESSAGE REQUEST message. // The UE transmits a SET UL MESSAGE RESPONSE message. f_NR5GC_SetULMessage(nr_Cell1); }

## Change 3

| Function name | f_NR_DecodeULDcchMsg |
| --- | --- |
| Reason for change | Following change1 |
| Summary of change | Following change1 |
| TTCN module | 8_1_5\RRC_Others_NR5GC.ttcn |

Before Change:

function f_NR_DecodeULDcchMsg(octetstring p_Octetstring) { //@sic R5s220330 sic@ var UL_DCCH_Message v_UL_Dcch_Msg; var bitstring v_Bitstring := oct2bit(p_Octetstring); var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE NR Capability cannot be decoded"); } }

After Change:

function f_NR_DecodeULDcchMsg(octetstring p_Octetstring) return UL_DCCH_Message { //@sic R5s220330 sic@ var UL_DCCH_Message v_UL_Dcch_Msg; var bitstring v_Bitstring := oct2bit(p_Octetstring); var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE NR Capability cannot be decoded"); } return v_UL_Dcch_Msg; }