3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250047

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3603 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_PDUSessionEstablishment_InnerLoop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current TTCN implementaiton of this function, the v_PDUType is now assigned based on PICS settings. The function f_GetPDNTypeFromDNN --> f_GetDefaultAPN_DNNType_FromPICS does not assign Emergency_DNN. This results in an emergency PDU session REQ also being set as IMS_DNN. This setting is then used in updating variable v_GSM_MobilityInfo, which is used in many places. This causes issues later in the function while extracting PDN Index , and the setting of EPS Bearer ID etc. based on v_GSM_MobilityInfo. Also the variable v_MediaType is set to IPCAN_EmergencyCall even if v_GSM_MobilityInfo.PDUType is set as IMS_DNN. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the received requestvalue IE in the UL NAS Transport, set the variable v_GSM_MobilityInfo.PDUType to Emergency_DNN and v_MediaType to IPCAN_EmergencyCall. Please see below for detailed implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All testcases that have an Emergency PDU Session establishment |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_NR5GC_PDUSessionEstablishment_InnerLoop |
| --- | --- |
| Reason for change | In current TTCN implementaiton of this function, the v_PDUType is now assigned based on PICS settings. The function f_GetPDNTypeFromDNN --> f_GetDefaultAPN_DNNType_FromPICS does not assign Emergency_DNN. This results in an emergency PDU session REQ also being set as IMS_DNN. This setting is then used in updating variable v_GSM_MobilityInfo, which is used in many places. This causes issues later in the function while extracting PDN Index , and the setting of EPS Bearer ID etc. based on v_GSM_MobilityInfo. Also the variable v_MediaType is set to IPCAN_EmergencyCall even if v_GSM_MobilityInfo.PDUType is set as IMS_DNN. This needs to be addressed. |
| Summary of change | Based on the received requestvalue IE in the UL NAS Transport, set the variable v_GSM_MobilityInfo.PDUType to Emergency_DNN and v_MediaType to IPCAN_EmergencyCall. Please see below for detailed implementation. |
| TTCN module | NR5GC_NASSteps.ttcn |

Before Change:

<<SKIPPED CODE>> if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@ // Ethernet PDU session if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address"); } if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) { v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header"); } } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case(IMS_DNN, Emergency_PDN) { // @sic R5-206296, R5s210518 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486, R5s240641 sic@ } case (URLLC_DNN) { v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@ } case else { v_SDAP_Config := cs_SDAP_Config; } } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@ // Ethernet PDU session if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address"); } if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) { v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header"); } } //WA#WI1367986 if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B) or match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '100'B)) { //This is according to 24.501 cl 9.3.11.47. v_GSM_MobilityInfo.PDUType := Emergency_PDN; v_MediaType := IPCAN_EmergencyCall; } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@ case(IMS_DNN, Emergency_PDN) { // @sic R5-206296, R5s210518 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); //WA#WI1367986 COMMENTED OUT v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486, R5s240641 sic@ } case (URLLC_DNN) { v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@ } case else { v_SDAP_Config := cs_SDAP_Config; } } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ <<SKIPPED CODE>>