3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250054

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3617 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In case of pc_bwp_sameNumerology_upto4_FR1_FDD = TRUE the current prose defines that step 5b performs downlink BWP switch from BWP#1 to BWP#2. However it also defines that in step 11Ab the SS transmits RLC PDU in the BWP#3. Since there is no other switching of downlink BWP between steps 5b and 11Ab, this is a conflict.In order to resolve this conflict we propose to change that step 5b performs downlink BWP switch BWP#1->BWP#3. Prose CR(R5-250280) associated with this change - The BWP Id for step 10d (i.e. FDD, pc_BWP_SameNumerology_upto2) on which the DCI of UL grant is going to be sent should be 1 and not 0, according to 38.523-1. - In step 11AABa2 the DCI of UL grant is going to be 0 and not 1, according to 38.523-1. - We believe that SS configuration of the DL BWP in step 11AABa4 is not needed because SS was fully configured in step 11AABa1 using combined template for both UL and DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Step 5b performs downlink BWP switch BWP#1->BWP#3 - Corrected the DCI BWP Id for step 10d - Corrected the DCI BWP Id for step 11AABa2 - Removed an extra SS BWP configuration in step 11AABa4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | In case of pc_bwp_sameNumerology_upto4_FR1_FDD = TRUE the current prose defines that step 5b performs downlink BWP switch from BWP#1 to BWP#2. However it also defines that in step 11Ab the SS transmits RLC PDU in the BWP#3. Since there is no other switching of downlink BWP between steps 5b and 11Ab, this is a conflict. In order to resolve this conflict we propose to change that step 5b performs downlink BWP switch BWP#1->BWP#3. Prose CR(R5-250280) associated with this change. |
| Summary of change | Step 5b performs downlink BWP switch BWP#1->BWP#3 |
| TTCN module | MAC_BWP_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; //@sic R5s240283 sic@ v_DL_SwitchIndicator := ActiveBWP_SwitchWithDCI; if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { // TDD //step 5a - 6a - 7a //IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_UL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { // FDD //Step 5b - 6b - 7b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_UL_SwitchIndicator := omit; } } else { if (not(v_IsFDD)) { // TDD //Step 5c - 6c - 7c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD ) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_UL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { //Step 5d - 6d - 7d //IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_UL_SwitchIndicator := omit; } }

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; //@sic R5s240283 sic@ v_DL_SwitchIndicator := ActiveBWP_SwitchWithDCI; if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { // TDD //step 5a - 6a - 7a //IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_UL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { // FDD //Step 5b - 6b - 7b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_UL_SwitchIndicator := omit; } } else { if (not(v_IsFDD)) { // TDD //Step 5c - 6c - 7c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD ) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_UL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { //Step 5d - 6d - 7d //IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_UL_SwitchIndicator := omit; } }

## Change 2

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | - The BWP Id for step 10d (i.e. FDD, pc_BWP_SameNumerology_upto2) on which the DCI of UL grant is going to be sent should be 1 and not 0, according to 38.523-1. - In step 11AABa2 the DCI of UL grant is going to be 0 and not 1, according to 38.523-1. |
| Summary of change | - Corrected the DCI BWP Id for step 10d - Corrected the DCI BWP Id for step 11AABa2 |
| TTCN module | MAC_BWP_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Step 9a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ //Initialise variable for step 10a/b/c/d v_UL_SwitchIndicator := ActiveBWP_SwitchWithDCI; if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //TDD //step 10a - 11a //IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_3; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_DL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { //Step 10b - 11b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_2; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_DL_SwitchIndicator := omit; } } else { if (not(v_IsFDD)) { //Step 10c - 11c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_DL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { // Step 10d - 11d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_DL_SwitchIndicator := omit; } } v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP, -, -, -, v_DL_SwitchIndicator, v_UL_SwitchIndicator); } else { // For FDD reconfigure only UL BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP, v_UL_SwitchIndicator); } // @siclog "Step 10a/b/c/d" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11a/b/c/d-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if (not(v_IsFDD)) { //TDD reconfigure UL DCI on new BWPId f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, -, v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, -, v_MaxBWP, omit); } if (v_IsFDD and not(v_BWP_SameNumerology_upto4)) { // for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1) v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_UL_SwitchIndicator := ActiveBWP_SwitchWithDCI; v_DL_SwitchIndicator := ActiveBWP_SwitchWithDCI; v_SN := v_SN + 1; //@siclog 11AABa1 siclog@

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Step 9a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ //Initialise variable for step 10a/b/c/d v_UL_SwitchIndicator := ActiveBWP_SwitchWithDCI; if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //TDD //step 10a - 11a //IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_3; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_DL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { //Step 10b - 11b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_2; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_DL_SwitchIndicator := omit; } } else { if (not(v_IsFDD)) { //Step 10c - 11c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_DL_SwitchIndicator := ActiveBWP_SwitchNoDCI; } else { // Step 10d - 11d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_DL_SwitchIndicator := omit; } } v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP, -, -, -, v_DL_SwitchIndicator, v_UL_SwitchIndicator); } else { // For FDD reconfigure only UL BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP, v_UL_SwitchIndicator); } // @siclog "Step 10a/b/c/d" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11a/b/c/d-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if (not(v_IsFDD)) { //TDD reconfigure UL DCI on new BWPId f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, -, v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, -, v_MaxBWP, omit); } if (v_IsFDD and not(v_BWP_SameNumerology_upto4)) { // for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1) v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_UL_SwitchIndicator := ActiveBWP_SwitchWithDCI; v_DL_SwitchIndicator := ActiveBWP_SwitchWithDCI; v_SN := v_SN + 1; //@siclog 11AABa1 siclog@

## Change 3

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | We believe that SS configuration of the DL BWP in step 11AABa4 is not needed because SS was fully configured in step 11AABa1 using combined template for both UL and DL. |
| Summary of change | Removed an extra SS BWP configuration in step 11AABa4 |
| TTCN module | MAC_BWP_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Steps 11AABa3 - 11AABa4" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, -, -, -, v_MaxBWP, omit);

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { … // @siclog "Steps 11AABa3 - 11AABa4" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //REMOVED v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //REMOVED f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, -, -, -, v_MaxBWP, omit);