3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250055

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3618 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 8.2.3.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The value of fr1-gap-r15 and mgta-r15 IE is not aligned to Table 8.2.3.11.1.3.3-1 in the current TTCN implementation, it is incorrectly set to false in the RRCConnectionReconfiguration at step11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the value of fr1-gap-r15 and mgta-r15 to TRUE to align with Table 8.2.3.11.1.3.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.11.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC |
| --- | --- |
| Reason for change | The value of fr1-gap-r15 and mgta-r15 IE is not aligned to Table 8.2.3.11.1.3.3-1 in the current TTCN implementation, it is incorrectly set to false in the RRCConnectionReconfiguration at step11. |
| Summary of change | Changed the value of fr1-gap-r15 and mgta-r15 to TRUE to align with Table 8.2.3.11.1.3.3-1 |
| TTCN module | RRCMeasurement_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_f1; var ChannelBandwidthDependency_Type v_ChBandDependency_f1; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell3; /************Skipping the code *****************/ //@siclog "Step 10" siclog@ //Wait 10s f_Delay(v_TimerValue_10s); //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration including measConfig to setup fr1-Gap and //nr-Config IE containing measConfig to report periodical measurements for NR serving Cell 1 and //NR neighbor cell 3 on FR1 frequency //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of fr1-Gap //and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_508_ENDC_Reconfiguration_MeasGap(eutra_Cell1, -, v_Security_Params.SK_Counter); //@siclog "Step 13" siclog@ //Wait and ignore MeasurementReport messages for 8 s to allow for UE to measure the neighbouring cells. /************Skipping the code *****************/ }

After Change:

function fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_f1; var ChannelBandwidthDependency_Type v_ChBandDependency_f1; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell3; /************Skipping the code *****************/ //@siclog "Step 10" siclog@ //Wait 10s f_Delay(v_TimerValue_10s); //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration including measConfig to setup fr1-Gap and //nr-Config IE containing measConfig to report periodical measurements for NR serving Cell 1 and //NR neighbor cell 3 on FR1 frequency //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of fr1-Gap //and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_508_ENDC_Reconfiguration_MeasGap(eutra_Cell1,cs_Meassurement_Config_Gap_ENDC(-,true,true) , v_Security_Params.SK_Counter); //@siclog "Step 13" siclog@ //Wait and ignore MeasurementReport messages for 8 s to allow for UE to measure the neighbouring cells. /************Skipping the code *****************/ }