3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250008

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3607 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC testcases 6.4.2.2, 8.1.1.4.2 and 8.1.6.1.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In NR5GC test cases 6.4.2.2, 8.1.1.4.2 and 8.1.6.1.4.2 there is no FollowedByRRCResume procedure after Preamble. Thus this needs to be set in correctly in function f_NR5GC_Preamble for STATE_INACTIVE_2A state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a boolean flag p_FollowedByRRCResume with default value as tsc_RRC_InactiveFollowedByRRCResume in function f_NR5GC_Preamble so that individual test case can set it correclty based on test prose steps requirement. Please see change sets for more details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2.NR5GC, 8.1.1.4.2.NR5GC and 8.1.6.1.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to f_NR5GC_Preamble	2

2.2	Change to f_TC_8_1_1_4_2_NR5GC	5

2.3	Change to f_TC_8_1_6_1_4_2_NR5GC	6

2.4	Change to f_TC_6_4_2_2_NR5GC	8

# Overview

This document lists all the essential changes needed to correct problems in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to f_NR5GC_Preamble

| Function name | f_NR5GC_Preamble |
| --- | --- |
| Reason for change | In NR5GC test cases 6.4.2.2, 8.1.1.4.2 and 8.1.6.1.4.2 there is no FollowedByRRCResume procedure after Preamble. Thus this needs to be set in correctly in function f_NR5GC_Preamble for STATE_INACTIVE_2A state. |
| Summary of change | Added a boolean flag p_FollowedByRRCResume with default value as tsc_RRC_InactiveFollowedByRRCResume in function f_NR5GC_Preamble so that individual test case can set it correclty based on test prose steps requirement. Please see change sets for more details. |
| TTCN module | Common\NR5GC\NR5GC_NASSteps.ttcn |
| MCC160 Comment | Alternative solution implemented: In f_NR5GC_Preamble : replace p_ActivateSecurityInactiveState with p_InactiveStateFollowedByRRCResume. In f_NR_RRC_InactiveDef: remove p_ActivateSecurityInactiveState and activate security in the Target cell. See MCC160 proposed implementation below. |

Before Change:

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B, template (omit) PreamblePreconfigureDataList p_PreconfigureUe := omit) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // send UT functions to UE before cell switched on fl_NR5GC_PreconfigUE(p_PreconfigureUe); // @sic R5-225300, R5-237389 sic@ // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); // @sic R5s210189 sic@ } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, tsc_RRC_InactiveFollowedByRRCResume, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

After Change:

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B, template (omit) PreamblePreconfigureDataList p_PreconfigureUe := omit, boolean p_FollowedByRRCResume := tsc_RRC_InactiveFollowedByRRCResume) runs on NR5GC_PTC //WA#WI=1367424 { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // send UT functions to UE before cell switched on fl_NR5GC_PreconfigUE(p_PreconfigureUe); // @sic R5-225300, R5-237389 sic@ // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); // @sic R5s210189 sic@ } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, p_FollowedByRRCResume, -, p_ActivateSecurityInactiveState); //WA#WI=1367424 } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

MCC160 proposed implementation

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId,

NG_STATE_Type p_State,

NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF,

boolean p_InactiveStateFollowedByRRCResume:= true,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (present) B4_Type p_ServiceType := '0000'B,

template (omit) PreamblePreconfigureDataList p_PreconfigureUe := omit)  runs on NR5GC_PTC

{ //@sic R5s190505: added p_ActivateSecurityInactiveState R5s250008: renamed to p_InactiveStateFollowedByRRCResume sic@

//@sic R5-199036: added p_IpPduDelayTime sic@

var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@

if (p_State == STATE_INACTIVE_2A) {

v_ReleaseConnection := noRrcConnectionRelease;

}

// send UT functions to UE before cell switched on

fl_NR5GC_PreconfigUE(p_PreconfigureUe); // @sic R5-225300, R5-237389 sic@

// turn on NR cell

f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session

// Switch on UE

f_NR5GC_SwitchOnAndResetIMS();

// Go to RRC_IDLE

f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); // @sic R5s210189 sic@

}

select (p_State) {

case (STATE_OFF_0B) {

// Now turn off again

f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A);

}

case (STATE_INACTIVE_2A) {

f_NR_RRC_InactiveDef(p_NR_CellId, p_InactiveStateFollowedByRRCResume);

}

case (STATE_CONNECTED_3A) {

// Go to RRC_CONNECTED

f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode);

}

}

}

==================================================================

function f_NR_RRC_InactiveDef(NR_CellId_Type p_NR_CellId,

boolean p_FollowedByRRCResume := tsc_RRC_InactiveFollowedByRRCResume,

template (omit) PLMN_Identity p_PLMN_Identity := omit,

template (omit) RRCRelease_v1540_IEs p_RRCRelease_v1540 := omit,

template (omit) SuspendConfig.sdt_Config_r17 p_SDT_Config := omit,

boolean p_ReestablishSDT_DRB := false,

template (omit) NR_RachProcedureConfig_Type p_NR_RachProcedureConfig := omit,

template (omit) SuspendConfig.ran_ExtendedPagingCycleConfig_r18  p_RAN_ExtendedPagingCycleConfig_r18 := omit,

template (omit) NR_CellId_Type p_NR_TargetRRCResume_CellId := omit

) runs on NR5GC_PTC return SubFrameTiming_Type

{ //@sic R5-195325 sic@

//@sic R5-243537 added return v_TimingRRCRelease => time at which RRCRelease is sent sic@

//@sic Added p_ReestablishSDT_DRB and p_NR_RachProcedureConfig for SDT tc sic@

//@sic R5s240440: added p_FollowedByRRCResume and p_NR_TargetRRCResume_CellId sic@

var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(p_NR_CellId);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

var template (omit) NR_RadioBearerList_Type v_RadioBearerList := omit;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var SubFrameTiming_Type v_TimingRRCRelease;

var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(p_NR_CellId);

var NR_CellId_Type v_RACHReconfigurationCell := p_NR_CellId;  //Cell on which RACH needs to be reconfigured - cell on which RRCResume is transmitted

if (isvalue(p_NR_TargetRRCResume_CellId)) { v_RACHReconfigurationCell := valueof(p_NR_TargetRRCResume_CellId);}

if (p_ReestablishSDT_DRB) {

v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList);   //For SDT: reconfigure all DRBs -> to ensure UE uses only the SDT-DRB

}

if (isvalue(p_NR_RachProcedureConfig)) {

f_NR_SS_RachProcedureConfig(v_RACHReconfigurationCell, p_NR_RachProcedureConfig);

} else if (p_FollowedByRRCResume) {

//@sic R5s240440 sic@

/* Configure RACH procedure to a list with 2 RACH:

- RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI => To send RRCReseumeRequest

- RACH procedure with contention-based with C-RNTI based contention resolution => In case UE use RACH procedure to send RLC ACK for RRCResume

*/

f_NR_SS_RachProcedureConfig(v_RACHReconfigurationCell, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def));

}

v_TimingRRCRelease := f_NR_RRCRelease_Common (p_NR_CellId, cs_38508_RRCRelease ( tsc_NR_RRC_TI_Def,

cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( v_CellIdentity, p_PLMN_Identity), p_SDT_Config, p_RAN_ExtendedPagingCycleConfig_r18),

-, -, -,

p_RRCRelease_v1540

),

-,

v_RadioBearerList);

if (p_FollowedByRRCResume){

//Activate security at SS side in advance so that SS is ready to send RRCResume message

// Only SRBs are configured - so security is activated on SRBs

// When DRBs are configured then security is also started on DRBs

//@sic R5s190505 change 2 sic@

f_NR_SS_AS_ActivateSecurity_Current(v_RACHReconfigurationCell,tsc_NR_38508_NextHopChainingCount);

}

return v_TimingRRCRelease;

}

Example of change in TC where UE is brought to Inactive state in another cell as it will perform RRCResume. Similar change done in

f_TC_6_4_2_1_NR5GC_TestBody

f_TC_6_4_2_3_NR5GC_TestBody

fl_TC_8_1_1_4_1_TestBody

=========================================================================================

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC

{// @sic R5s201047, R5-203382, R5s210094, R5-210062, R5s210454, R5-216995, R5-220543, R5s220481, R5-233462 sic@

var template (value) NR_CellPowerList_Type v_CellPowerList_T0;

var template (value) NR_CellPowerList_Type v_CellPowerList_T1;

var template (value) NR_CellPowerList_Type v_CellPowerList_T2;

var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var PLMN_Identity v_ASN_PLMN;

var NAS_PlmnId v_PLMN;

var NR_DRBInfoList_Type v_ExistingDRBs;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var integer i := 0;

var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList;

var template (value) CellGroupConfig v_SCGConfig;

var template (value) SpCellConfig v_SpCellConfig;

var NR_UplinkBWP_Type v_NR_UplinkBWP;

timer t_Tmin;

timer t_Tmax;

v_CellPowerList_T0 := {

cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)

};

v_CellPowerList_T1 := {

cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell13, -78, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)

};

v_CellPowerList_T2 := {

cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)

};

// @siclog "Step 0A" siclog@

// SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2

f_NR_SetCellPowerList(v_CellPowerList_T0);

// @siclog "Step 0B" siclog@

// The UE is switched on

f_UT_SwitchOnUE(UT);

// @siclog "Step 0C1-0C19a1" siclog@

// The steps 1-19Ta1 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed

v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12);

v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity);

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

f_NR5GC_GetMobileIdGUTI (nr_Cell12),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)),

{bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}),

cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13))));  // Table 6.4.1.1.3.3-3

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

f_NR5GC_Registration_Complete(nr_Cell12);

if (pc_noOf_PDUsSameConnection < 0 ) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection < 0");

}

f_NR5GC_PDUSessionEstablishment(nr_Cell12, -, tsc_NR_RbId_SRB1);

// @siclog "Step 0D" siclog@

// SS starts timers of tmin = 2min and tmax = (6 minutes + 5min)

t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0));

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0));

// @siclog "Step 0Ea1-0F" siclog@

// steps 19TAa1-20 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed

if (pc_noOf_PDUsNewConnection >0 or pc_noOf_PDUsSameConnection == 0){ // @sic R5s240120, R5-241166 sic@

f_NR_RRCRelease(nr_Cell12);

f_NR5GC_RRC_Idle_Extension(nr_Cell12); // @sic R5-236305 sic@

}

f_NR_RRC_InactiveDef(nr_Cell12, tsc_RRC_InactiveFollowedByRRCResume, -,  -, -, -, -, -, nr_Cell13);

v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

v_NR_RLC_BearerToAddModList[0] := cs_NR_BearerConfigDef(f_NR_LogicalChannelId_SRB(tsc_SRB2), omit, omit, cs_38508_LogicalChannelConfig_SRB(tsc_SRB2), true_);

for (i:= 0; i < lengthof(v_ExistingDRBs); i := i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

v_NR_RLC_BearerToAddModList[i+1] := cs_NR_BearerConfigDef(f_NR_LogicalChannelId_DRB(v_ExistingDRBs[i].DRB_Id), omit, omit, omit, true_);

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP (nr_Cell13, tsc_NR_BWP_Id);

v_NR_UplinkBWP.Dedicated.R15.pusch_Config := omit;

v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell13, cs_NR_ServingCellConfigDef(omit, 0, omit, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit)));

v_SCGConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(nr_Cell13), v_NR_RLC_BearerToAddModList, -, -, -, v_SpCellConfig);

f_NR5GC_MobileInfo_SetSessionInfoList({}); // @sic R5s210407 sic@

// @siclog "Step 1" siclog@

// SS adjusts cell levels according to row T1 of table 6.4.1.1.3.2-1/2

f_NR_SetCellPowerList(v_CellPowerList_T1);

// @siclog "Step 2" siclog@

// Check: Does the UE transmit an RRCResumeRequest message on NR Cell 13?

f_NR_RRC_ResumeRequest_Def(nr_Cell13, -, mo_Signalling);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

Example of change in TC where the security parameter simply needs to be removed. Similar change done in:

f_TC_6_4_1_2_NR5GC_TestBody

f_TC_6_4_2_2_NR5GC_TestBody

f_TC_6_4_2_3_NR5GC_TestBody

f_TC_6_6_1_2_NR5GC_TestBody

f_TC_6_7_2_1_NR5GC_TestBody

f_TC_6_7_2_2_NR5GC_TestBody

f_TC_7_1_1_13_1_NR_TestBody

f_TC_7_1_1_13_2_NR_TestBody

f_TC_7_1_1_13_3_NR_TestBody

f_TC_7_1_1_13_4_NR_TestBody

fl_TC_7_1_1_13_5_NR5GC_TestBody

fl_TC_8_1_1_1a_2_TestBody

fl_TC_8_1_1_4_1_TestBody

fl_TC_8_1_1_4_2_TestBody

f_TC_8_1_5_11_3_NR5GC_TestBody

f_TC_8_1_5_11_4_NR5GC_TestBody

f_TC_8_1_5_13_1_NR5GC_TestBody

f_TC_8_1_5_13_2_NR5GC_TestBody

f_TC_8_1_5_13_3_NR_TestBody

fl_TC_8_2_6_3_3_TestBody

fl_TC_8_2_6_3_4_TestBody

f_TC_11_7_3_NR5GC_TestBody

fl_TC_11_7_4_NR5GC_TestBody

fl_TC_14_1_1_1_NR5GC_TestBody

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC

{//@sic R5s200638 R5-204545 R5-213163 R5s210892 R5-233462 sic@

var template (value) NR_CellPowerList_Type v_CellPowerList_T0;

var template (value) NR_CellPowerList_Type v_CellPowerList_T1;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var float v_TimerValue1 := 60.0;

timer t_WaitTime;

v_CellPowerList_T0 := {

cs_NR_CellPower (nr_Cell1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2),

cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2),

cs_NR_CellPower (nr_Cell13, -78, tsc_NR_ServingCellSSS_EPRE_FR2)

};

v_CellPowerList_T1 := {

cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)

};

// @siclog "Step 1" siclog@

// SS adjusts cell levels according to row T0 of table 6.4.1.2.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_T0);

// @siclog "Step 2" siclog@

// Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12?

f_NR_RRC_ResumeRequest_Def(nr_Cell12,-, mo_Signalling);  // @sic R5s200892 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Steps 3-6" siclog@

// Steps 2 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 with condition MOBILITY are performed

// Note 1: The REGISTRATION REQUEST is accepted with the PLMN of NR Cell 1 listed as an Equivalent PLMN

f_NR_ULGrantConfiguration_Start(nr_Cell12); // @sic R5s210042 sic@

f_NR_RRCSetup_Def (nr_Cell12);

// receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH):

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility,

v_GMM_MobilityInfo,  // @sic R5-206426 sic@

f_NR5GC_GetMobileIdGUTI (nr_Cell12),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)),

{bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}),

cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1))),   // Table 6.4.1.2.3.3-4: REGISTRATION ACCEPT for NR Cell 12

cs_NSSAI_SSTs('15'O) // @sic R5-211331 sic@

);

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12,

tsc_NR_RbId_SRB1,

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

// step 15 of the Registration procedure

f_NR5GC_Registration_Complete(nr_Cell12);

// @siclog "Step 6A-6D" siclog@

// Steps 5 to 8 of the generic test procedure in TS 38.508-1 Table 4.5.4.2-3.

// @sic R5-215699 sic@ Note 3: No SERVICE ACCEPT message should be included in step 6C (corresponding to step 7 of Table 4.5.4.2-3 in TS 38.508-1)

f_NR_RRC_ActivateSecurity (nr_Cell12, v_ReceivedMsg.SecurityProtection.NasCount);

f_NR_RRCReconfigExistingDRBs(nr_Cell12, omit);

// @siclog "Step 7" siclog@

// The SS transmits an RRCRelease message with suspend.

f_NR_RRC_InactiveDef(nr_Cell12, tsc_RRC_InactiveNotFollowedByRRCResume, f_NR_CellInfo_GetPLMN (nr_Cell12), -, -, -, -, -, nr_Cell1);

// @siclog "Step 8" siclog@

// Check: Does the UE send an RRCResumeRequest on NR Cell 13 and NR Cell 1 within 60s?

t_WaitTime.start(v_TimerValue1);

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND((nr_Cell1, nr_Cell13), cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update)))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.4.1.2 Step 8");

}

[] t_WaitTime.timeout{}

}

// @siclog "Step 9" siclog@

// SS adjusts cell levels according to row T1 of table 6.4.1.2.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_T1);

// @siclog "Step 10" siclog@

// Set UE to Automatic PLMN selection mode

// Note 2:  Step 10 is to ensure UE is set back to automatic PLMN selection mode for the next test case

f_UT_AutomaticPLMN_Select(UT);

} // end of f_TC_6_4_1_2_NR5GC_TestBody

## Change to f_TC_8_1_1_4_2_NR5GC

| Function name | f_TC_8_1_1_4_2_NR5GC |
| --- | --- |
| Reason for change | Test body steps are not followed by rrcResume procedure after Preamble. i.e., Step 3 - The SS transmits an RRCSetup message in response to RRCResumeRequest message. |
| Summary of change | Set p_FollowedByRRCResume as tsc_RRC_InactiveNotFollowedByRRCResume. |
| TTCN module | NR5GC\8_1_1\RRC_Resume_NR5GC.ttcn |
| MCC160 Comment | Accepted – See MCC160 implementation as follows: f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, tsc_RRC_InactiveNotFollowedByRRCResume); |

Before Change:

function f_TC_8_1_1_4_2_NR5GC() runs on NR5GC_PTC { // RRC resume / Suspend-Resume / RRC setup / T319 expiry f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false); f_NR_TestBody_Set(true); fl_TC_8_1_1_4_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change:

function f_TC_8_1_1_4_2_NR5GC() runs on NR5GC_PTC { // RRC resume / Suspend-Resume / RRC setup / T319 expiry f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false,-, -, -, -, tsc_RRC_InactiveNotFollowedByRRCResume); //WA#WI=1367424 f_NR_TestBody_Set(true); fl_TC_8_1_1_4_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

## Change to f_TC_8_1_6_1_4_2_NR5GC

| Function name | f_TC_8_1_6_1_4_2_NR5GC |
| --- | --- |
| Reason for change | Test body steps are not followed by rrcResume procedure after Preamble. i.e., Step 3A - The UE transmits an RRCSetupRequest message. |
| Summary of change | Set p_FollowedByRRCResume as tsc_RRC_InactiveNotFollowedByRRCResume. |
| TTCN module | NR5GC\8_1_6\RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment | Accepted – See MCC160 implementation as follows: f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, tsc_RRC_InactiveNotFollowedByRRCResume); |

Before Change:

function f_TC_8_1_6_1_4_2_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / RRC Resume f_NR5GC_Init(NR_1); // @sic R5-217920, R5s211598 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false); // @sic R5-217920, R5s211598 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_6_1_4_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A_T3540, tsc_NR_RbId_SRB1); // @sic R5-217920, R5s211598, R5s240238 sic@ }

After Change:

function f_TC_8_1_6_1_4_2_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / RRC Resume f_NR5GC_Init(NR_1); // @sic R5-217920, R5s211598 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false, -, -, -, -, tsc_RRC_InactiveNotFollowedByRRCResume); // @sic R5-217920, R5s211598 sic@ //WA#WI=1367424 f_NR_TestBody_Set(true); fl_TC_8_1_6_1_4_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A_T3540, tsc_NR_RbId_SRB1); // @sic R5-217920, R5s211598, R5s240238 sic@ }

## Change to f_TC_6_4_2_2_NR5GC

| Function name | f_TC_6_4_2_2_NR5GC |
| --- | --- |
| Reason for change | Test body steps are not followed by rrcResume procedure after Preamble. i.e., Step 4 - The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. |
| Summary of change | Set p_FollowedByRRCResume as tsc_RRC_InactiveNotFollowedByRRCResume. |
| TTCN module | NR5GC\6_4_2\Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted – See MCC160 implementation as follows: f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, tsc_RRC_InactiveNotFollowedByRRCResume); |

Before Change:

function f_TC_6_4_2_2_NR5GC() runs on NR5GC_PTC { // Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state // @sic R5-201500 R5s200403 R5s201118 R5s210501 sic@ var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList; var template (value) SSB_MTC v_SSB_MTC; var boolean v_IsFR1; f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells // Table 6.4.2.2.3.3-2: SIB2 of NR Cell 6 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell6); if (v_IsFR1) { f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10); } else { f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 5); } f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5); // Table 6.4.2.2.3.3-3: SIB4 for NR Cell 1 v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[1], v_InterFreqCarrierFreqList[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 1, 4); // Table 6.4.2.2.3.3-3: SIB4 for NR Cell 3 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList[1]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, 10); v_SSB_MTC := f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell3, nr_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell3, 0, v_SSB_MTC.periodicityAndOffset); // Table 6.4.2.2.3.3-3: SIB4 for NR Cell 6 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell6, {v_InterFreqCarrierFreqList[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 0, 4); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -82); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2 f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false); f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2); // @sic R5-233462 sic@ - nr_Cell1 at T0 f_NR_TestBody_Set(true); f_TC_6_4_2_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_INACTIVE_2A); } //end of f_TC_6_4_2_2_NR5GC

After Change:

function f_TC_6_4_2_2_NR5GC() runs on NR5GC_PTC { // Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state // @sic R5-201500 R5s200403 R5s201118 R5s210501 sic@ var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList; var template (value) SSB_MTC v_SSB_MTC; var boolean v_IsFR1; f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells // Table 6.4.2.2.3.3-2: SIB2 of NR Cell 6 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell6); if (v_IsFR1) { f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10); } else { f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 5); } f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5); // Table 6.4.2.2.3.3-3: SIB4 for NR Cell 1 v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[1], v_InterFreqCarrierFreqList[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 1, 4); // Table 6.4.2.2.3.3-3: SIB4 for NR Cell 3 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList[1]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, 10); v_SSB_MTC := f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell3, nr_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell3, 0, v_SSB_MTC.periodicityAndOffset); // Table 6.4.2.2.3.3-3: SIB4 for NR Cell 6 f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell6, {v_InterFreqCarrierFreqList[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 0, 4); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -82); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2 f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false,-,-,-,-,tsc_RRC_InactiveNotFollowedByRRCResume); //WA#WI=1367424 f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2); // @sic R5-233462 sic@ - nr_Cell1 at T0 f_NR_TestBody_Set(true); f_TC_6_4_2_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_INACTIVE_2A); } //end of f_TC_6_4_2_2_NR5GC