3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250012

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3610 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE supporting pc_IP_Ping as true may fail the case at step4, as UE will not send back DRB data same as step3, It is because loopback mode will not be activated if UE establishes internet PDN at premable step. To enable loopback mode if(not pc_IP_Ping) this condition need to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed if condition to activate loopback mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS 	iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_10_1_2_1_NR5GC |
| --- | --- |
| Reason for change | UE supporting pc_IP_Ping as true may fail the case at step4, as UE will not send back DRB data same as step3, It is because loopback mode will not be activated if UE establishes internet PDN at premable step. To enable loopback mode if(not pc_IP_Ping) this condition need to be removed. |
| Summary of change | Removed if condition to activate loopback mode. |
| TTCN module | GSM_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_10_1_2_1_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Accepted / Reactivation var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_DRBId; // @sic R5s230095 sic@ var template (omit) NR_DRBInfo_Type v_DRBInfo; // @sic R5s230095 sic@ f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TESTModeB_ON); // @sic R5-198972, R5s240305 sic@ // check if internet PDU session established in preamble // @sic R5s230095 sic@ v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); f_NR5GC_Activate_TestMode (v_NGC_NASCellA, tsc_UE_TestLoopMode_TypeB, tsc_NR_RbId_SRB2); // activate - SEE CHANGE 3 } if(not pc_IP_Ping) { // @sic R5s240305 sic@ f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); } f_NR_TestBody_Set(true); // @sic R5s220796 sic@ //Main test body fl_TC_10_1_2_1_Body(); f_NR_TestBody_Set(false); // @sic R5s220796 sic@ // Finish //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NGC_NASCellA); // @sic R5s190944 sic@ //@sic R5s230130 sic@ f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change

function f_TC_10_1_2_1_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Accepted / Reactivation var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_DRBId; // @sic R5s230095 sic@ var template (omit) NR_DRBInfo_Type v_DRBInfo; // @sic R5s230095 sic@ f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TESTModeB_ON); // @sic R5-198972, R5s240305 sic@ // check if internet PDU session established in preamble // @sic R5s230095 sic@ v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); f_NR5GC_Activate_TestMode (v_NGC_NASCellA, tsc_UE_TestLoopMode_TypeB, tsc_NR_RbId_SRB2); // activate - SEE CHANGE 3 } f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); f_NR_TestBody_Set(true); // @sic R5s220796 sic@ //Main test body fl_TC_10_1_2_1_Body(); f_NR_TestBody_Set(false); // @sic R5s220796 sic@ // Finish //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NGC_NASCellA); // @sic R5s190944 sic@ //@sic R5s230130 sic@ f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }