3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250051

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4880 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EMM test case 9.2.3.1.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At Step 20A, f_EUTRA_RRC_ConnectionRelease is using Eutran CellB instead of CellA. This needs to be done on CellA which is the current serving cell |  |  |  |  |  |  |  |  |
| Summary of change: |  | Passed CellA as parameter instead of CellB in the function call f_EUTRA_RRC_ConnectionRelease. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_TC_9_2_3_1_23_Body() |
| --- | --- |
| Reason for change | At Step 20A, f_EUTRA_RRC_ConnectionRelease is using Eutran CellB instead of CellA. This needs to be done on CellA which is the current serving cell |
| Summary of change | Passed CellA as parameter instead of CellB in the function call f_EUTRA_RRC_ConnectionRelease. |
| TTCN module | NAS_TrackingArea_EOnly.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_9_2_3_1_23_Body() runs on EUTRA_PTC { var template (value) CellPowerList_Type v_CellPowerList; var NAS_MSG_Indication_Type v_NasInd; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var GutiParameters_Type v_GutiParams_B := f_EUTRA_CellInfo_GetGuti(eutra_CellB); var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus; var EPS_BearerIdentity v_EpsBearerId := f_EUTRA_InitialRegistrationEpsBearerId(firstPDN); // @sic R5s170264 sic@ var DRB_Identity v_DRB_Id := f_EUTRA_NB_EpsBearerAssociatedDRB(v_EpsBearerId); // @sic R5s170264 sic@ var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var ProcedureTransactionIdentifier v_EPS_TI := '00'O; ************************************* Code Skipped ********************************************* //@siclog "Step 20" siclog@ // The UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'? v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA, v_RRC_TI, cr_EstablishmentCause_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cdr_TAU_Request_OldGuti(tsc_EpsUpdate_Periodic, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@ v_AdditionalUpdateType, f_GutiParameters2MobileIdentity(omit, v_GutiParams), cr_TAI(v_PLMN, tsc_TAC1)))); v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; // Wait for 25s after step 20 to ensure that T3430 and T3411 expire. //@siclog "Step 20A" siclog@ t_T3430_T3411.start(v_T3430_T3411); // @sic R5-247693 sic@ f_EUTRA_RRC_ConnectionRelease_Local(eutra_CellB, f_EUTRA_GetNextSendOccasion(eutra_CellB)); // @sic R5-247693 sic@ t_T3430_T3411.timeout; // @sic R5-247693 sic@ //@siclog "Step 20B" siclog@ // @sic R5-247693 sic@ // Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'? v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA, v_RRC_TI, cr_EstablishmentCause_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cdr_TAU_Request_OldGuti(tsc_EpsUpdate_Periodic, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@ v_AdditionalUpdateType, f_GutiParameters2MobileIdentity(omit, v_GutiParams), cr_TAI(v_PLMN, tsc_TAC1)))); **************************************************************************************************************

After Change:

function fl_TC_9_2_3_1_23_Body() runs on EUTRA_PTC { var template (value) CellPowerList_Type v_CellPowerList; var NAS_MSG_Indication_Type v_NasInd; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var GutiParameters_Type v_GutiParams_B := f_EUTRA_CellInfo_GetGuti(eutra_CellB); var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus; var EPS_BearerIdentity v_EpsBearerId := f_EUTRA_InitialRegistrationEpsBearerId(firstPDN); // @sic R5s170264 sic@ var DRB_Identity v_DRB_Id := f_EUTRA_NB_EpsBearerAssociatedDRB(v_EpsBearerId); // @sic R5s170264 sic@ var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var ProcedureTransactionIdentifier v_EPS_TI := '00'O; ************************************* Code Skipped ********************************************* //@siclog "Step 20" siclog@ // The UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'? v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA, v_RRC_TI, cr_EstablishmentCause_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cdr_TAU_Request_OldGuti(tsc_EpsUpdate_Periodic, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@ v_AdditionalUpdateType, f_GutiParameters2MobileIdentity(omit, v_GutiParams), cr_TAI(v_PLMN, tsc_TAC1)))); v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; // Wait for 25s after step 20 to ensure that T3430 and T3411 expire. //@siclog "Step 20A" siclog@ t_T3430_T3411.start(v_T3430_T3411); // @sic R5-247693 sic@ f_EUTRA_RRC_ConnectionRelease_Local(eutra_CellA, f_EUTRA_GetNextSendOccasion(eutra_CellA)); // @sic R5-247693 sic@ t_T3430_T3411.timeout; // @sic R5-247693 sic@ //@siclog "Step 20B" siclog@ // @sic R5-247693 sic@ // Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'? v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA, v_RRC_TI, cr_EstablishmentCause_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cdr_TAU_Request_OldGuti(tsc_EpsUpdate_Periodic, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@ v_AdditionalUpdateType, f_GutiParameters2MobileIdentity(omit, v_GutiParams), cr_TAI(v_PLMN, tsc_TAC1)))); **************************************************************************************************************