3GPP TSG-RAN WG5 Meeting #71

Nanjing, China May 23rd-27th 2016 	R5-162097

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3848 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UTRA Absolute Priority based reselection test cases 8.3.1.52,8.3.1.53 and 8.3.1.54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  |  | Date: |  | 2016-4-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1. As per Qualcomm discussion paper R5-155067 in RAN5#69 “Setting of FGI 3 or FGI 4 bit during Absolute Priority Selection in Cell_FACH test cases “, It was concluded that, the applicability conditions of the Test Cases required a change. RAN5 requested guidance on this change from RAN2 through LS R5-155780 “LS on Applicability of FGI3/4 for UMTS test cases for Absolute Priority reselection in CELL_FACH”. Based on LS reply R5-162018 (R2-161818 from RAN2#93), RAN2 concluded that for UE’s supporting EUTRA, If the UE supports HIGH/ALL Priority layer measurements and cell reselection procedure in CELL_FACH, then setting FGI3/FGI 4 respectively to “true” be applicable for UTRA Inter-Frequency test cases. Hence, changes are required to the Definition sections and to the Related ICS/IXIT statement(s) under the Method of Test sections of TS 34.123-1 for the Test Cases 8.3.1.52,8.3.1.53 and 8.3.1.54. 2. From the test procedure of Test case 8.3.1.53, Cell 5 was incorrectly referenced as Cell 4 at step 1 of the Expected sequence 3.In 8.3.1.52 System Information Change Indication is defined twice, once in Definition and applicability and then in Method of test clause hence removed it from method of test clause |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Definition and applicability sections of the test cases 8.3.1.52,8.3.1.53 and 8.3.1.54 clarified to include - - UE state : CELL_FACH - Additional Related ICS/IXIT statement(s) added to the method of test sections of each test case 2. Cell 4 changed to Cell 5 at step 1 of Test case 8.3.1.53 3. Removed System Information Change Indication message defined twice in 8.3.1.52 4. A note has been added to 8.3.1.54 to clarify priority of Cell4 and Cell5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.52.1, 8.3.1.52.4,8.3.1.53.1, 8.3.1.53.4,8.3.1.54.1, 8.3.1.54.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 34.123-2 CR 0768 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact Revision 1 - Removed System Information Change Indication message defined twice in 8.3.1.52 |  |  |  |  |  |  |  |  |

<Start of Changed Section>

#### 8.3.1.52	Cell Update: Inter Frequency cell reselection in CELL_FACH based on absolute priority

8.3.1.52.1	Definition and applicability

All UEs which support UTRA FDD, EUTRA and Rel 8  absolute priority cell reselection for high priority layers  in CELL_FACH.

8.3.1.52.2	Conformance requirement

5.2.6.1.2a Measurement rules for inter-frequency and inter-RAT cell reselection when absolute priorities are used

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

…

If the UE has received absolute priority information for inter-frequency layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-frequency layers with a priority higher than the priority of the current serving layer.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

…

-	For inter-frequency layers with a priority equal or lower than the priority of the current serving layer:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-frequency layers of equal or lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of

inter-frequency layers of equal or lower priority.

-	The UE shall not perform measurements of inter-frequency layers for which the UE has no absolute priority.

…

5.2.6.1.4a Absolute priority based criteria for inter-frequency and inter-RAT cell reselection

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection

criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and

inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

If System Information Block 18 with UTRAN information is provided then the UE shall apply the inter-frequency priority rules below only to UTRAN cells of the selected PLMN or equivalent PLMNs. If System Information Block 18 is not provided, or is provided but contains no information for UTRAN, then the rules below apply for the entire UTRAN RAT.

…

If priority information is provided for any inter-frequency layers, cells belonging to inter-frequency layers for which no priority or no threshold is assigned shall not be considered for reselection.

…

If none of the inter-frequency layers in UTRAN are provided with both priority and threshold parameters, the cell reselection criteria in subclause 5.2.6.1.4 shall apply for inter-frequency layers in UTRAN.

…

If both criteria in subclause 5.2.6.1.4 and criteria in subclause 5.2.6.1.4a are simultaneously fulfilled, the order in which the reselection criteria are evaluated is left to UE implementation.

...

The UE may apply either priorities broadcast in system information or priorities received through dedicated signalling.

The rules regarding which set of priorities shall apply are defined in [4]. UE specific priorities are only valid in the RPLMN, and shall be cleared when the validity time of the UE specific priorities expires, or when leaving the RPLMN.

UE specific priorities are not applied in camped on any cell state.

…

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

-	Criterion 2: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a inter-frequency cell on an evaluated equal absolute priority layer is greater than Threshx,low during a time interval Treselection;

-	Criterion 3: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

-	Criterion 5: SqualServingCell < Threshserving,low2 and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

Cell reselection to an inter-frequency cell on an equal absolute priority layer to the camped frequency shall be performed if criterion 2 is fulfilled.

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

The UE should not perform cell reselection while the UE has an allocated common E-DCH resource.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are scaled according to the UE mobility state and target RAT, as specified in 5.2.6.1.1a.

8.6.7.3a	UTRA priority info list

If the IE "UTRA priority info list" is received in System Information Block Type 19, the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	store the value of the IEs "Sprioritysearch1", "Sprioritysearch2", "Threshserving,low" and "Threshserving,low2" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST;

2>	if any of the values in any of the occurrences of the  IE "Frequency List" in any occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST  is the frequency of the serving cell:

3>	store the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	otherwise:

3>	clear the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA FDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA TDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

1>	otherwise:

2>	update the PRIORITY_INFO_LIST IE "Priority status" to cause "sys_info_priority".

2>	store information in the IE "UTRA Serving Cell" in the IE "UTRA Serving Cell" within the variable PRIORITY_INFO_LIST;

2>		for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA FDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA FDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD" in that occurrence of the IE "Frequency List".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA TDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA TDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD" in that occurrence of the IE "Frequency List".

References

1.	TS 25.304, 5.2.6.1.2a [Rev. 11]

2.	TS 25.304, 5.2.6.1.4a

3.	TS 25.331, 8.6.7.3a

8.3.1.52.3	Test purpose

1. To confirm that UE does not perform higher priority cell reselection based on absolute priority in CELL_FACH state, when power levels do not meet cell reselection criteria.

2. To confirm that UE performs higher priority cell reselection based on absolute priority in CELL_FACH state, when power levels meet cell reselection criteria.

8.3.1.52.4	Method of test

Initial conditions

System Simulator:

-	2 cells Cell 1 and Cell 4

-	Cell 2 is mapped to cell 4 (inter frequency) of 34.108 clause 6.1 and both cells belong to different frequency bands.

-	Cell 1 as the active cell with SS downlink transmission power settings according to “T0” in table 8.3.1.x below.

-	SIB19 to be transmitted by both cells.

MASTER INFORMATION BLOCK

Use the same message sub-type found in clause 6.1 of TS 34.108, with the following exception.

| Information Element | Value/remark |
| --- | --- |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| SB 1 Cell Value tag | Set to (Current SIB4 value tag + 1) |

SCHEDULING BLOCK 1

| Information Element | Value/remark |
| --- | --- |
| SIB 11 Cell Value Tag | Set to (Current SIB 11 value tag + 1) |

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

User Equipment:

-	UE is brought to CELL_FACH state on Cell 1 with "T0" in table 8.3.1.x below.

-	UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statement(s)

-	UE supports UTRA FDD yes/no

- UE supports EUTRA yes/no

- UE supports FGI3 (UTRA CELL_FACH absolute priority cell reselection for high priority layers)  yes/no

Test procedure

Table 8.3.1.52

| Parameter | Unit | Cell 1 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 |
| UTRA RF Channel Number |  | Mid Range Test Frequency |  |  | High Range Test Frequency |  |  |
| CPICH Ec | dBm/3.84MHz | -60 | -60 | -60 | -90 | -77 | -65 |
| Srxlev* | dB | 21 | 21 | 21 | -9 | 4 | 16 |
| qRxLevMinFDD | dB | -81 |  |  | -81 |  |  |
| Note-Srxlev is calculated in UE. During T1, UE does not perform cell reselection to High priority cell 4. During T2, UE performs cell reselection to High priority cell 4. |  |  |  |  |  |  |  |

SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 4 as the higher priority cell and cell 1 as the lower priority cell (See specific message contents). SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification. SS waits for 5s to ensure UE reads all the SIB information.

SS configures downlink transmission power settings according to columns "T1" in the table 8.3.1.x. UE does not reselect to higher priority cell 4 as power levels of cell 4 do not meet the absolute priority cell reselection criteria defined in SIB TYPE 19. UE should not be sending a CELL UPDATE in this case on cell 4.

SS configures the downlink transmission power settings according to columns “T2” from table 8.3.1.x. UE performs cell reselection to higher priority cell 4 as power levels of cell 4 now meet the absolute priority cell reselection criteria defined in SIB TYPE 19. UE sends a CELL UPDATE with cause: cell reselection. SS responds with CELL UPDATE CONFIRM. SS releases the RRC connection.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | SYSTEM INFORMATION BLOCK TYPE 19 | SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 4 as the higher priority cell and cell 1 as the lower priority cell (See specific message contents). |
| 2 |  |  | SYSTEM INFORMATION CHANGE INDICATION | SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification (See specific message contents) and waits for 5s for UE to read all SIB information. |
| 3 | SS |  |  | SS configures its downlink transmission power settings according to columns "T1" in table 8.3.1.52. |
| 4 | UE |  |  | UE does not perform cell reselection to High Priority cell 4 as absolute priority cell reselection criterion is not satisfied. |
| 5 | SS |  |  | SS configures its downlink transmission power settings according to columns "T2" in table 8.3.1.52. |
| 6 |  |  | CELL UPDATE | UE performs cell reselection to Higher Priority cell 4 and sends CELL UPDATE on the new serving cell 4 and value "cell reselection" shall be indicated in IE "Cell update cause" |
| 7 |  |  | CELL UPDATE CONFIRM |  |
| 7a |  |  | UTRAN MOBILITY INFORMATION CONFIRM |  |
| 8 |  |  | RRC CONNECTION RELEASE | SS releases the RRC connection. |
| 9 |  |  | RRC CONNECTION RELEASE COMPLETE |  |

Specific Message Contents

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to TS 34.108 clause 6.1.0b, with the following exceptions:

System Information Block type19 (step 1)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList |  |  |
| utra-ServingCell |  |  |
| Priority | 4 |  |
| s-PrioritySearch1 | 8 (16 dB) |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | 10 (20 dB) |  |
| utran-FDD-FrequencyList |  |  |
| Uarfcn | Same downlink uarfcn as used for cell 4 |  |
| Priority | 5 |  |
| threshXhigh | 5 (10 dB) |  |
| threshXlow | 1 (2 dB) |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | -81 dBm |  |
| utran-TDD-FrequencyList | Not present |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  |
| nonCriticalExtensions | Present |  |
| SysInfoType19-vb30ext |  |  |
| cell-fach-meas-ind | high-priority-layers |  |

System Information Change Indication (Step 2)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Paging record list | Not Present |
| BCCH modification info |  |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| BCCH Modification time | Not Present |

CELL UPDATE CONFIRM (Step 7)

| Information Element | Value/remark |
| --- | --- |
| New C-RNTI | '1010 1010 1010 1010' |

8.3.1.52.5	Test requirements

1. At step 4, UE does not perform cell reselection to high priority cell 4, as cell reselection criterion is not satisfied.

2. At step 6, UE reselects to higher priority cell 4, as cell reselection criteria matches and sends a CELL UPDATE with cause: cell reselection.

#### 8.3.1.53	Cell Update: Absolute priority based cell reselection failure to inter-frequency cell for which no priority or no threshold is assigned

8.3.1.53.1	Definition and applicability

All UEs which support UTRA FDD, EUTRA and Rel 8 absolute priority cell reselection for high priority layers in CELL_FACH.

8.3.1.53.2	Conformance requirement

5.2.6.1.2a Measurement rules for inter-frequency and inter-RAT cell reselection when absolute priorities are used

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

…

If the UE has received absolute priority information for inter-frequency layers, the UE shall follow these rules:

- The UE shall perform measurements of inter-frequency layers with a priority higher than the priority of the

current serving layer.

NOTE: The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are

above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

…

- For inter-frequency layers with a priority equal or lower than the priority of the current serving layer:

- If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform

measurements of inter-frequency layers of equal or lower priority.

- If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of

inter-frequency layers of equal or lower priority.

- The UE shall not perform measurements of inter-frequency layers for which the UE has no absolute priority.

…

5.2.6.1.4a Absolute priority based criteria for inter-frequency and inter-RAT cell reselection

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection

criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and

inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

If System Information Block 18 with UTRAN information is provided then the UE shall apply the inter-frequency

priority rules below only to UTRAN cells of the selected PLMN or equivalent PLMNs. If System Information Block 18

is not provided, or is provided but contains no information for UTRAN, then the rules below apply for the entire

UTRAN RAT.

…

If priority information is provided for any inter-frequency layers, cells belonging to inter-frequency layers for which no

priority or no threshold is assigned shall not be considered for reselection.

…

If none of the inter-frequency layers in UTRAN are provided with both priority and threshold parameters, the cell

reselection criteria in subclause 5.2.6.1.4 shall apply for inter-frequency layers in UTRAN.

…

If both criteria in subclause 5.2.6.1.4 and criteria in subclause 5.2.6.1.4a are simultaneously fulfilled, the order in which

the reselection criteria are evaluated is left to UE implementation.

...

The UE may apply either priorities broadcast in system information or priorities received through dedicated signalling.

The rules regarding which set of priorities shall apply are defined in [4]. UE specific priorities are only valid in the

RPLMN, and shall be cleared when the validity time of the UE specific priorities expires, or when leaving the RPLMN.

UE specific priorities are not applied in camped on any cell state.

…

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

- Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than

Threshx,high during a time interval Treselection;

- Criterion 2: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a inter-frequency

cell on an evaluated equal absolute priority layer is greater than Threshx,low during a time interval Treselection;

- Criterion 3: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a cell on an

evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

- Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than

Threshx,high2 during a time interval Treselection;

- Criterion 5: SqualServingCell < Threshserving,low2 and the SqualnonServingCell,x of a cell on an evaluated lower absolute

priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1

or 4 is fulfilled.

Cell reselection to an inter-frequency cell on an equal absolute priority layer to the camped frequency shall be

performed if criterion 2 is fulfilled.

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3

or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among

the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current

serving cell.

The UE should not perform cell reselection while the UE has an allocated common E-DCH resource.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in

SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if

provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are

scaled according to the UE mobility state and target RAT, as specified in 5.2.6.1.1a.

8.6.7.3a	UTRA priority info list

If the IE "UTRA priority info list" is received in System Information Block Type 19, the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	store the value of the IEs "Sprioritysearch1", "Sprioritysearch2", "Threshserving,low" and "Threshserving,low2" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST;

2>	if any of the values in any of the occurrences of the  IE "Frequency List" in any occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST  is the frequency of the serving cell:

3>	store the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	otherwise:

3>	clear the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA FDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA TDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

1>	otherwise:

2>	update the PRIORITY_INFO_LIST IE "Priority status" to cause "sys_info_priority".

2>	store information in the IE "UTRA Serving Cell" in the IE "UTRA Serving Cell" within the variable PRIORITY_INFO_LIST;

2>		for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA FDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA FDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD" in that occurrence of the IE "Frequency List".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA TDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA TDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD" in that occurrence of the IE "Frequency List".

References

1.	TS 25.304, 5.2.6.1.2a [Rel. 11]

2.	TS 25.304, 5.2.6.1.4a

3.	TS 25.331, 8.6.7.3a

8.3.1.53.3	Test purpose

1. To confirm that if priority information is provided for any inter-frequency layers, cells belonging to inter-frequency layers for which no priority or no threshold is assigned shall not be considered for reselection.

2. To confirm that UE performs higher priority cell reselection based on absolute priority in CELL_FACH state, when priority is assigned to the cell.

8.3.1.53.4	Method of test

Initial conditions

System Simulator:

2 cells Cell 1 and Cell 4

Cell5 is a dummy cell referenced in SIB-19

Cell 1 is the active cell with SS downlink transmission power settings according to “T0” in table 8.3.1.53 below.

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

User Equipment:

UE supports FDD and Rel-8 RAT priority cell reselection.

UE is brought to CELL_FACH state on Cell 1 with "T0" in table 8.3.1.53 below.

UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statement(s)

- UE supports UTRA FDD yes/no.

- UE supports EUTRA yes/no

- UE supports FGI3 (UTRA CELL_FACH absolute priority cell reselection for high priority layers)  yes/no

Test Procedure

Table 8.3.1.53

| Parameter | Unit | Cell 1 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T0 | T1 |
| UTRA RF Channel Number |  | Mid Range Test Frequency |  | High Range Test Frequency |  |
| CPICH Ec | dBm/3.84MHz | -60 | -60 | -90 | -65 |
| Srxlev* | dB | 21 | 21 | -9 | 16 |
| qRxLevMinFDD | dB | -81 |  | -81 |  |
| Note-Srxlev is calculated in UE. During T1, Cell 4 meets the criteria for higher priority cell reselection |  |  |  |  |  |

SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 5(dummy cell which does not exist) as the higher priority cell and cell 1 as the lower priority cell (See specific message contents). SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification. SS waits for 5s to ensure UE reads all the SIB information.

SS configures downlink transmission power settings according to columns "T1" in the table 8.3.1.53. UE does not reselect to cell 4 although it fulfils the cell reselection criteria, as no priority is assigned to this cell in SIB19. UE should not be sending a CELL UPDATE in this case on cell 4.

SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 4 as the higher priority cell and cell 1 as the lower priority cell (See specific message contents). SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification. SS waits for 5s to ensure UE reads all the SIB information.

UE performs cell reselection to higher priority cell 4 since priority is now assigned to this cell in SIB TYPE 19. UE sends a CELL UPDATE with cause: cell reselection. SS responds with CELL UPDATE CONFIRM. SS releases the RRC connection.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | SYSTEM INFORMATION BLOCK TYPE 19 | SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 5 as the higher priority cell and cell 1 as the lower priority cell (See specific message contents). |
| 2 |  |  | SYSTEM INFORMATION CHANGE INDICATION | SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification (See specific message contents) and waits for 5s for UE to read all SIB information. |
| 3 | SS |  |  | SS configures its downlink transmission power settings according to columns "T1" in table 8.3.1.53. |
| 4 | UE |  |  | UE does not perform cell reselection to cell 4 although it fulfils the higher priority cell reselection criteria, since no priority is assigned to this cell in SIB TYPE 19. |
| 5 |  |  | SYSTEM INFORMATION BLOCK TYPE 19 | SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 4 as the higher priority cell and cell 1 as the lower priority cell (See specific message contents). |
| 6 |  |  | SYSTEM INFORMATION CHANGE INDICATION | SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification (See specific message contents) and waits for 5s for UE to read all SIB information. |
| 7 |  |  | CELL UPDATE | UE performs cell reselection to Higher Priority cell 4 and sends CELL UPDATE on the new serving cell 4 and value "cell reselection" shall be indicated in IE "Cell update cause" |
| 8 |  |  | CELL UPDATE CONFIRM |  |
| 8a |  |  | UTRAN MOBILITY INFORMATION CONFIRM |  |
| 9 |  |  | RRC CONNECTION RELEASE | SS releases the RRC connection. |
| 10 |  |  | RRC CONNECTION RELEASE COMPLETE |  |

Specific Message Contents

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to

TS 34.108 clause 6.1.0b, with the following exceptions:

SYSTEM INFORMATION BLOCK TYPE 19 (Step 1)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList { |  |  |
| utra-ServingCell { |  |  |
| Priority | 4 |  |
| s-PrioritySearch1 | 8 (16 dB) |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | 10 (20 dB) |  |
| } |  |  |
| utran-FDD-FrequencyList { |  |  |
| uarfcn | Same downlink uarfcn as used for cell 5 |  |
| priority | 5 |  |
| threshXhigh | 5 (10 dB) |  |
| threshXlow | 1 (2 dB) |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | -81 dBm |  |
| utran-TDD-FrequencyList | Not present |  |
| } |  |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  |
| nonCriticalExtensions | Present |  |
| SysInfoType19-vb30ext |  |  |
| cell-fach-meas-ind | high-priority-layers |  |

SYSTEM INFORMATION BLOCK TYPE 19 (Step 5)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList { |  |  |
| utra-ServingCell { |  |  |
| Priority | 4 |  |
| s-PrioritySearch1 | 8 (16 dB) |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | 10 (20 dB) |  |
| } |  |  |
| utran-FDD-FrequencyList |  |  |
| uarfcn | Same downlink uarfcn as used for cell 4 |  |
| priority | 5 |  |
| threshXhigh | 5 (10 dB) |  |
| threshXlow | 1 (2 dB) |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | -81 dBm |  |
| utran-TDD-FrequencyList | Not present |  |
| } |  |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  |
| nonCriticalExtensions | Present |  |
| SysInfoType19-vb30ext |  |  |
| cell-fach-meas-ind | high-priority-layers |  |

SYSTEM INFORMATION CHANGE INDICATION (Step 2, Step 6)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Paging record list | Not Present |
| BCCH modification info |  |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| BCCH Modification time | Not Present |

CELL UPDATE CONFIRM (Step 8)

| Information Element | Value/remark |
| --- | --- |
| New C-RNTI | '1010 1010 1010 1010' |

8.3.1.53.5	Test requirements

1) At step 4, UE does not perform cell reselection to cell 4, as no priority is assigned to this cell in SIB TYPE 19 at step 1.

2) At step 7, UE reselects to higher priority cell 4, as priority is now assigned to this cell and sends a CELL UPDATE with cause: cell reselection.

#### 8.3.1.54	Cell Update: Absolute priority based cell reselection when more than one cell meets the criterion

8.3.1.54.1	Definition and applicability

All UEs which support UTRA FDD ,EUTRA and Rel 8 absolute priority cell reselection for high priority layers in CELL_FACH.

8.3.1.54.2	Conformance requirement

5.2.6.1.2a	Measurement rules for inter-frequency and inter-RAT cell reselection when   absolute priorities are used

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states. In CELL_FACH state, if (High Priority Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4] and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2) or (All Layers is indicated in CELL_FACH Absolute Priority Measurement Indicator [4]) then the measurement rules below apply, otherwise the measurement rules in 5.2.6.1.1 and 5.2.6.1.2 apply.

…

If the UE has received absolute priority information for inter-frequency layers, the UE shall follow these rules:

- The UE shall perform measurements of inter-frequency layers with a priority higher than the priority of the

current serving layer.

NOTE: The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are

above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

…

- For inter-frequency layers with a priority equal or lower than the priority of the current serving layer:

- If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform

measurements of inter-frequency layers of equal or lower priority.

- If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of

inter-frequency layers of equal or lower priority.

- The UE shall not perform measurements of inter-frequency layers for which the UE has no absolute priority.

…

5.2.6.1.4a Absolute priority based criteria for inter-frequency and inter-RAT cell reselection

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection

criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and

inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

If System Information Block 18 with UTRAN information is provided then the UE shall apply the inter-frequency

priority rules below only to UTRAN cells of the selected PLMN or equivalent PLMNs. If System Information Block 18

is not provided, or is provided but contains no information for UTRAN, then the rules below apply for the entire

UTRAN RAT.

…

If priority information is provided for any inter-frequency layers, cells belonging to inter-frequency layers for which no

priority or no threshold is assigned shall not be considered for reselection.

…

If none of the inter-frequency layers in UTRAN are provided with both priority and threshold parameters, the cell

reselection criteria in subclause 5.2.6.1.4 shall apply for inter-frequency layers in UTRAN.

…

If both criteria in subclause 5.2.6.1.4 and criteria in subclause 5.2.6.1.4a are simultaneously fulfilled, the order in which

the reselection criteria are evaluated is left to UE implementation.

...

The UE may apply either priorities broadcast in system information or priorities received through dedicated signalling.

The rules regarding which set of priorities shall apply are defined in [4]. UE specific priorities are only valid in the

RPLMN, and shall be cleared when the validity time of the UE specific priorities expires, or when leaving the RPLMN.

UE specific priorities are not applied in camped on any cell state.

…

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

- Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than

Threshx,high during a time interval Treselection;

- Criterion 2: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a inter-frequency

cell on an evaluated equal absolute priority layer is greater than Threshx,low during a time interval Treselection;

- Criterion 3: SrxlevServingCell < Threshserving,low or SqualServingCell < 0 and the SrxlevnonServingCell,x of a cell on an

evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

- Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than

Threshx,high2 during a time interval Treselection;

- Criterion 5: SqualServingCell < Threshserving,low2 and the SqualnonServingCell,x of a cell on an evaluated lower absolute

priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1

or 4 is fulfilled.

Cell reselection to an inter-frequency cell on an equal absolute priority layer to the camped frequency shall be

performed if criterion 2 is fulfilled.

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3

or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among

the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current

serving cell.

The UE should not perform cell reselection while the UE has an allocated common E-DCH resource.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in

SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if

provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are

scaled according to the UE mobility state and target RAT, as specified in 5.2.6.1.1a.

8.6.7.3a	UTRA priority info list

If the IE "UTRA priority info list" is received in System Information Block Type 19, the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	store the value of the IEs "Sprioritysearch1", "Sprioritysearch2", "Threshserving,low" and "Threshserving,low2" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST;

2>	if any of the values in any of the occurrences of the  IE "Frequency List" in any occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST  is the frequency of the serving cell:

3>	store the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	otherwise:

3>	clear the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA FDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA TDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

1>	otherwise:

2>	update the PRIORITY_INFO_LIST IE "Priority status" to cause "sys_info_priority".

2>	store information in the IE "UTRA Serving Cell" in the IE "UTRA Serving Cell" within the variable PRIORITY_INFO_LIST;

2>		for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA FDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA FDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD" in that occurrence of the IE "Frequency List".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA TDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA TDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD" in that occurrence of the IE "Frequency List".

References

1.	TS 25.304, 5.2.6.1.2a [Rel. 11]

2.	TS 25.304, 5.2.6.1.4a

3.	TS 25.331, 8.6.7.3a

8.3.1.54.3	Test purpose

To confirm that if more than one cell meets the absolute priority based cell reselection criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

8.3.1.54.4	Method of test

Initial conditions

System Simulator:

- 3 cells Cell 1, Cell 4 and Cell 5

- Cell 1 is the active cell with SS downlink transmission power settings according to “T0” in table 8.3.1.54 below.

- All cells to be configured to transmit SIB 19 with default contents as in 34.108.

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

User Equipment:

- UE supports FDD and Rel-8 RAT priority cell reselection.

- UE is brought to CELL_FACH state on Cell 1 with "T0" in table 8.3.1.54 below.

- UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statement(s)

- UE supports UTRA FDD.

- UE supports EUTRA yes/no

- UE supports FGI3 (UTRA CELL_FACH absolute priority cell reselection for high priority layers)  yes/no

Test Procedure

Table 8.3.1.54

| Parameter | Unit | Cell 1 |  | Cell 4 |  | Cell 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T0 | T1 | T0 | T1 |
| UTRA RF Channel Number |  | Mid Range Test Frequency |  | High Range Test Frequency |  | High Range Test Frequency |  |
| CPICH Ec | dBm/3.84MHz | -60 | -60 | -90 | -65 | -90 | -70 |
| Srxlev* | dB | 21 | 21 | -9 | 16 | -9 | 11 |
| qRxLevMinFDD | dB | -81 |  | -81 |  | -81 |  |
| Note-Srxlev is calculated in UE. During T1, cell 4 and cell 5 meet the higher priority cell reselection criterion. |  |  |  |  |  |  |  |

SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 4 and cell 5 as the higher priority cells and cell 1 as the lower priority cell (See specific message contents). SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification. SS waits for 5s to ensure UE reads all the SIB information.

SS configures the downlink transmission power settings according to columns “T1” from table 8.3.1.x. UE now performs cell reselection to  cell 4 as Srxlev of cell 4  is higher than that of cell 5 even though both the cells fulfil the absolute priority cell reselection criteria defined in SIB TYPE 19. UE sends a CELL UPDATE with cause: cell reselection. SS responds with CELL UPDATE CONFIRM. SS releases the RRC connection.

Note: Priority of a frequency layer is defined per UTRA frequency, hence, priority value of Cell 4 and Cell 5 shall be similar

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | SYSTEM INFORMATION BLOCK TYPE 19 | SS transmits MIB with the "MIB Value Tag" IE different from the original setting and the SIB TYPE 19 with cell 4 and cell 5 as the higher priority cells and cell 1 as the lower priority cell (See specific message contents). |
| 2 |  |  | SYSTEM INFORMATION CHANGE INDICATION | SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification (See specific message contents) and waits for 5s for UE to read all SIB information. |
| 3 | SS |  |  | SS configures its downlink transmission power settings according to columns "T1" in table 8.3.1.54. |
| 4 |  |  | CELL UPDATE | UE performs cell reselection to Higher Priority cell 4 and sends CELL UPDATE on the new serving cell 4 and value "cell reselection" shall be indicated in IE "Cell updatecause" |
| 5 |  |  | CELL UPDATE CONFIRM |  |
| 5a |  |  | UTRAN MOBILITY INFORMATION CONFIRM |  |
| 6 |  |  | RRC CONNECTION RELEASE | SS releases the RRC connection. |
| 7 |  |  | RRC CONNECTION RELEASE COMPLETE |  |

Specific Message Contents

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to

TS 34.108 clause 6.1.0b, with the following exceptions:

SYSTEM INFORMATION BLOCK TYPE 19 (step 1)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList { |  |  |
| utra-ServingCell { |  |  |
| Priority | 4 |  |
| s-PrioritySearch1 | 8 (16 dB) |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | 10 (20 dB) |  |
| } |  |  |
| utran-FDD-FrequencyList |  |  |
| uarfcn | Same downlink uarfcn as used for cell 4 |  |
| priority | 5 |  |
| threshXhigh | 5 (10 dB) |  |
| threshXlow | 1 (2 dB) |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | -81 dBm |  |
| utran-TDD-FrequencyList | Not present |  |
| } |  |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  |
| nonCriticalExtensions | Present |  |
| SysInfoType19-vb30ext |  |  |
| cell-fach-meas-ind | high-priority-layers |  |

SYSTEM INFORMATION CHANGE INDICATION (Step 2)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Paging record list | Not Present |
| BCCH modification info |  |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| BCCH Modification time | Not Present |

CELL UPDATE CONFIRM (Step 5)

| Information Element | Value/remark |
| --- | --- |
| New C-RNTI | '1010 1010 1010 1010' |

8.3.1.54.5 Test requirements

1) At step 4, UE reselects to higher priority cell 4, and sends a CELL UPDATE with cause: cell reselection.

<End of Changed Section>