3GPP TSG-RAN WG5 Meeting #71

Nanjing, China May 23rd-27th 2016	R5-162098

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3849 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UTRA Absolute Priority based reselection test cases 8.3.1.1d,8.3.1.1e and 8.3.1.1f |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  |  | Date: |  | 2016-04-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1.As per Qualcomm discussion paper R5-155067 in RAN5#69 “Setting of FGI 3 or FGI 4 bit during Absolute Priority Selection in Cell_FACH test cases “, It was concluded that, the applicability conditions of the Test Cases required a change. RAN5 requested guidance on this change from RAN2 through LS R5-155780 “LS on Applicability of FGI3/4 for UMTS test cases for Absolute Priority reselection in CELL_FACH”. Based on LS reply R5-162018 (R2-161818 from RAN2#93), RAN2 concluded that for UE’s supporting EUTRA, If the UE supports HIGH/ALL Priority layer measurements and cell reselection procedure in CELL_FACH, then setting FGI3/FGI 4 respectively to “true” be applicable for UTRA Inter-Frequency test cases Hence, changes are required to the Definition sections and to the Related ICS/IXIT statement(s) under the Method of Test sections of TS 34.123-1 for the Test Cases 8.3.1.1d,8.3.1.1e and 8.3.1.1f 2.Also for test case 8.3.1.1e, If an inter-frequency layer with a priority higher than the priority of the current serving layer, and another inter-frequency layer with a priority lower than the priority of the current serving layer are of the same frequency, then the UE would be unable to decide which inter-frequency layer has the higher priority. Hence based on TS 25.304 5.2.6.1.2a, the UE will be unable to perform measurements of the inter-frequency layer with higher a priority “ If the UE has received absolute priority information for inter-frequency layers, the UE shall follow these rules: - The UE shall perform measurements of inter-frequency layers with a priority higher than the priority of the current serving layer ” |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Definition and applicability sections of the test cases 8.3.1.1d,8.3.1.1e and 8.3.1.1f clarified to include - - UE state : CELL_FACH - Additional Related ICS/IXIT statement(s) added to the method of test sections of each test case 2. Changed the System Simulator UTRA RF Channel Number of Cell 7(previously Cell 5) From High to Low Range Test frequency in the method of Test for test case 8.3.1.1e as per TS 34.108 Clause 6.1. 3. Since the priorities are defined per UTRA frequency. The System Simulator UTRA RF Channel Number of Cell 7 is changed From High to Low Range Test frequency in the method of Test for test case 8.3.1.1d as per TS 34.108 Clause 6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1d.1, 8.3.1.1d.4,,8.3.1.1e.1, 8.3.1.1e.4, 8.3.1.1f.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 34.123-2 CR ... 0769 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact Revision1 – Updated Cell5 to be mapped to Cell 7 instead in TC 8.3.1.1.d and 8.3.1.1.e |  |  |  |  |  |  |  |  |

<Start of changed Section>

#### 8.3.1.1d	Inter-frequency absolute priority based reselection in CELL_FACH (Lower Priority)

8.3.1.1d.1	Definition

This test case verifies reselection to the cells on different frequency bands and requires UE to support Priority Based Reselection in CELL_FACH.

8.3.1.1d.2	Conformance requirement

…

If the UE has received absolute priority information for inter-frequency layers, the UE shall follow these rules:

…

-	For inter-frequency layers with a priority equal or lower than the priority of the current serving layer:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-frequency layers of equal or lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-frequency layers of equal or lower priority.

-	The UE may choose not to perform measurements of inter-frequency layers for which the UE has no absolute priority.

…

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

…

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

-	Criterion 2: (SrxlevServingCell < Threshserving,low or SqualServingCell < = Threshserving,low2) and the SrxlevnonServingCell,x of a inter-frequency cell on an evaluated equal absolute priority layer is greater than Threshx,low during a time interval Treselection;

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= Threshserving,low2) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

…

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

…

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

…

The UE should not perform cell reselection while the UE has an allocated common E-DCH resource.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are scaled according to the UE mobility state, as specified in 5.2.6.1.1a.

…

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" , "EUTRA-RSRQ-offsetWB" and "EUTRA-RSRQOnAllSymbolsOffset" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

- act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

….

If the IE "UTRA priority info list" is received in System Information Block Type 19, the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	store the value of the IEs "Sprioritysearch1", "Sprioritysearch2", "Threshserving,low" and "Threshserving,low2" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST;

2>	if any of the values in any of the occurrences of the  IE "Frequency List" in any occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST  is the frequency of the serving cell:

3>	store the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	otherwise:

3>	clear the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA FDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA TDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

1>	otherwise:

2>	update the PRIORITY_INFO_LIST IE "Priority status" to cause "sys_info_priority".

2>	store information in the IE "UTRA Serving Cell" in the IE "UTRA Serving Cell" within the variable PRIORITY_INFO_LIST;

2>		for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA FDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

….

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA FDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD" in that occurrence of the IE "Frequency List".

…

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA TDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA TDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD" in that occurrence of the IE "Frequency List".

References

1.	TS 25.304, clause 5.2.6.1.2a, 5.2.6.1.4a

2.	TS 25.331, clause 8.1.1.6.19, 8.6.7.3a

8.3.1.1d.3	Test purpose

1.	Verify that for a UE signalling both FGI 3 and FGI 4, when Inter-Frequency cells with high , equal or low priority are present and SIB19 Cell_FACH Absolute Priority Measurement Indicator is set to “ALL Layers”, the UE while in Cell_FACH state performs inter-frequency measurements and reselects to a  lower priority Inter-frequency cells only when corresponding cell re-selection criteria are met.

8.3.1.1d.4	Method of test

Initial conditions

System Simulator:

-	3 cells Cell 1, Cell 4 and Cell 7. Cell1 belongs to a different frequency band than Cell 4 and Cell 7.

-	Cell 1 as the active cell with SS downlink transmission power settings according to “T0” in table 8.3.1.1d below.

-	SIB19 to be transmitted by Cell1 with neighbour configuration mentioned below.

User Equipment:

-	UE is brought to CELL_FACH state on Cell 1 with “T0” in table 8.3.1.1d below.

-	UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statement(s)

- UE supports UTRA FDD yes/no

- UE supports EUTRA yes/no

- UE supports FGI4 (UTRA CELL_FACH absolute priority cell reselection for high priority layers)  yes/no

NOTE: UE’s that indicate support of EUTRA Feature group Indicator FGI4 also indicate support of EUTRA Feature group Indicator FGI3

Table 8.3.1.1d

| Parameter | Unit | Cell 1 |  |  | Cell 4 |  |  | Cell 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 | T0 | T1 | T2 |
| UTRA RF Channel Number |  | Mid Range Test Frequency |  |  | High Range Test Frequency |  |  | Low Range Test Frequency |  |  |
| CPICH Ec (FDD) | dBm/3.84MHz | -60 | -65 | -80 | -75 | -75 | -70 | -90 | -75 | -65 |
| Srxlev* | dB | 21 | 16 | 1 | 6 | 6 | 11 | -9 | 6 | 16 |
| qRxLevMinFDD | dB | -81 |  |  | -81 |  |  | -81 |  |  |
| Note: -Srxlev is calculated in UE. During T1, UE should not meet the higher nor lower priority reselection criteria During T2, UE should meet lower priority cell reselection criteria, and should not meet the higher priority reselection criteria |  |  |  |  |  |  |  |  |  |  |

Test procedure

Method C is applied.

a)	SS transmits MIB with the “MIB value tag” IE different from the original setting and the SIB TYPE 19 with cell 7 as the higher priority cell and cell 4 as the lower priority cell

b)	SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification and SS waits for 5s to ensure UE reads all the SIB information.

c)	SS configures downlink transmission power settings according to columns “T1” in the table 8.3.1.1d

d)	UE does not reselect to cell 7 or Cell 4 as power levels do not meet the absolute priority cell reselection criteria defined in SIB19.

e)	SS monitors that the UE doesn’t send CELL UPDATE on cell 4 or cell 7

f)	SS configures the downlink transmission power settings according to columns “T2” from table 8.3.1.1d.

g)	SS waits for “Treselection” time configured in SIB3 and UE should perform reselection to lower priority cell 4 as the absolute priority reselection criteria is met now and UE should not reselect to higher priority cell 7 as the absolute priority reselection criteria is not met

h)	UE sends a CELL UPDATE with cause – CELL RESELECTION on cell 4 and SS responds with CELL UPDATE CONFIRM.

Specific Message Contents

MASTER INFORMATION BLOCK

Use the same message sub-type found in clause 6.1 of TS 34.108, with the following exception.

| Information Element | Value/remark |
| --- | --- |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| SB 1 Cell Value tag | Set to (Current SIB4 value tag + 1) |

SCHEDULING BLOCK 1

| Information Element | Value/remark |
| --- | --- |
| SIB 11 Cell Value Tag | Set to (Current SIB 11 value tag + 1) |

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to TS 34.108 clause 6.1.0b, with the following exceptions:

System Information Block type 19 (Step a)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList { |  |  |
| utra-ServingCell |  |  |
| Priority | [4] |  |
| s-PrioritySearch1 | [8 (16 dB)] |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | [4 (8 dB)] |  |
| threshServingLow2 | Not Present |  |
| utran-FDD-FrequencyList |  |  |
| uarfcn | Same downlink uarfcn as used for cell 7 |  |
| priority | [5] |  |
| threshXhigh | [10 (20 dB)] |  |
| threshXlow | [5 (10 dB)] |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | [-41(81 dBm)] |  |
| uarfcn | Same downlink uarfcn as used for cell 4 |  |
| priority | [3] |  |
| threshXhigh | [10 (20 dB)] |  |
| threshXlow | [5 (10 dB)] |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | [-41(81 dBm)] |  |
| utran-TDD-FrequencyList | Not present |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList (SIZE (1..maxNumEUTRAFreqs) | Not present |  |
| cell-fach-meas-ind | All-priority-layers |  |

PAGING TYPE 1 (Step b and Step g)

| Information Element | Value/Remark |
| --- | --- |
| PagingType1 |  |
| pagingRecordList | Not present |
| bcch-ModificationInfo |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |
| bcch-ModificationTime | Not present |
| laterNonCriticalExtensions SEQUENCE {} | Not present |

8.3.1.1d.5	Test requirements

1)	At step d), UE does not reselect to a higher priority Cell 7 nor to a lower priority cell 4

2)	At step h), UE reselects to lower priority Cell 4.

#### 8.3.1.1e	Inter-frequency absolute priority based reselection in CELL_FACH (Higher Priority)

8.3.1.1e.1	Definition

This test case verifies reselection to the cells on different frequency bands and requires UE to support Priority Based Reselection in CELL_FACH.

8.3.1.1e.2	Conformance requirement

If the UE has received absolute priority information for inter-frequency layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-frequency layers with a priority higher than the priority of the current serving layer.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

…

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

…

If priority information is provided for any inter-frequency layers, cells belonging to inter-frequency layers for which no priority or no threshold is assigned shall not be considered for reselection.

…

If none of the inter-frequency layers in UTRAN are provided with both priority and threshold parameters, the cell reselection criteria in subclause 5.2.6.1.4 shall apply for inter-frequency layers in UTRAN.

…

If both criteria in subclause 5.2.6.1.4 and criteria in subclause 5.2.6.1.4a are simultaneously fulfilled, the order in which the reselection criteria are evaluated is left to UE implementation.

…

The UE may apply either priorities broadcast in system information or priorities received through dedicated signalling. The rules regarding which set of priorities shall apply are defined in [4]. UE specific priorities shall be cleared when the validity time of the UE specific priorities expires, or when a PLMN selection is performed on request by NAS. UE specific priorities are not applied in camped on any cell state.

…

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

…

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

…

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

…

The UE should not perform cell reselection while the UE has an allocated common E-DCH resource.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are scaled according to the UE mobility state, as specified in 5.2.6.1.1a.

…

If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE should store all relevant IEs included in this system information block. If the cell is not operating in MBSFN mode according to subclause 8.1.1.6.3 the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	clear every stored value of "Threshx, high", "Threshx, low", "Threshx, high2", "Threshx, low2" , "EUTRA-RSRQ-offsetWB" and "EUTRA-RSRQOnAllSymbolsOffset" in every occurrence of the IE "Priority Info List" in the variable PRIORITY_INFO_LIST.

1>	otherwise:

2>	clear the variable PRIORITY_INFO_LIST.

1>	clear the variable EUTRA_FREQUENCY_INFO_LIST;

1>	act upon the received IE "UTRA priority info list" as described in subclause 8.6.7.3a;

….

If the IE "UTRA priority info list" is received in System Information Block Type 19, the UE shall:

1>	if the value of the IE "Priority status" in the variable PRIORITY_INFO_LIST equals "dedicated_priority":

2>	store the value of the IEs "Sprioritysearch1", "Sprioritysearch2", "Threshserving,low" and "Threshserving,low2" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST;

2>	if any of the values in any of the occurrences of the  IE "Frequency List" in any occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST  is the frequency of the serving cell:

3>	store the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	otherwise:

3>	clear the value of the IE "priority" in the IE "UTRA Serving Cell" in the variable PRIORITY_INFO_LIST.

2>	for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA FDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if the value of the IE "UARFCN" exists in the list of UTRA TDD priorities in the PRIORITY_INFO_LIST then, in the occurrence of IE "Frequency List" within the variable PRIORITY_INFO_LIST with that value of the IE "UARFCN":

4>	store the value of the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

1>	otherwise:

2>	update the PRIORITY_INFO_LIST IE "Priority status" to cause "sys_info_priority".

2>	store information in the IE "UTRA Serving Cell" in the IE "UTRA Serving Cell" within the variable PRIORITY_INFO_LIST;

2>		for each occurrence of the IE "UTRAN FDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA FDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD".

….

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA FDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low", "QqualminFDD" and "QrxlevminFDD" in that occurrence of the IE "Frequency List".

…

2>	for each occurrence of the IE "UTRAN TDD Frequencies":

3>	if an entry already exists in the IE "Priority Info List" in the variable PRIORITY_INFO_LIST with the same priority value as indicated in the IE "priority" and has the CHOICE "Radio Access Technology" set to "UTRA TDD":

4>	create a new entry in the IE "Frequency List" in that occurrence of IE "Priority Info List", and store the value of "UARFCN" in the IE "UARFCN" in this occurrence of the IE "Frequency List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD".

3>	otherwise:

4>	create a new entry in the IE "Priority Info List" and store the value of "priority" in the IE "priority" in this occurrence of the IE "Priority Info List" within the variable PRIORITY_INFO_LIST, and in that new entry:

5>	set the CHOICE "Radio Access Technology" to "UTRA TDD" and store the value of "UARFCN" in the IE "UARFCN" in the first occurrence of the IE "Frequency List";

5>	store the IEs "Threshx, high", "Threshx, low" and "QrxlevminTDD" in that occurrence of the IE "Frequency List".

References

1.	TS 25.304, clause 5.2.6.1.2a, 5.2.6.1.4a

2.	TS 25.331, clause 8.1.1.6.19, 8.6.7.3a

8.3.1.1e.3	Test purpose

1.	Verify that for a UE signalling both FGI 3 and FGI 4, when Inter-Frequency cells with high , equal or low priority are present and SIB19 Cell_FACH_Absolute_Priority_Measurement_Indicator is set to “HIGH Layers” , the UE while in Cell_FACH state performs inter-frequency measurements and reselects to only high priority Inter-frequency cells only when corresponding cell re-selection criteria are met.

8.3.1.1e.4	Method of test

Initial conditions

System Simulator:

-	3 cells Cell 1, Cell 4 and Cell 7. Cell 1 belongs to a different frequency band than Cell 4 and Cell 7 .

-	Cell 1 as the active cell with SS downlink transmission power settings according to “T0” in table 8.3.1.1e below.

-	SIB19 to be transmitted by Cell 1 with neighbour configuration mentioned below.

User Equipment:

-	UE is brought to CELL_FACH state on Cell 1 with “T0” in table 8.3.1.1e below.

-	UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statement(s)

- UE supports UTRA FDD yes/no

- UE supports EUTRA yes/no

- UE supports FGI3 (UTRA CELL_FACH absolute priority cell reselection for high priority layers)  yes/no

Table 8.3.1.1e

| Parameter | Unit | Cell 1 |  |  | Cell 4 |  |  | Cell 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 | T0 | T1 | T2 |
| UTRA RF Channel Number |  | Mid Range Test Frequency |  |  | High Range Test Frequency |  |  | Low Range Test Frequency |  |  |
| CPICH Ec (FDD) | dBm/3.84MHz | -60 | -60 | -60 | -75 | -65 | -60 | -90 | -80 | -62 |
| Srxlev* | dB | 21 | 21 | 21 | 6 | 16 | 21 | -9 | 1 | 19 |
| qRxLevMinFDD | dB | -81 |  |  | -81 |  |  | -81 |  |  |
| Note: Srxlev is calculated in UE. During T1, UE should not meet the higher priority reselection criteria nor lower priority selection criteria During T2, UE should meet higher priority reselection criteria |  |  |  |  |  |  |  |  |  |  |

Test procedure

Method C is applied.

a)	SS transmits MIB with the “MIB value tag” IE different from the original setting and the SIB TYPE 19 with cell 7 as the higher priority cell and cell 4 as the lower priority cell

b)	SS notifies the UE by sending a SYSTEM INFORMATION CHANGE INDICATION with the BCCH modification and SS waits for 5s to ensure UE reads all the SIB information.

c)	SS configures downlink transmission power settings according to columns “T1” in the table 8.3.1.1e

d)	UE does not reselect to cell 4 or cell 7 as power levels do not meet the absolute priority cell reselection criteria defined in SIB19.

e)	SS monitors that the UE doesn’t send CELL UPDATE on cell 4 or cell 7

f)	SS configures the downlink transmission power settings according to columns “T2” from table 8.3.1.1e.

g)	SS waits for “Treselection” time configured in SIB3 and UE should perform reselection to higher priority cell 7 as the absolute priority reselection criteria is met now.

h)	UE sends a CELL UPDATE with cause – CELL RESELECTION on cell 7 and SS responds with CELL UPDATE CONFIRM.

Specific Message Contents

MASTER INFORMATION BLOCK

Use the same message sub-type found in clause 6.1 of TS 34.108, with the following exception.

| Information Element | Value/remark |
| --- | --- |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| SB 1 Cell Value tag | Set to (Current SIB4 value tag + 1) |

SCHEDULING BLOCK 1

| Information Element | Value/remark |
| --- | --- |
| SIB 11 Cell Value Tag | Set to (Current SIB 11 value tag + 1) |

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to TS 34.108 clause 6.1.0b, with the following exceptions:

System Information Block type 19 (Step a)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList |  |  |
| utra-ServingCell |  |  |
| Priority | [4] |  |
| s-PrioritySearch1 | [8 (16 dB)] |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | [4 (8 dB)] |  |
| threshServingLow2 | Not Present |  |
| utran-FDD-FrequencyList |  |  |
| uarfcn | Same downlink uarfcn as used for cell 7 |  |
| priority | [5] |  |
| threshXhigh | [5 (10 dB)] |  |
| threshXlow | [1 (2 dB)] |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | [-41(81 dBm)] |  |
| uarfcn | Same downlink uarfcn as used for cell 4 |  |
| priority | [3] |  |
| threshXhigh | [5 (10 dB)] |  |
| threshXlow | [1 (2 dB)] |  |
| qQualMinFDD | -24 dB |  |
| qRxLevMinFDD | [-41(81 dBm)] |  |
| utran-TDD-FrequencyList | Not present |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList (SIZE (1..maxNumEUTRAFreqs) | Not present |  |
| cell-fach-meas-ind | high-priority-layers |  |

PAGING TYPE 1 (Step b and Step g)

| Information Element | Value/Remark |
| --- | --- |
| PagingType1 |  |
| pagingRecordList | Not present |
| bcch-ModificationInfo |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |
| bcch-ModificationTime | Not present |
| laterNonCriticalExtensions SEQUENCE {} | Not present |

8.3.1.1e.5	Test requirements

1)	At step d), UE does not reselects to a higher priority Cell 7 and lower priority cell 4

2)	At step h), UE reselects to higher priority Cell 7.

#### 8.3.1.1f	Inter-frequency reselection to a no priority layer when NW and UE supports absolute priority in CELL_FACH

8.3.1.1f.1	Definition

This test case is identical to test case 8.3.1.1a from inter-frequency neighbours and reselection perspective and identical to testcase 8.3.1.1d from the NW and UE capability perspective (i.e. SIB19, FGI3 and FGI4). With NW and UE supporting absolute priority in CELL_FACH and inter-frequency layers are not configured with priority, UE is expected to perform cell reselection in CELL_FACH to cells belonging to different frequency bands.

8.3.1.1f.2	Conformance requirement

Same conformance requirement as clause 8.3.1.1.2

…

If none of the inter-frequency layers in UTRAN are provided with both priority and threshold parameters, the cell reselection criteria in subclause 5.2.6.1.4 shall apply for inter-frequency layers in UTRAN.

…

Reference

3GPP TS 25.331 clause 8.3.1

3GPP TS 25.304 clause 5.2.6.1.4a

8.3.1.1f.3	Test purpose

1.	To confirm that the UE executes a cell update procedure after the successful reselection of another UTRA cell in different frequency band.

2.	To confirm that the UE sends the correct uplink response message when executing cell update procedure due to cell reselection.

8.3.1.1f.4	Method of test

Initial Condition

Same initial conditions as clause 8.3.1.1.4 except,

Cell 2 is mapped to cell 4 (inter frequency) of 34.108 clause 6.1 and both cells belong to different frequency bands.

Following updated System information is transmitted in both cells after UE is brought to initial state of PS-DCCH+DTCH_FACH (state 6-11) in cell 1.

Related ICS/IXIT statement(s)

-	 UE supports multiple bands simultaneously	yes/no

- UE supports EUTRA yes/no

- UE supports FGI4 (UTRA CELL_FACH absolute priority cell reselection for all priority layers)  yes/no

-

NOTE: UE’s that indicate support of EUTRA Feature group Indicator FGI4 also indicate support of EUTRA Feature group Indicator FGI3

MASTER INFORMATION BLOCK

Use the same message sub-type found in clause 6.1 of TS 34.108, with the following exception.

| Information Element | Value/remark |
| --- | --- |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| SIB 4 Cell Value tag | Set to (Current SIB4 value tag + 1) |
| SB 1 Cell Value tag | Set to (Current SIB4 value tag + 1) |

SCHEDULING BLOCK 1

| Information Element | Value/remark |
| --- | --- |
| SIB 11 Cell Value Tag | Set to (Current SIB 11 value tag + 1) |
| SIB 19 Cell Value Tag | Set to (Current SIB 19 value tag + 1) |

System Information Block type 4

Use the same message type found in clause 6.1.0b of TS 34.108, with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| -Qualmin | -16dB |

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

System Information Block type 19

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to TS 34.108 clause 6.1.0b, with the following exceptions:

System Information Block type 19 (Step a)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList |  |  |
| utra-ServingCell |  |  |
| Priority | [4] |  |
| s-PrioritySearch1 | [8 (16 dB)] |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | [4 (8 dB)] |  |
| threshServingLow2 | Not Present |  |
| utran-FDD-FrequencyList | Not present |  |
| utran-TDD-FrequencyList | Not present |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList |  |  |
| earfcn | 3250 |  |
| measurementBandwidth | mbw100 |  |
| priority | 7 |  |
| qRxLevMinEUTR | -60 |  |
| threshXhigh 9 | 9 |  |
| threshXlow 2 | 2 |  |
| eutraDetection | TRUE |  |
| cell-fach-meas-ind | All-priority-layers |  |

SYSTEM INFORMATION CHANGE INDICATION

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| BCCH modification info |  |
| MIB Value tag | Set equal to Value tag sent in modified MIB |

Expected sequence

Same expected sequence as clause 8.3.1.1.4 with the following additional test steps:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 30a |  |  | CELL UPDATE CONFIRM | New C-RNTI identity is assigned to the UE. |
| 30b |  |  | UTRAN MOBILITY INFORMATION CONFIRM |  |

Specific Message Contents

Same specific message contents as clause 8.3.1.1.4. The message content for the additional test steps are given below.

CELL UPDATE CONFIRM (Step 30a)

Use the same message sub-type found in TS 34.108, clause 9, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| New C-RNTI | '0000000000000001' |

Test Procedure

Same test procedure as clause 8.3.1.1.4 with the following exception

After step 30a, SS shall send CELL UPDATE CONFIRM message to the UE. UE shall reply with UTRAN MOBILITY INFORMATION CONFIRM message.

NOTE:	If the UE supports more than 2 frequency bands, the test may be executed for various band combinations.

8.3.1.1f.5	Test requirement

Same test requirements as clause 8.3.1.1.5 with the following additional test requirements:

At step 30a, the SS shall send a CELL UPDATE CONFIRM

At step 30b, the UE shall transmit a UTRAN MOBILITY INFORMATION CONFIRM message

<End of changed Section>