3GPP TSG-RAN WG5 Meeting #72	R5-165187r1

Gothenburg, Sweden Aug 22nd-26th

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3862 | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UTRA Absolute Priority based reselection test case 8.3.1.1f |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  |  | Date: |  | 2016-08-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Omission of the ICS/IXIT statement(s) of Test case 8.3.1.1f on latest TS 34.123-1 Ver 12.4.0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added ICS/IXIT statement: UE supports FGI4(UTRA CELL_FACH absolute priority cell reselection for all priority layers) to test case 8.3.1.1f |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1f.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 34.123-2 CR 0771 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact R5-165187r1– Fixed the ICS\IXIT Statement to include UTRA CELL_FACH absolute priority cell reselection for “all” priority layers instead of “high” priority layers |  |  |  |  |  |  |  |  |

<Start of changed Section>

#### 8.3.1.1f	Inter-frequency reselection to a no priority layer when NW and UE supports absolute priority in CELL_FACH

8.3.1.1f.1	Definition

This test case is identical to test case 8.3.1.1a from inter-frequency neighbours and reselection perspective and identical to testcase 8.3.1.1d from the NW and UE capability perspective (i.e. SIB19, FGI3 and FGI4). With NW and UE supporting absolute priority in CELL_FACH and inter-frequency layers are not configured with priority, UE is expected to perform cell reselection in CELL_FACH to cells belonging to different frequency bands.

8.3.1.1f.2	Conformance requirement

Same conformance requirement as clause 8.3.1.1.2

…

If none of the inter-frequency layers in UTRAN are provided with both priority and threshold parameters, the cell reselection criteria in subclause 5.2.6.1.4 shall apply for inter-frequency layers in UTRAN.

…

Reference

3GPP TS 25.331 clause 8.3.1

3GPP TS 25.304 clause 5.2.6.1.4a

8.3.1.1f.3	Test purpose

1.	To confirm that the UE executes a cell update procedure after the successful reselection of another UTRA cell in different frequency band.

2.	To confirm that the UE sends the correct uplink response message when executing cell update procedure due to cell reselection.

8.3.1.1f.4	Method of test

Initial Condition

Same initial conditions as clause 8.3.1.1.4 except,

Cell 2 is mapped to cell 4 (inter frequency) of 34.108 clause 6.1 and both cells belong to different frequency bands. Cell 1 on Primary Band and Cell 4 on secondary band.

User Equipment:

-	UE is brought to CELL_FACH state on Cell 1 with “T0” as per table 8.3.1.1.

-	UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statement(s)

-	UE supports multiple bands simultaneously	yes/no

- UE supports EUTRA yes/no

- UE supports FGI4 (UTRA CELL_FACH absolute priority cell reselection for all priority layers)  yes/no

NOTE: UE’s that indicate support of EUTRA Feature group Indicator FGI4 also indicate support of EUTRA Feature group Indicator FGI3.

Refer to Test Procedure in clause 8.3.1.1.4

After step 30a, SS shall send CELL UPDATE CONFIRM message to the UE. UE shall reply with UTRAN MOBILITY INFORMATION CONFIRM message.

Steps 31 to 34 from 8.3.1.1.4 shall be skipped.

Expected Sequence

Same expected sequence as clause 8.3.1.1.4 with the following additional test steps:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 30a |  |  | CELL UPDATE CONFIRM | New C-RNTI identity is assigned to the UE. IE "RRC State Indicator" is set to Cell_FACH |
| 30b |  |  | UTRAN MOBILITY INFORMATION CONFIRM | Go to Step 35 to verify that UE is in CELL_FACH state. |

Specific Message Contents

MASTER INFORMATION BLOCK

Use the same message sub-type found in clause 6.1 of TS 34.108, with the following exception.

| Information Element | Value/remark |
| --- | --- |
| MIB Value Tag | Set to (Current MIB value tag + 1) |
| SIB 4 Cell Value tag | Set to (Current SIB4 value tag + 1) |
| SB 1 Cell Value tag | Set to (Current SIB4 value tag + 1) |

SCHEDULING BLOCK 1

| Information Element | Value/remark |
| --- | --- |
| SIB 11 Cell Value Tag | Set to (Current SIB 11 value tag + 1) |
| SIB 19 Cell Value Tag | Set to (Current SIB 19 value tag + 1) |

System Information Block type 4

Use the same message type found in clause 6.1.0b of TS 34.108, with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| -Qualmin | -16dB |

System Information Block type 11

Use the same message type found in clause 6.1 of TS 34.108, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| FACH measurement occasion info |  |
| - FACH Measurement occasion cycle length coefficient | 3 |
| - Inter-frequency FDD measurement indicator | TRUE |
| - Inter-frequency TDD measurement indicator | FALSE |
| - Inter-RAT measurement indicators | Not Present |

System Information Block type 19

The contents of the SYSTEM INFORMATION BLOCK TYPE 19 message in this test case is identical to TS 34.108 clause 6.1.0b, with the following exceptions:

System Information Block type 19 (Step a)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| SysInfoType19 |  | REL-8 or later |
| utra-PriorityInfoList |  |  |
| utra-ServingCell |  |  |
| Priority | [4] |  |
| s-PrioritySearch1 | [8 (16 dB)] |  |
| s-PrioritySearch2 | Not present |  |
| threshServingLow | [4 (8 dB)] |  |
| threshServingLow2 | Not Present |  |
| utran-FDD-FrequencyList | Not present |  |
| utran-TDD-FrequencyList | Not present |  |
| gsm-PriorityInfoList | Not present |  |
| eutra-FrequencyAndPriorityInfoList |  |  |
| earfcn | 3250 |  |
| measurementBandwidth | mbw100 |  |
| priority | 7 |  |
| qRxLevMinEUTR | -60 |  |
| threshXhigh 9 | 9 |  |
| threshXlow 2 | 2 |  |
| eutraDetection | TRUE |  |
| cell-fach-meas-ind | All-priority-layers |  |

SYSTEM INFORMATION CHANGE INDICATION

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| BCCH modification info |  |
| MIB Value tag | Set equal to Value tag sent in modified MIB |

CELL UPDATE CONFIRM (Step 30a)

Use the same message sub-type found in TS 34.108, clause 9, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| New C-RNTI | '0000000000000001' |

Test Procedure

Same test procedure as clause 8.3.1.1.4 with the following exception

After step 30a, SS shall send CELL UPDATE CONFIRM message to the UE. UE shall reply with UTRAN MOBILITY INFORMATION CONFIRM message.

NOTE:	If the UE supports more than 2 frequency bands, the test may be executed for various band combinations.

8.3.1.1f.5	Test requirement

Same test requirements as clause 8.3.1.1.5 with the following additional test requirements:

At step 30a, the SS shall send a CELL UPDATE CONFIRM

At step 30b, the UE shall transmit a UTRAN MOBILITY INFORMATION CONFIRM message

<End of changed Section>