3GPP TSG-RAN WG5 Meeting #72 	          R5-165411r1

# Gotenburg, Sweden, 22nd - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 3545 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New eMTC Test Case – 7.1.2.3a Correct selection of RACH parameters/ Preamble selected by MAC itself/ Contention based random access procedure/ Enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-UEConTest |  |  |  |  |  |  | Date: |  | 2016-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Addition of new eMTC test case 7.1.2.3a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added New eMTC Test Case – 7.1.2.3a Correct selection of RACH parameters/ Preamble selected by MAC itself/ Contention based random access procedure/ Enhanced coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3a (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.523-2 CR 0893 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other Comments |  | TTCN Impact |  |  |  |  |  |  |  |  |
|  |  | Revision1: - Updated test steps in Table 7.1.2.3a.3.3-1: Main behaviour - Editorial changes. |  |  |  |  |  |  |  |  |

#### 7.1.2.3a	Correct selection of RACH parameters/ Preamble selected by MAC itself/ Contention based random access procedure/ Enhanced coverage

7.1.2.3a.1 	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE in enhanced coverage receives a Paging message and preambles contained in Random Access Preamble groups in preambleMappingInfoList for each CE level is indicated in SIB2 }

then { UE in enhanced coverage transmits correct preamble calculated based on firstPreamble and lastPreamble indicated in SIB2 }

}

(2)

with { UE in E-UTRA RRC_IDLE state and has transmitted initial Msg1}

ensure that {

when { SS sends numRepetitionPerPreambleAttempt in SIB2 }

then { UE in enhanced coverage transmits a preamble with the number of repetitions required for preamble transmission corresponding to the selected preamble group using the selected PRACH corresponding to the selected CE level, corresponding RA-RNTI, prach-ConfigIndex and preambleInitialReceivedTargetPower indicated in SIB2}

}

(3)

with { UE in E-UTRA RRC_IDLE state and has transmitted Msg3 }

ensure that {

when { SS does not respond before contention resolution timer expiry }

then { UE in enhanced coverage transmits a random access preamble using a preamble contained in Random Access Preamble groups in preambleMappingInfoList as used for the first transmission of Msg3 }

}

(4)

with { UE in E-UTRA RRC_IDLE state and has transmitted Msg3  }

ensure that {

when { SS does not respond before contention resolution timer expiry after more than maxNumPreambleAttemptCE transmissions from UE }

then { UE in enhanced coverage transmits a random access preamble using a preamble in the same group of random access preambles as used for the first transmission of Msg3 }

}

7.1.2.3a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clause 5.1.1, 5.1.2, 5.1.3 and 5.1.5. Unless otherwise stated these are Rel-13 requirements.

[TS 36.321, clause 5.1.1]

The following information for related Serving Cell is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in enhanced coverage:

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the available set of PRACH resources associated with each enhanced coverage level supported in the Serving Cell for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group, preambleMappingInfo (SpCell only).

The preambles that are contained in Random Access Preambles groups for each enhanced coverage level, if it exists, are calculated from the parameters firstPreamble and lastPreamble.

…

-	the criteria to select PRACH resources based on RSRP measurement per enhanced coverage level supported in the Serving Cell rsrp-ThresholdsPrachInfoList.

-	the maximum number of preamble transmission attempts per enhanced coverage level supported in the Serving Cell maxNumPreambleAttemptCE.

-	the number of repetitions required for preamble transmission per attempt for each enhanced coverage level supported in the Serving Cell numRepetitionPerPreambleAttempt.

-	the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10].

-	the RA response window size ra-ResponseWindowSize and the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only) per enhanced coverage level supported in the Serving Cell.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax-CE.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6). For NB-IoT the DELTA_PREAMBLE is set to 0.

- 	if sizeOfRA-PreamblesGroupA is not equal to numberOfRA-Preambles, then Random Access Preambles group B exists for all enhanced coverage levels and is calculated as above.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	If, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

…

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	If Msg3 has not yet been transmitted, the MAC entity shall, for NB-IoT UEs, BL UEs or UEs in enhanced coverage:

-	select the Random Access Preambles group and the PRACH resource corresponding to the selected enhanced coverage level and, in case of NB-IoT, additionally corresponding to the support for multi-tone Msg3 transmission;

…

-	else, if Msg3 is being retransmitted, the MAC entity shall:

-	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

-	randomly select a Random Access Preamble within the selected group. The random function shall be such that each of the allowed selections can be chosen with equal probability;

-	except for NB-IoT, set PRACH Mask Index to 0.

-	determine the next available subframe containing PRACH permitted by the restrictions given by the prach-ConfigIndex (except for NB-IoT), the PRACH Mask Index (except for NB-IoT, see subclause 7.3), physical layer timing requirements [2] and in case of NB-IoT selected enhanced coverage level and the support for multi-tone Msg3 transmission (a MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH subframe);

-	if the transmission mode is TDD and the PRACH Mask Index is equal to zero:

-	if ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	randomly select, with equal probability, one PRACH from the PRACHs available in the determined subframe.

-	else:

-	randomly select, with equal probability, one PRACH from the PRACHs available in the determined subframe and the next two consecutive subframes.

-	else:

-	determine a PRACH within the determined subframe in accordance with the requirements of the PRACH Mask Index.

-	for NB-IoT UEs, BL UEs or UEs in enhanced coverage, select the ra-ResponseWindowSize and mac-ContentionResolutionTimer corresponding to the selected enhanced coverage level and PRACH.

[TS 36.321, clause 5.1.3]

The random-access procedure shall be performed as follows:

-	set PREAMBLE_RECEIVED_TARGET_POWER to preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set to:
PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

…

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	instruct the physical layer to transmit a preamble with the number of repetitions required for preamble transmission corresponding to the selected preamble group (i.e., numRepetitionPerPreambleAttempt) using the selected PRACH corresponding to the selected enhanced coverage level, corresponding RA-RNTI, preamble index or for NB-IoT subcarrier index, and PREAMBLE_RECEIVED_TARGET_POWER.

-	else:

-	instruct the physical layer to transmit a preamble using the selected PRACH, corresponding RA-RNTI, preamble index and PREAMBLE_RECEIVED_TARGET_POWER.

[TS 36.321, clause 5.1.5]

Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH. If the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage, the MAC entity shall use the mac-ContentionResolutionTimer for the corresponding enhanced coverage level if it exists.

Once Msg3 is transmitted, the MAC entity shall:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission;

-	regardless of the possible occurrence of a measurement gap or Sidelink Discovery Gap for Reception, monitor the PDCCH until mac-ContentionResolutionTimer expires or is stopped;

...

-	if mac-ContentionResolutionTimer expires:

-	discard the Temporary C-RNTI;

-	consider the Contention Resolution not successful.

-	if the Contention Resolution is considered not successful the MAC entity shall:

…

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	indicate a Random Access problem to upper layers.

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	indicate a Random Access problem to upper layers.

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	proceed to the selection of a Random Access Resource

7.1.2.3a.3	Test description

7.1.2.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

-	System information set using parameters as specified in Table 7.1.2.3a.3.4-1.

UE:

None.

Preamble:

-	The UE is in Registered, Idle Mode (state 2) according to TS 36.508 [18].

7.1.2.3a.3.2	Test procedure sequence

Table 7.1.2.3a.3.2-1 illustrates the downlink power levels to be applied for the cell at various time instance of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instance on which these values shall be applied are described in the texts in this clause.

Table 7.1.2.3a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -70 | The power level value is such that UE satisfies CE level 0 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -79 | The power level value is such that UE satisfies CE level 1 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -88 | The power level value is such that UE satisfies CE level 2 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -96 | The power level value is such that UE satisfies CE level 3 |

7.1.2.3a.3.3	Test procedure sequence

Table 7.1.2.3a.3.3-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | Exception: Steps 1-9 are repeated once for each time instant specified in table 7.1.2.3a.3.2-1. | - | - | - | - |
| 1 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
|  | Exception: Step2, Step3 and Step4 is repeated for numRepetitionPerPreambleAttempt times configured for corresponding CE level | - | - | - | - |
| 2 | Check: Does the UE transmit a preamble on PRACH, calculated based on parameters firstPreamble-r13 and lastPreamble-r13 for corresponding CE level defined in SIB2? | --> | PRACH Preamble | 1 | P |
| 3 | Check: Does the UE transmit same preamble as step2 on PRACH? | --> | PRACH Preamble | 2 | P |
| 4 | Check: Does the UE transmit a preamble on PRACH, calculated based on parameters firstPreamble-r13 and lastPreamble-r13 for corresponding CE level defined in SIB2? | --> | PRACH Preamble | 1 | P |
| 5 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step4, including T-CRNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 6 | The UE transmits an RRCConnectionRequest message. | --> | MAC PDU | - | - |
|  | Exception: Step7 is repeated for numRepetitionPerPreambleAttempt times configured for corresponding CE level | - | - | - | - |
| 7 | Check: Does the UE transmit preamble on PRACH using a preamble belonging to same CE level as of step2? | --> | PRACH Preamble | 3 | P |
| 8 | The SS waits for preambleTransMax-CE + 1-4 further PRACH preambles and ignores them. | - | - | 3, 4 | P |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) according to TS 36.508 [18]. | - | - | - | - |

7.1.2.3a.3.4	Specific message contents

Table 7.1.2.3a.3.4-1: SystemInformationBlockType2 (all steps, table 7.1.2.3a.3.3-1, FFS)