3GPP TSG-RAN WG5 Meeting #72	R5-165800r2

Göteborg, Sweden, 22 - 26 August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 1523 | rev | - | Current version: | 12.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Test Case: 7.3.50: E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-UEConfTest |  |  |  |  |  |  | Date: |  | 08/12/2016 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Cat-M1 CEMode A RLM test cases for FD-FDD out-of-sync with DRX are not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the definition of UE category M1 applicability to clause 3.1 Add two symbols (Qout_Cat M1, Qin_Cat M1) to clause 3.2 CR Table H.2.4-1 Define Cat-M1 CEMode A RLM test case for FD-FDD out-of-sync with DRX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test cases defined for evaluating Cat-M1 CEMode A RLM test case for FD-FDD out-of-sync with DRX |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 2, 3.1, 3.2, H2.4 and 7.3.50 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-2 CR 0430 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Updates in r1: - Moved Editor’s notes after the section title - Added “Annex E needs to be updated” in Editor’s notes because there is no cell configuration mapping for the specified test case - Section 7.3.50.2, changed the sentence to be consistence with other TCs. - Section 7.3.50.4.2, changed “State 3A-RF” to “State 3A-RF-CE” - Moved table 7.3.50.3-3 from Section 7.3.50.3 to Section 7.3.50.4.1 and changed the table number from 7.3.50.3-3 to 7.3.50.4.1-1 and its references accordingly (in Sections 7.3.50.3 and 7.3.50.4.1 and Table 7.3.50.5-1 (note 7). - Moved 3 tables (7.3.50.3-4 to 7.3.50.3-6) from Section 7.3.50.3 to Section 7.3.50.5 and changed the table numbers from 7.3.50.3-4, 7.3.50.3-5 and 7.3.50.3-6 to 7.3.50.5-1, 7.3.50.5-2 and 7.3.50.5-3, respectively, and their references accordingly (in Sections 7.3.50.3 and 7.3.50.4.2). - Deleted “2Tx” condition in Table 7.3.50.4.3-5 - In table 7.3.50.4.1-1, changed “repletion” into “repetition” - In table 7.3.50.4.3-2, changed rf-ConfigIndex from “NULL” to “Not Present” and added clause 7.2.2 since it cannot be set to NULL and Cat M UE does not support ri-ConfigIndex per TS 36.213 (page 110). Updates in r2: Move H2.4 for change 3 7.3.50.2: changed category M to M1 |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

# 2	References

[31]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

3.1	Definitions

UE category M1 applicability: The requirements for UE category M1 are derived assuming: DL Category M1 and Uplink Category M1, operation in any LTE system bandwidth but with a channel bandwidth of 1.4 MHz and transmission bandwidth of 6 PRBs in downlink and uplink, and a single antenna receiver. DL UE category M1 and UL UE category M1 are defined in TS 36.306 [14].  The requirements for CEMode A shall apply provided the UE category M1 is configured with CEMode A, SCH Ês/Iot  -6 dB and CRS Ês/Iot  -6 dB.  The CEMode A and the number of repetition levels for different physical channels are defined in TS 36.213 [8]. The requirements for CEMode B shall apply provided the UE category M1 is configured with CEMode B, SCH  Ês/Iot  -15 dB and CRS Ês/Iot  -15 dB. The CEMode B and the number of repetition levels for different physical channels are defined in TS 36.213 [8].

3.2	Symbols

Qout_Cat M1                 		Defined in TS36.133 clause 7.19.2

Qin_Cat M1			Defined in TS36.133 clause 7.19.2

---END OF CHANGE 1---

---START OF CHANGE 2---

### 7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

- The SNRs are TBD

- UE Transmit minimum power and Transmit OFF power are TBD for a carrier frequency above 3GHz

- The Test system uncertainties applicable to this test are undefined

- The Test tolerances applicable to this test are undefined

#### - Annex E needs to be updated

#### 7.3.50.1	Test purpose

The purpose of this test is to verify that the FD-FDD category M1 UE configured in CEMode A properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in TS 36.133 Section 7.19 [4].

#### 7.3.50.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 13 and forward of UE category M1. Applicability requires support for FGI bit 5.

#### 7.3.50.3	Minimum conformance requirements

The UE Cat-M1 applicability of the requirements for performing radio link monitoring is defined in Section 3.1.

The UE shall estimate the downlink radio link quality and compare it to the thresholds Qout_Cat M1 and Qin_Cat M1 for the purpose of monitoring downlink radio link quality of the PCell.

When DRX is used for FD-FDD and TDD category M1 UEs, the Qout_CatM1 evaluation period (TEvaluate_Qout_DRX_CatM1) and the Qin_CatM1 evaluation period (TEvaluate_Qin_DRX_CatM1) specified in Table 7.3.50.3-1 (TS 36.133 [4] Table 7.19.2.2-1) will be used.

When eDRX_CONN cycle is used for FD-FDD and TDD UE category M1 UEs, the Qout_CatM1 evaluation period (TEvaluate_Qout_DRX_CatM1) and the Qin_CatM1 evaluation period (TEvaluate_Qin_DRX_CatM1) specified in Table 7.3.50.3-2 (TS 36.133 [4] Table 7.19.2.2-2) will be used.

When the downlink radio link quality of the PCell estimated over the last TEvaluate_Qout_DRX_CatM1 [s] period becomes worse than the threshold Qout_CatM1, Layer 1 of the UE shall send out-of-sync indication for the PCell to the higher layers within TEvaluate_Qout_DRX_CatM1 [s] evaluation period. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 36.331 [5].

When the downlink radio link quality of the PCell estimated over the last TEvaluate_Qin_DRX_CatM1 [s] period becomes better than the threshold Qin_CatM1, Layer 1 of the UE shall send in-sync indications for the PCell to the higher layers within TEvaluate_Qin_DRX_CatM1 [s] evaluation period. A L3 filter shall be applied to the in-sync indications as specified in TS 36.331 [5].

The out-of-sync and in-sync evaluations of the PCell shall be performed as specified in clause 4.2.1 in TS 36.213 [8]. When DRX is used, two successive indications from Layer 1 shall be separated by at least max(10ms, DRX_cycle_length). When eDRX_CONN is used, two successive indications from Layer 1 shall be separated by at least max(10 ms, eDRX_CONN cycle length).

Upon start of T310 timer as specified in clause 5.3.11 in TS 36.331 [5], the UE shall monitor the link for recovery using the evaluation period and Layer 1 indication interval corresponding to the non-DRX mode until the expiry or stop of T310 timer.

The transmitter power of the UE shall be turned off within 40 ms after expiry of T310 timer as specified in clause 5.3.11 in TS 36.331 [5].

Table 7.3.50.3-1: Qout_CatM1 and Qin_CatM1 Evaluation Period in DRX for FD-FDD and TDD UE category M1

| DRX cycle length (s) | TEvaluate_Qout_DRX_CatM1 and TEvaluate_Qin_DRX_CatM1 (s) (DRX cycles) |
| --- | --- |
| ≤ 0.01 | Non-DRX requirements in TS36.133 [4] clause 7.19.2.1 are applicable. |
| 0.01 < DRX cycle ≤0.04 | Note (20) |
| 0.04 < DRX cycle ≤ 0. 64 | Note (10) |
| 0.64 < DRX cycle ≤ 2.56 | Note (5) |
| NOTE: Evaluation period length in time depends on the length of the DRX cycle in use |  |

Table 7.3.50.3-2: Qout_CatM1 and Qin_CatM1 evaluation period when eDRX_CONN cycle is configured for FD-FDD and TDD UE category M1

| eDRX_CONN cycle length (s) | TEvaluate_Qout_DRX_CatM1 and TEvaluate_Qin_DRX_CatM1 (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56 < eDRX_CONN cycle ≤ 10.24 | Note (5) |
| NOTE: Evaluation period length in time depends on the length of the eDRX_CONN cycle in use |  |  |

The normative reference for this requirement is TS 36.133 [4] clause 7.19.1, 7.19.2 and A.7.3.50.

The test parameters are given in Tables 7.3.50.4.1-1, 7.3.50.5-1, 7.3.50.5-2 and 7.3.50.5-3.

#### 7.3.50.4	Test description

There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 7.3.50.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync states.

There are two sets of values for MPDCCH aggregation level and MPDCCH repetition level, respectively. In each run of the test, the test equipment randomly selects one set of values (Set 1 or Set 2) for both MPDCCH aggregation level and MPDCCH repetition level, and sends the RRC configuration to the UE. MPDCCH aggregation level is determined by RRC parameter numberPRB-Pairs, and value 6 and 4 are corresponding to Set 1 and Set 2, respectively. MPDCCH repetition level is determined by RRC parameter mPDCCH-NumRepetition, and value 8 and 4 are corresponding to Set 1 and Set 2, respectively. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Figure 7.3.50.4-1: SNR variation for out-of-sync testing in DRX

##### 7.3.50.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.

1.    Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A. 10 (without using the faders) using only main Tx/Rx antenna.

2.    The general test parameter settings for the different subtests are set up according to Table 7.3.50.4.1-1.

3.    Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 7.3.50.4.3.

5.    There is one cell and one E-UTRA FD-FDD carrier specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

Table 7.3.50.4.1-1-: General test parameters for E-UTRAN FD-FDD out-of-sync tests in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in TS 36.212 [31] |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in TS 36.133 [4] clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | Set 1: 24 Set 2:16 |  |
|  | MPDCCH repetition level |  | Set 1: 8 Set 2: 4 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
|  | A, B |  | -3 |  |
| DRX cycle |  | ms | 1280 | See Table 7.3.50.5-2 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213 [8]. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

7.3.50.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 7.2A.3.

2.	Set the parameters according to T1 in Table 7.3.50.5-1. Propagation conditions are set according to Annex B clause B.1. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.3.50.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.3.50.5-1. T3 starts.

5.	If the SS a) detects uplink power equal to or higher than [-39] dBm (as defined in TS 36.521-1[10] clause 6.3.2.5 for carrier frequency less than [3]GHz) in each subframe configured for CQI transmission (according CQI reporting mode PUCCH 1-0) during the period from time point A to time point B, and b) does not detect any uplink power higher than [-48.5] dBm (as defined in TS 36.521-1[10] clause 6.3.3.5 for carrier frequency less than [3]GHz) from time point C (6500 ms (D1) after the start of T3) until T3 expires, then the number of successful tests is increased by one. Otherwise the number of failed tests is increased by one.

6.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 7.3.50.5-1.

7.	If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 7.2A.3.

8.    Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

##### 7.3.50.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 7.3.50.4.3-1: Common Exception messages for E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.4-1 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.3.50.4.3-2: CQI-ReportConfig-DEFAULT: Additional E-UTRAN FD-FDD Radio Link Monitoring

Test for Out-of-sync in DRX for UE category M1 configured in CEMode A

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULTT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig- DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportPeriodic | ON |  |  |
| cqi-ReportPeriodic ::= CHOICE { |  |  | CQI_PERIODIC |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 0 | (see Table 7.2.2-1A in TS 36.213 [8]) |  |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | Not present | (see Table 7.2.2- 1B and clause 7.2.2 in TS 36.213 [8]) |  |
| simultaneousAckNackAndCQI | FALSE | BOOLEAN |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CQI_PERIODIC | When periodic CQI reporting should be enabled |

Table 7.3.50.4.3-3: MAC-MainConfig-RBC: E-UTRAN FD-FDD Radio Link Monitoring test for In-sync in DRX UE category M1 configured in CEMode A

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| periodicBSR-Timer |  | sf20 |  |  |
| retxBSR-Timer |  | sf1280 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  | pc_FeatrGrp_5 AND DRX_S |
| Setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf10 |  |  |
| drx-InactivityTimer |  | psf1 |  |  |
| drx-RetransmissionTimer |  | psf1 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  | Sf1280 typical value in real network for real-time services. |  |
| Sf1280 |  | 0 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| } |  |  |  |  |

Table 7.3.50.4.3-4: PhysicalConfigDedicated-DEFAULT: Additional E-UTRAN FD-FDD Radio Link Monitoring test for Out-of-sync in UE category M1 configured in CEMode A

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| schedulingRequestConfig |  | SchedulingRequest-Config-DEFAULT |  | RBC |
| } |  |  |  |  |

Table 7.3.50.4.3-5: PhysicalConfigDedicated-DEFAULT: Additional E-UTRAN FD-FDD Radio Link Monitoring test for Out-of-sync in DRX in UE category M1 configured in CEMode A

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| antennaInfo CHOICE { |  |  |  |  |
| defaultValue |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.50.4.3-6: SchedulingRequest-Config-DEFAULT: Additional E-UTRAN FD-FDD Radio Link Monitoring test for Out-of-sync in DRX for UE category M1 with CE mode A

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20 SchedulingRequest-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-PUCCH-ResourceIndex | 41 | 10 MHz channel bandwidth parameter |  |
| sr-ConfigIndex | 0 |  |  |
| dsr-TransMax | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.3.50.5	Test requirement

Table 7.3.50.5-1: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.17 FDD Note 7 |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | TBD | TBD | TBD |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.3.50.4-1. Note 7: Aggregation level and repetition level specified in Table 7.3.50.4.1-1 are used for this test. |  |  |  |  |

Table 7.3.50.5-2: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 [5] |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table 7.3.50.5-3: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [5] |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 [5] and section 10.1 in TS 36.213 [8]. |

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The uplink signal is verified on the basis of the UE output power:

-	UE output power equal to or higher than Transmit minimum power [-39] dBm (as defined in TS 36.521-1 [10] clause 6.3.2.5 for f<3GHz) means uplink signal,

-	UE output power equal to or less than Transmit OFF power [-48.5] dBm (as defined in TS 36.521-1 [10] clause 6.3.3.5 for f<3GHz) means no uplink signal.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

---END OF CHANGE 2---

---START OF CHANGE 3---

## H.2.4	System information blocks message contents exceptions for E-UTRAN radio link monitoring (RLM)

Table H.2.4-1: SystemInformationBlockType2: E-UTRAN Radio Link Monitoring test for out-of-sync

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType 2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| pdsch-ConfigCommon SEQUENCE { |  |  |  |
| referenceSignalPower | Set to an arbitrarily selected value above -11dBm and within the IE allowed range described in 36.331[5] | The selected IE value depends on the test system implementation and should be declared in the test report. |  |
| } |  |  |  |
| soundingRS-UL-ConfigCommon CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-TimersAndConstants { |  |  |  |
| t300 | ms1000 |  |  |
| t301 | ms1000 |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

---END OF CHANGE 3---