3GPP TSG-RAN WG5 Meeting #76	   R5-173846r1

Berlin, Germany, 21st – 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1157 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Generic Test Procedure for NG eCall setup and MSD update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2017-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of Generic Test Procedure for NG eCall setup and MSD update |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Clause defined for the addition of Generic Test Procedure for eCall over IMS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCall over IMS workplan test cases will have to include this procedure in the test Prose, better to define a generic Procedure to avoid redundancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.XX (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN Impact Please Note: RAN5 Secretary to assign Cxx. Also, A.2.x and A.2.Y to be updated based on R5-173992. Revision1: - Added ACK in the generic procedure. - Renumbered test steps. |  |  |  |  |  |  |  |  |

<Start of new section>

# C.XX	Generic Test Procedure for NG eCall setup and MSD Update

Test procedure:

- SS waits for UE to send an INVITE request with initial MSD.

2)   SS ACK reception of INVITE request.

3)	SS responds to the INVITE request with a 200 OK response.

4)	SS sends a SIP INFO to request UE for a MSD update.

5)	UE responds to the MSD update request with 200 OK response.

6)	SS waits for UE to send a SIP INFO with updated MSD.

7)	The SS responds to the SIP INFO with a 200 OK response.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with initial MSD. |
| 2 |  |  | ACK | SS ACK for INVITE. |
| 3 |  |  | 200 OK | SS responds INVITE with 200 OK. |
| 4 |  |  | SIP INFO | SS sends SIP INFO to request MSD update. |
| 5 |  |  | 200 OK | UE responds SIP INFO with 200 OK. |
| 6 |  |  | SIP INFO | UE sends SIP INFO with MSD update. |
| 7 |  |  | 200 OK | SS responds SIP INFO with 200 OK. |

Specific Message Contents

INVITE (Step 1)

Use the default message “INVITE for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec (AMR/8000 and/or AMR-WB/16000) shall be present in the media attributes, optionally including channel number "/1". |

200 OK for INVITE (Step 3)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS:37 - b=RS:0 - b=RR:0 Attributes for media: - a=rtpmap: (payload type) AMR/8000/1 or AMR-WB/16000/1 [Note 1] [Note2] - a=fmtp: (format) mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. Note 2: If UE included AMR-WB/16000 in step 1, SS uses AMR-WB/16000/1. Otherwise SS uses AMR/8000/1. |

INFO (Step 4)

Use the default message “INFO for MT” in annex A.2.Y.

INFO (Step 6)

Use the default message “INFO for MO” in annex A.2.X.

<End of new section>