3GPP TSG-RAN5 Meeting #76	R5-174216r1

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 1978 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new RRM FDD 5DL CA TC 8.16.77 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | 7Layers |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel13-UEConTest |  |  |  |  |  |  | Date: |  | 2017-08-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Work Item LTE_CA_Rel13-UEConTest requires a new TC for “E-UTRAN FDD 5DL CA activation and deactivation of known SCell in non-DRX” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new TC 8.16.77 which specifies testing for “E-UTRAN FDD 5DL CA activation and deactivation of known SCell in non-DRX” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item LTE_CA_Rel13-UEConTest could not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.77 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR… CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Revision has removed Editors Note “The entry for Annex E is missing” since this is meanwhile considered in CR R5-174139. |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 8.16.77	E-UTRAN FDD 5DL CA activation and deactivation of known SCell in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

- Applicability Release to be confirmed and necessary FGI bits to be determined

- Square brackets “K (0 ≤ K ≤ [3])” remain in one core requirement, but do not affect this test case

- The message contents, including titles, are copied from 3DL Test case 8.16.37 and need to be updated for 5DL

#### 8.16.77.1	Test purpose

To verify that the SCell activation and deactivation times are within the requirements stated in TS 36.133 [4] section 7.7 for UE configured with five downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in FDD.

#### 8.16.77.2	Test applicability

This test applies to all types of E-UTRA FDD Release [12] and forward UE that support E-UTRA FDD and 5DL CA with FDD as PCell. Applicability requires support for FGI bit 25[FFS].

#### 8.16.77.3	Minimum conformance requirements

While activating a SCell if the UE does not receive any command to activate, deactivate, configure or deconfigure the other SCell during the SCell activation delay then the UE shall meet the SCell activation delay requirements specified in TS 36.133 [4] section 7.7.2.

While activating a SCell if the other SCell is activated, deactivated, configured or deconfigured by the UE then the UE shall meet the SCell activation delay requirements (Tactivate_total) according to the following expression:

Tactivate_total = Tactivate_basic+ K*5

Where:

Tactivate_total is the total time to activate a SCell and is expressed in subframes.

Tactivate_basic is the SCell activation delay specified in TS 36.133 [4] section 7.7.2;

K (1 ≤ K ≤ [3]) is the number of times the other SCell is activated, deactivated, configured or deconfigured while the SCell is being activated;

While activating a SCell:

-	The interruption on the PCell and/or on the activated SCell due to the SCell activation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 if:

-	The PCell and/or the activated SCell being interrupted and the SCell being activated belong to E-UTRA FDD, or

-	The activated SCell being interrupted and the SCell being activated belong to E-UTRA FDD and the PCell belongs to E-UTRA FDD.

-	Otherwise, the interruption on PCell and/or on the activated SCell due to the SCell activation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9.

Starting from the subframe specified in section 4.3 of TS 36.213 [8] and until the UE has completed the SCell activation, the UE shall send CSI with CQI index = 0 (out of range) if the UE is configured to report the CQI for the SCell being activated.

The UE shall deactivate a SCell and meet the SCell deactivation delay requirements specified in TS 36.133 [4] section 7.7.3 regardless of whether the other SCell is activated, deactivated, configured or deconfigured or not by the UE during the SCell deactivation delay.

While deactivating a SCell:

-	The interruption on the PCell and/or on the activated SCell due to the SCell deactivation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 if:

-	The PCell and/or the activated SCell being interrupted and the SCell being deactivated belong to E-UTRA TDD or

-	The activated SCell being interrupted and the SCell being deactivated belong to E-UTRA FDD and the PCell belongs to E-UTRA FDD.

-	Otherwise, the interruption on PCell and/or the activated SCell due to the SCell deactivation specified in TS 36.133 [4] section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9.

The normative reference for this requirement is TS 36.133 [4] clause 7.7 and A.8.16.77.

#### 8.16.77.4	Test description

##### 8.16.77.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-2 and TS 36.508 [7] clauses 4.4.2 and 4.3.1 for different CA bandwidth classes.

Channel Bandwidth to be tested: The largest aggregated bandwidth combination supported by UE from Table 8.16.77.5-1 as defined in TS 36.508 [7] clause 4.3.1 for different CA configurations as defined in TS 36.521-1[10] clause 5.4.2A.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.63 as appropriate

2.	The general test parameter settings are set up according to Table 8.16.77.4.1-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 8.16.77.4.3.

5.	Cell 1 is the PCell on the primary component carrier, Cell 2 is the SCell1 on RF channel number 2, Cell 3 is the deactivated SCell2 on RF channel number 3,  Cell 4 is the SCell3 on RF channel number 4, and Cell 5 is Configured deactivated SCell on RF channel number 5. Cell 1 is the cell used for connection setup with the power levels set according to Annex C.0 and C.1 for this test. Cell 2, Cell 3, Cell 4, and Cell 5 shall be powered OFF.

Table 8.16.77.4.1-1: General test parameters for E-UTRAN FDD 5 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4,5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell3 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

##### 8.16.77.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. Cell 1, Cell 3, Cell 4 and Cell5 have constant signal levels throughout the test.

Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), Cell4 (deactivated SCell3) on radio channel 4 (SCC3), and Cell5 (deactivated SCell4) on radio channel 5 (SCC4),  but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in PCell throughout the whole test.

During T2, the Test procedure requires the UE to send the first CSI report for SCell1 in a subframe (m+8), but also allows a subframe (m+10) if the subframe (m+8) was subject to interruption. The SS determines whether the CSI report in subframe (m+8) was interrupted or not by monitoring ACK/NACK sent in PCell in subframe (m+8).

NOTE:	The requirements are dependant on the CA configuration (e.g. intra band, inter band; see TS 36.133 [4], clause 7.8) hence the test procedure has been divided into three variants, found in steps 8a, 8b and 8c and in steps 11a, 11b and 11c. Depending on the CA configuration under test only one of those shall be run respectively.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3.

2.	Configure SCC2, SCC3 and SCC4 according to Annex C.0, C.1 for all downlink physical channels.

3.	The SS shall configure SCell2 (Cell 3) on the SCC2 , SCell3 (Cell 4) on the SCC3 and SCell4(Cell 5) on the SCC 5 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.77.4.3.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	Set the parameters according to T1 in Table 8.16.77.5-1. Propagation conditions are set according to Annex B clauses B.1.1. T1 starts. Configure SCC1 according to Annex C.0, C.1 for all downlink physical channels. The SS shall configure SCell1 (Cell 2) on the SCC1 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.77.4.3.

6.	The SS activates SCell1 by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted m which is an even number. If the SS receives ACK for MAC-CE sent by the UE, T2 starts in subframe m, and the test proceeds to step 7, otherwise go to step 12.

7.	The SS activates SCell2, SCell3 and SCell4 by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe (m+10), subframe(m+20) and subframe(m+30) respectively.

8.	The UE shall start sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCells activation.

8a.	For all intra-band case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+39),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+39)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23), the subframes (m+25) to (m+33)and the subframes (m+35) to (m+43) up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one, Otherwise, count a fail for the event “Activation” and go to step 12.

8b.	For the case where the band relation between PCC and SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8)

-	or (m+10) if the subframe (m+8) was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+39),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+39)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23), the subframes (m+25) to (m+33)and the subframes (m+35) to (m+43) up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+5) to (m+13), the subframes (m+15) to (m+23) , the subframes (m+25) to (m+33) the subframes (m+35) to (m+43) up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one, Otherwise, count a fail for the event “Activation” and go to step 12.

8c.	For the case where the band relation between PCC and SCell1 is intra-band and the others SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+39),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+39)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+13), the subframes (m+15) to (m+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43) up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (n+15) to (n+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43)up to the end of T2,

-	Then the number of successes for the event “Activation” is increased by one, Otherwise, count a fail for the event “Activation” and go to step 12.

9.	When T2 expires, the SS deactivate SCell1 by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted n which is an even number. If the SS receives ACK for MAC-CE sent by the UE, T3 starts in subframe n, and the test proceeds to step 10, otherwise go to step 12.

10.	The SS deactivates SCell2 , SCell3 and SCell4 by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe (n+10) , subframe (n+20) and subframe (n+30) respectively.

11.	The UE shall stop sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCell1 deactivation.

11a. For all intra-band case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43) up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11b. For the case where the band relation between PCC and SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43) up to the end of T3,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+5) to (n+13), the subframes (n+15) to (n+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43) up to the end of T3

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

11c. For the case where the band relation between PCC and SCell1 is intra-band and the others SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+13), the subframes (n+15) to (n+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43) up to the end of T3,

-	and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (n+15) to (n+23), the subframes (m+25) to (m+33) and the subframes (m+35) to (m+43) up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

12.When T3 expires or Activation in step 6 was not acknowledged, or a fail was counted for the event “Activation” in step 8a, 8b or 8c, or Deactivation in step 9 was not acknowledged, the SS shall transmit a RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [7] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3),
or
- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3,

15.	 Repeat steps 2-14 until a test verdict has been achieved,

Each of the events “Activation” and “Deactivation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

##### 8.16.77.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 8.16.37.4.3-1: Common Exception messages for E-UTRAN FDD 3DL CA activation and deactivation of known SCell in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |

Table 8.16.37.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional E-UTRAN FDD 3DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: 36.508 clause 4.8.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT using condition CQI_PERIODIC |  | RBC |
| cqi-ReportConfig-r10 | Not present |  | SCell_AddMod |
| } |  |  |  |

Table 8.16.37.4.3-3: CQI-ReportConfig-DEFAULT: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  | CQI_PERIODIC |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 158 |  |  |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.37.4.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional E-UTRAN FDD 3DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: 36.508 clause 7.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | Not present |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  | SCell1 |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.37.4.3-5: CQI-ReportConfigSCell-r10-DEFAULT: Additional E-UTRAN FDD 3DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.16.37.4.3-6: CQI-ReportPeriodic-r10-DEFAULT: Additional E-UTRAN FDD 3DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 0 |  |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | TRUE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.37.4.3-7: MeasObjectEUTRA-GENERIC: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: 36.508 clause 4.6.6 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| cellsToAddModList SEQUENCE (SIZE (1..maxCellMeas)) { } | Not present |  |  |
| measCycleSCell-r10 | sf320 |  | Cell 2, Cell 3 |
| } |  |  |  |

Table 8.16.37.4.3-8: MeasObjectEUTRA-GENERIC: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: TS 36.508 [7] clause 4.6.6, Table 4.6.6-2 MeasObjectEUTRA-GENERIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3 |
| } |  |  |  |

Table 8.16.37.4.3-9: QuantityConfig-DEFAULT: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: TS 36.508 clause 4.6.6, Table 4.6.6-3A: QuantityConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.16.37.4.3-10: SystemInformationBlockType2: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-1 SystemInformationBlockType2 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| mbsfn-SubframeConfig | Not present |  | Cell 1 |
| } |  |  |  |

Table 8.16.37.4.3-11: SystemInformationBlockType3: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-2 SystemInformationBlockType3 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3 |
| } |  |  |  |

Table 8.16.37.4.3-12: SystemInformationBlockType5: Additional E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Derivation Path: TS 36.508 [7] clause 7.2.1.3, Table 7.2.1.3-3 SystemInformationBlockType5 exceptions |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| neighCellConfig | ‘00’B (Not all neighbour cells have the same MBSFN subframe allocation as serving cell) |  | Cell 1, Cell 2, Cell 3 |
| } |  |  |  |

#### 8.16.77.5	Test requirement

Table 8.16.77.5-1 defines the primary level settings including [test tolerances for 5DL PCell in FDD CA activation and deactivation of Known SCell in non-DRX test.

Table 8.16.77.5-1: Cell specific test parameters for E-UTRAN FDD 5 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T3 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), outside the subframes (m+25) to (m+29), and outside the subframes (m+35) to (m+39).

Figure 8.16.77.5-1 to 8.16.77.5-3  shows the derivation of the Test procedure requirements for DTX during T2, based on the core requirements for interruption. The applicable figure is selected according to the CA configuration.

NOTE: The differences between activation and deactivation are as follows:

-	Activation  Deactivation

-	First CSI  Last CSI

-	Latest valid CSI report  Not exist (no need to check since CSI report was already stopped)

Figure 8.16.77.5-1: Procedure derivation for Activation (all intra-band case)

1)	Activation command for SCell1 (T2 starts)

2)	ACK for MAC-CE for SCell1 activation

3)	Possible interruption period by SCell1 activation

4)	First CSI report timing (could be invalid CQI)

5)	Activation command for SCell2

6)	Possible DTX reception period on SS due to interruption by SCell1 activation

7)	ACK for MAC-CE for SCell2 activation

8)	Possible interruption period by SCell2 activation

9) Activation command for SCell3

10) Possible DTX reception period on SS due to interruption by SCell2 activation

11) ACK for MAC-CE for SCell3 activation

12) Possible interruption period by SCell3 activation

13) Activation command for SCell4

14) Possible DTX reception period on SS due to interruption by SCell3 activation

15) ACK for MAC-CE for SCell4 activation

16) Possible interruption period by SCell4 activation

17) Latest valid CSI report timing (valid CQI)

18)	Possible DTX reception period on SS due to interruption by SCell4 activation

Figure 8.16.77.5-2 Procedure derivation for Activation
(the case where the band relation between PCC and SCells are inter-band)

- Activation command for SCell1 (T2 starts)

- ACK for MAC-CE for SCell1 activation

- Possible interruption period by SCell1 activation

- Allowed interruption timing (1 subframe in m+5~m+9) on UE by SCell1 activation,
that is, possible DTX timing on SS

- First CSI report timing (could be invalid CQI) at m+8 (might be m+10 due to interruption)

- Activation command for SCell2

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell1 activation

- Possible DTX reception period on SS due to interruption by SCell1 activation

- ACK for MAC-CE for SCell2 activation

- Possible interruption period by SCell2 activation

- Allowed interruption timing (1 subframe in m+15~m+19) on UE by SCell2 activation,
that is, possible DTX timing on SS

- Activation command for SCell3

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell2 activation

- Possible DTX reception period on SS due to interruption by SCell2 activation

- ACK for MAC-CE for SCell3 activation

- Possible interruption period by SCell3 activation

- Allowed interruption timing (1 subframe in m+25~m+29) on UE by SCell3 activation,
that is, possible DTX timing on SS

- Activation command for SCell4

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell3 activation

- Possible DTX reception period on SS due to interruption by SCell3 activation

- ACK for MAC-CE for SCell4 activation

- Possible interruption period by SCell4 activation

- Allowed interruption timing (1 subframe in m+35~m+39) on UE by SCell4 activation,
that is, possible DTX timing on SS

- Latest valid CSI report timing (valid CQI)

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell4 activation

- Possible DTX reception period on SS due to interruption by SCell4 activation

Figure 8.16.77.5-3: Procedure derivation for Activation (The case where the band relation between PCC and SCell1 is intra-band, SCell2, SCell3 and SCell4 is inter-band)

- Activation command for SCell1 (T2 starts)

- ACK for MAC-CE for SCell1 activation

- Possible interruption period by SCell1 activation

- First CSI report timing (could be invalid CQI)

- Activation command for SCell2

- Possible DTX reception period on SS due to interruption by SCell1 activation

- ACK for MAC-CE for SCell2 activation

- Possible interruption period by SCell2 activation

- Allowed interruption timing (1 subframe in m+15~m+19) on UE by SCell2 activation,
that is, possible DTX timing on SS

- Activation command for SCell3

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell2 activation

- Possible DTX reception period on SS due to interruption by SCell2 activation

- ACK for MAC-CE for SCell3 activation

- Possible interruption period by SCell3 activation

- Allowed interruption timing (1 subframe in m+25~m+29) on UE by SCell3 activation,
that is, possible DTX timing on SS

- Activation command for SCell4

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell3 activation

- Possible DTX reception period on SS due to interruption by SCell3 activation

- ACK for MAC-CE for SCell4 activation

- Possible interruption period by SCell4 activation

- Allowed interruption timing (1 subframe in m+35~m+39) on UE by SCell4 activation,
that is, possible DTX timing on SS

- Latest valid CSI report timing (valid CQI)

- Corresponding DTX timing on PUCCH by allowed interruption timing by SCell4 activation

- Possible DTX reception period on SS due to interruption by SCell4 activation

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19) , outside the subframes (n+25) to (n+29) and outside the subframes (n+35) to (n+39).

The derivation of the Test procedure requirements for DTX during T3 is based on the core requirements for interruption, using the applicable figure according to the CA configuration, in a similar way to T2.

All of the requirements defined in the Test Procedure shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90% with a confidence level of 95% for each of the events.

The statistical pass/ fail decisions are done separately for activation and deactivation.

Decide the test pass, if activation and deactivation are passed, otherwise fail the UE.

< End of changes >