3GPP TSG-RAN WG5 Meeting #77 	R5-176062r1

# Reno, USA, 27th Nov – 1st Dec 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1082 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add applicability for eCall over IMS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EIEI-UEConTest |  |  |  |  |  |  | Date: |  | 2017-11-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Agreed CR R5-174540 added applicability statement and UE service capability for new eCall over IMS TCs 11.3.1 and 11.3.2 but was missed to be implemented. Current CR adds applicability for TC 11.3.1, 11.3.2, 11.3.3, 11.3.5, 11.3.6,11.3.7 and 11.3.8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added test applicability for test cases 11.3.1, 11.3.2, 11.3.3, 11.3.5, 11.3.6,11.3.7 and 11.3.8. - Added new test condition Cxxx, Czzz and Ckkk. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS eCall test cases applicability will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.523-1 CR4142, CR4181, CR4250, CR4251, CR4252, CR4253, CR4254 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact Revision1: - Added new test conditions Cttt, Cggg and Cwww - Updated UTRA, GERAN, IMS Emergency, Test eCall capability in the conditions and comments. Note: Cxxx, Czzz, Cttt, Cggg, Cwww and Ckkk needs to be assigned. |  |  |  |  |  |  |  |  |

# 4	Recommended Test Case Applicability

<Text Skipped>

<Start of first modified section>

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability |  | Additional Information |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |

Skipped Items

| 11.2.10 | LIMITED-SERVICE / EPS does not support IMS Emergency / Emergency call using the CS domain | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.11 | LIMITED-SERVICE / Inter-system mobility / E-UTRA to UTRA CS / SRVCC Emergency Call Handover to UTRAN | Rel-9 | C139 | UEs supporting E-UTRA and UTRA and SRVCC and IMS emergency call and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.12 | LIMITED-SERVICE / Inter-system mobility / E-UTRA to GSM CS / SRVCC Emergency Call Handover to GERAN | Rel-9 | C231 | UEs supporting E-UTRA and GERAN and SRVCC and IMS emergency call and NOT Category M1 | pc_eFDD |  |  |  |
| 11.3 | eCall over IMS |  |  |  |  |  |  |  |
| 11.3.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS | Rel-14 | Cxxx | UEs supporting E-UTRA and IMS eCall and eCall only and Manual type of eCall initiation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service | Rel-14 | Cttt | UEs supporting E-UTRA and IMS eCall and eCall only and Manual type of eCall initiation and capable of triggering a Test eCall | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.3 | eCall capable / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA or GERAN | Rel-14 | Czzz | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall and eCall Capable and Automatic type of eCall initiation and IMS emergency call | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.5 | eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is supported / RACH failure in EUTRA cell / eCall using the CS domain | Rel-14 | Ckkk | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall and eCall only and Automatic type of eCall initiation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.6 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted | Rel-14 | Cttt | UEs supporting E-UTRA and IMS eCall and eCall only and Manual type of eCall initiation and capable of triggering a Test eCall | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success | Rel-14 | Cggg | UEs supporting E-UTRA and UTRA and IMS eCall and eCall only and Manual type of eCall initiation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.8 | eCall Only mode / SRVCC Handover to CS domain / GERAN / MSD Update / Success | Rel-14 | Cwww | UEs supporting E-UTRA and GERAN and IMS eCall and eCall only and Manual type of eCall initiation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |

<End of First modified section>

<Start of second modified section>

Table 4-1a: Applicability of tests Conditions

C285 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/132 THEN R ELSE N/A

C286 IF(A.4.1-1/1 OR A.4.1-1/2) AND NOT (A.4.3.2-2A/1) AND A.4.4-1/2 AND A.4.4-2/1 THEN R ELSE N/A

C287 IF(A.4.1-1/1 OR A.4.1-1/2) AND NOT (A.4.3.2-2A/1) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 THEN R ELSE N/A

C288 IF A.4.1-1/8 AND A.4.4-1/132 AND A.4.4-1/136 THEN R ELSE N/A

C289 IF A.4.1-1/8 AND A.4.4-1/132 AND A.4.4-1/137 THEN R ELSE N/A

Cxxx IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

Cttt IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 AND [8]A.10/19 THEN R ELSE N/A

Czzz IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/54 AND [8]A.10/17 AND A.4.2.1.1-1/4 THEN R ELSE N/A

Ckkk IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A

Cggg IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

Cwww IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

<End of second modified section>