3GPP TSG-RAN-WG5 Meeting #77 	R5-176265r1

# Reno, USA, 27 November 2017 – 1 December 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | PCR | NA | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCPTT TC 6.3.1 On-network / Location / Event Triggered Location Information Report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet (revision 1 by U.S. Department of Commerce) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n/a |  |  |  |  |  |  |  |  |  |
| Work item code: | MCPTT-ConTest |  |  |  |  |  |  | Date: |  | 2017-11-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of a new TC according to the WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a new TC according to the WP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT functionality will not be verified and "bad" devices can enter the market and disrupt the MCPTT services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.3.1	On-network / One MCPTT System / Location / Event Triggered Location Information report

### 6.3.1.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client)registered and authorized for MCPTT service and not in an emergency state }

ensure that {
  when { UE (MCPTT Client) receives a location information configuration message with a PLMN change trigger and the UE (MCPTT Client) moves to a different PLMN }

then { UE (MCPTT Client) sends the PLMN changed triggered location information report by sending a SIP MESSAGE message }

}

(2)

with { UE (MCPTT Client)registered and authorized for MCPTT service and not in an emergency state }

ensure that {
  when { UE (MCPTT Client) receives a location information configuration message with a ConfigureScope equal to "Full" and with a Periodic trigger element }

then { UE (MCPTT Client) sends the Periodic triggered location information report by sending a SIP MESSAGE message and UE (MCPTT Client) sends the subsequent Periodic triggered location information report by sending a SIP MESSAGE message only after minimumIntervalLength for NonEmergencyLocationInformation has expired }

}

(3)

with { UE (MCPTT Client) registered and authorized for MCPTT service }

ensure that {

when { UE (MCPTT Client) receives a location information configuration message with a ConfigureScope equal to "Update" and with a Cell Change trigger element and the UE (MCPTT Clien) moves to a different cell }

then { UE (MCPTT Client) sends the cell change triggered location information report by sending a SIP MESSAGE and UE (MCPTT Client) sends the Periodic triggered location information report by sending a separate SIP MESSAGE message only after minimumIntervalLength for NonEmergencyLocationInformation has expired }

}

6.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clause 13.3.4.1, 13.3.4.2. Unless otherwise stated, these are Rel-13 requirements.

[TS24.379 clause 13.3.4.1]

If a location reporting trigger fires the MCPTT client checks if the minimumReportInterval timer is running. If the timer is running the MCPTT client waits until the timer expires. When the minimumReportInterval timer fires, the MCPTT client:

1)	shall, if any of the reporting triggers are still true, send a location information report as specified in subclause 13.3.4.2.

If the MCPTT client receives a location information request as specified in subclause 13.3.3, the MCPTT client shall send a location report as specified in subclause 13.3.4.2

[TS24.379 clause 13.3.4.2]

If the MCPTT client needs to send a SIP request for other reasons (e.g. a SIP MESSAGE request containing an MBMS listening report as described in clause 14), the MCPTT client:

1)	 shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body and in the <location-info> root element the MCPTT client shall include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	<TriggerId> child elements, where each element is set to the value of the <Trigger-Id> attribute associated with the trigger that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired;

2)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer; and

3)	shall reset all triggers.

If the MCPTT client does not need to send a SIP request for other reasons, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. The MCPTT client;

1)	shall include in the Request-URI, the SIP URI received in the P-Asserted-Identity header field in the received SIP MESSAGE request for location report configuration;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml";

3)	shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body and in the <location-info> root element include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	a <TriggerId> child element set to the value of each <Trigger-Id> value of the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcptt along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [6];

5)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer;

6)	shall reset all triggers; and

7)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4].

6.3.1.3	Test description

6.3.1.3.1	Pre-test conditions

System Simulator

-	SS (MCPTT server)

UE:

-	The MCPTT client is authorised for MCPTT service

-	One MCPTT user is registered for receiving MCPTT service through the MCPTT Client

-	The MCPTT client is not in an emergency state

-	Other pre-settings

-	In the Service Configuration document as specified in 3GPP TS 24.484 [13], in the <signalling-protection> element

-	The <confidentiality-protection> element is set to "true"

-	The <integrity-protection> element is set to "true"

Editor's Note:	For facilitating the Test Case TTCN initial verification the <confidentiality-protection> element may be temporarily set, in the TTCN code, to "false"; when initial verification is successfully achieved then it shall be set to "true".

-	The MCPTT UE has been provisioned with key material associated with the user's MCPTT ID as specified in TS 36.579-1 [2], clause 5.6.2.

Table 6.2.1.3.1-1: USIM configuration

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| TBD |  |  |  |  |

Preamble

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

6.3.1.3.2	Test procedure sequence

Table 6.3.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCPTT call establishment are described in TS 56.579-1 [2], subclause 5.4.4 'Generic Test Procedure for MCPTT CT communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 1 | The SS sends a location reporting configuration containing the PLMN change trigger criteria with full configuration scope element and the minimum report interval to the UE (MCPTT Client) | <--- | SIP MESSAGE | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the PLMN change are described in TS 56.579-1 [2], subclause X.X.X 'Generic Test Procedure for PLMN Change in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE containing the PLMN change trigger identification along with the location information report as specified in step 1? | ---> | SIP MESSAGE | 1 | P |
| 3 | The SS sends a location reporting configuration containing the periodic trigger criteria with full configuration scope element and the periodic reporting interval to the UE (MCPTT Client) | <--- | SIP MESSAGE | - | - |
| 4 | Check: Does the UE (MCPTT Client) send the first SIP MESSAGE containing the periodic trigger identification along with location information report? | ---> | SIP MESSAGE | 2 | P |
| 5 | SS starts timer=10 sec (the non-emergency minimum interval length defined in the SIP MESSAGE message sent in step 3) | - | - | - | - |
| 6 | Timer=10 sec expires | - | - | - | - |
| 7 | Check: Does the UE (MCPTT Client) send the second SIP MESSAGE containing the periodic trigger identification along with location information report after the non-emergency minimum interval length expired? | ---> | SIP MESSAGE | 2 | P |
| 8 | The SS sends a location reporting configuration containing the cell change trigger criteria with update configuration scope element and the minimum report interval to the UE (MCPTT Client) | <--- | SIP MESSAGE | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the cell change are described in TS 56.579-1 [2], subclause X.X.X 'Generic Test Procedure for Cell Change in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 9 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE containing the cell change trigger identification along with the location information report as specified in step 8? | ---> | SIP MESSAGE | 3 | P |
| 10 | SS starts timer=10 sec (the non-emergency minimum interval length defined in the SIP MESSAGE message sent in step 8) | - | - | - | - |
| 11 | Timer=10 sec expires | - | - | - | - |
| 12 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE containing the periodic trigger identification along with location information report after the non-emergency minimum interval length expired?? | ---> | SIP MESSAGE | 3 | P |
| - | EXCEPTION: SS releases the E-UTRA connection. | - | - | - | - |

6.3.1.3.3	Specific message contents

Table 6.3.1.3.3-1: SIP MESSAGE (Step 1, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed;boundary="any allowed value |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-info+xml" |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | "application/vnd.3gpp.mcptt-affiliation-command+xml" | TS 24.379 [9] clause F.3 |  |
| Location-Info | As described in Table 6.3.1.3.3-2 |  |  |  |

Table 6.3.1.3.3-2: Location-Info in SIP MESSAGE (Table 6.3.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.22.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Configuration | present |  |  |  |
| NonEmergencyLocationInformation | present |  |  |  |
| ServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| GeographicalCoordinate | present |  |  |  |
| minimumIntervalLength | "10" |  |  |  |
| EmergencyLocationInformation" | present |  |  |  |
| ServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| GeographicalCoordinate | present |  |  |  |
| minimumIntervalLength | "5" |  |  |  |
| TriggeringCriteria | present |  |  |  |
| PlmnChange | present |  |  |  |
| AnyPlmnChange | present | An optional element - the presence of this element specifies that any PLMN change is a trigger |  |  |
| TriggerID | "ANY PLMN" | Mandatory if AnyPlmnChange is present – shall be set to a unique string |  |  |

Table 6.3.1.3.3-3: SIP MESSAGE (Step 2, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed;boundary="any allowed value |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-info+xml" |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | "application/vnd.3gpp.mcptt-affiliation-command+xml" | TS 24.379 [9] clause F.3 |  |
| Location-Info | As described in Table 6.3.1.3.3-4 |  |  |  |

Table 6.3.1.3.3-4: Location-Info in SIP MESSAGE (Table 6.3.1.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.22.4.1-1 condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Report | present |  |  |  |
| TriggerID | "ANY PLMN" |  |  |  |
| CurrentLocation | present |  |  |  |
| CurrentServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| CurrentCoordinate | present |  |  |  |
| ReportType | "NonEmergency" |  |  |  |

Table 6.3.1.3.3-5: SIP MESSAGE (Step 3, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed;boundary="any allowed value |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-info+xml" |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | "application/vnd.3gpp.mcptt-affiliation-command+xml" | TS 24.379 [9] clause F.3 |  |
| Location-Info | As described in Table 6.3.1.3.3-6 |  |  |  |

Table 6.3.1.3.3-6: Location-Info in SIP MESSAGE (Table 6.3.1.3.3-5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.22.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Configuration | present |  |  |  |
| NonEmergencyLocationInformation | present |  |  |  |
| ServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| GeographicalCoordinate | present |  |  |  |
| minimumIntervalLength | "10" | value in seconds |  |  |
| EmergencyLocationInformation" | present |  |  |  |
| ServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| GeographicalCoordinate | present |  |  |  |
| minimumIntervalLength | "5" |  |  |  |
| TriggeringCriteria | present |  |  |  |
| PeriodicReport | "9" | value in seconds |  |  |
| TiggerID | "PERIODIC" |  |  |  |

Table 6.3.1.3.3-7: SIP MESSAGE (Steps 4, 7, 12, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed;boundary="any allowed value |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-info+xml" |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | "application/vnd.3gpp.mcptt-affiliation-command+xml" | TS 24.379 [9] clause F.3 |  |
| Location-Info | As described in Table 6.3.1.3.3-8 |  |  |  |

Table 6.3.1.3.3-8: Location-Info in SIP MESSAGE (Table 6.3.1.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.22.4.1-1 condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Report | present |  |  |  |
| TriggerID | "PERIODIC" |  |  |  |
| CurrentLocation | present |  |  |  |
| CurrentServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| CurrentCoordinate | present |  |  |  |
| ReportType | "NonEmergency" |  |  |  |

Table 6.3.1.3.3-9: SIP MESSAGE (Step 8, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed;boundary="any allowed value |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-info+xml" |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | "application/vnd.3gpp.mcptt-affiliation-command+xml" | TS 24.379 [9] clause F.3 |  |
| Location-Info | As described in Table 6.3.1.3.3-10 |  |  |  |

Table 6.3.1.3.3-10: Location-Info in SIP MESSAGE (Table 6.3.1.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.22.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Configuration | present |  |  |  |
| ConfigureScope | "Update" |  |  |  |
| NonEmergencyLocationInformation | present |  |  |  |
| ServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| GeographicalCoordinate | present |  |  |  |
| minimumIntervalLength | "10" |  |  |  |
| EmergencyLocationInformation" | present |  |  |  |
| ServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| GeographicalCoordinate | present |  |  |  |
| minimumIntervalLength | "5" |  |  |  |
| TriggeringCriteria | present |  |  |  |
| CellChange | present |  |  |  |
| AnyCellChange | present |  |  |  |
| TriggerID | "CELLCHANGE" |  |  |  |

Table 6.3.1.3.3-11: SIP MESSAGE (Step 9, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed;boundary="any allowed value |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-info+xml" |  |  |
| MIME-Content-Type | not present | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | "application/vnd.3gpp.mcptt-affiliation-command+xml" | TS 24.379 [9] clause F.3 |  |
| Location-Info | As described in Table 6.3.1.3.3-12 |  |  |  |

Table 6.3.1.3.3-12: Location-Info in SIP MESSAGE (Table 6.3.1.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.22.4.1-1 condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Report | present |  |  |  |
| TriggerID | "CELLCHANGE" |  |  |  |
| CurrentLocation | present |  |  |  |
| CurrentServingEcgi | present |  |  |  |
| NeighbouringEcgi | present |  |  |  |
| MbmsSaId | present |  |  |  |
| MbsfnArea | present |  |  |  |
| CurrentCoordinate | present |  |  |  |
| ReportType | "NonEmergency" |  |  |  |