3GPP TSG RAN WG5 Meeting #79  	R5-183921r1

# Busan, South Korea, 21 - 25, May 2018

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.521-1 | CR | N/A | rev | N/A | Current version: | 0.4.0 |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of test requirement for 6.2.2 Maximum Power Reduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-06-29 |
| Category: | N/A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Table format in 6.2.2.5 need to follow what was done in A-MPR test in LTE instead of MPR test in LTE. - RP-181262(R4-1808530) has updated the requirements for 6.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.2.2.3 and test requirement tables in 6.2.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Content in test requirement tables in 6.2.2.5 is not detail enough. - Content in 6.2.2.3 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Baseline: RP-181262. Revised from R5-182881r2. - Removed TBD from Initial Conditions, Test Procedure, and Message Contents. - Decided on how to deal with TDD slots with transient periods in test procedure. R5-183921r1: - Update the minimum requirements according to RP-181262(R4-1808530). - Update Editor’s note to list all open issues. - Remove brackets for DCI format in test procedures. - Number of test points is equal to ACLR. - Update test requirements according to new test points and RP-181262(R4-1808530). - Update TT values according to R5-183952. |  |  |  |  |  |  |  |  |

<Start of modified section>

6.2.2	Maximum Power Reduction (MPR)

Editor’s Note:

- Starting time position of 200ms in test procedure 2 is FFS.

- Test frequency, test SCS definition in TS 38.508-1 is FFS.

- Definition of system information in TS 38.508-1 is FFS.

- Generic procedures with condition NR in TS 38.508-1 is FFS.

- Propagation conditions is TBD.

- Uplink Physical channel is FFS

- Downlink Physical Channels is RAN4 pending

- UL RMC is RAN4 pending.

- PC2 MPR is RAN4 pending.

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

The requirements of this test apply in test cases 6.5.2.2.1 Adjacent Channel Leakage power Ratio to all types of NR Power Class [2] and 3 UE release 15 and forward

6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE Power Class [2] and 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-2 and 6.2.2.3-1, respectively for channel bandwidths that meets both following criteria:

•	Channel bandwidth ≤ 100MHz.

•	Relative channel bandwidth ≤ 4% for TDD bands and ≤ 3% for FDD bands.

Where relative channel bandwith = 2*BWChannel / (FUL_low  + FUL_high).

Table 6.2.2.3-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

Table 6.2.2.3-2: Maximum Power Reduction (MPR) for Power Class 2

| Modulation | MPR (dB) |  |  |
| --- | --- | --- | --- |
|  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = LRB – RBStart,Low – LCRB

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

For UE Power Class 2, an Edge RB allocation is one for which the RB’s are allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RB’s.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex [TBD].

Table 6.2.2.4.1-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in TS 38.508-1 [5] subclause [TBD] |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Low | Reduction (MPR) test case | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 8 | Low |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 9 | High |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 10 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 11 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 12 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 13 | Default |  | CP-OFDM QPSK | Inner Full |
| 14 | Low |  | CP-OFDM QPSK | Outer_1RB_Left |
| 15 | High |  | CP-OFDM QPSK | Outer_1RB_Right |
| 16 | Default |  | CP-OFDM QPSK | Outer Full |
| 17 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 18 | Low |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 19 | High |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 20 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 21 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 22 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |

Table 6.2.2.4.1-2: Test Configuration Table for Power Class 2

TBD

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to [TBD], and uplink signals according to [TBD].

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to [TBD].

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD slots with transient periods are not under test.

NOTE 1: When switching to CP-OFDM waveform, as specified in the test configuration table 6.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table [4.6.3-n] PUSCH-Config without DFT-s-OFDM condition. When switching to DFT-s-OFDM waveform, send an NR RRCReconfiguration message with DFT-s-OFDM condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 and Table 6.2.2.5-4.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n8, n12, n25, n34, n39, n40, n41, n66, n70, n80, n81, n82, n84, n86) for Power Class 3

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 2 | 23 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 3 | 23 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 4 | 23 | 1 | 0 | 22 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 5 | 23 | 1 | 0 | 22 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 1 | 0 | 22 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 1 | 0 | 22 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 2 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 9 | 23 | 2 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 10 | 23 | 2 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 12 | 23 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 13 | 23 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 3 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 15 | 23 | 3 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 16 | 23 | 3 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 17 | 23 | 2 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 18 | 23 | 3 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 3 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 3 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 21 | 23 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 22 | 23 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: TT=0.7dB for BW ≤ 40MHz; TT=[1.0] for 40MHz < BW ≤ 100MHz. |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements(for Bands n71, n83) for Power Class 3

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 2 | 23 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 3 | 23 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 4 | 23 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 5 | 23 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 6 | 23 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 2 | 0 | 21 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 9 | 23 | 2 | 0 | 21 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 10 | 23 | 2 | 0 | 21 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 11 | 23 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 12 | 23 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 13 | 23 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 15 | 23 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 16 | 23 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 17 | 23 | 2 | 0 | 21 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 18 | 23 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 21 | 23 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 22 | 23 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7dB. |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements(for Bands n77, n78, n79) for Power Class 3

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 2 | 23 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 3 | 23 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 4 | 23 | 1 | 0 | 22 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 5 | 23 | 1 | 0 | 22 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 6 | 23 | 1 | 0 | 22 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 7 | 23 | 1 | 0 | 22 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 8 | 23 | 2 | 0 | 21 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 9 | 23 | 2 | 0 | 21 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 10 | 23 | 2 | 0 | 21 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 11 | 23 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 12 | 23 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 13 | 23 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 14 | 23 | 3 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 15 | 23 | 3 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 16 | 23 | 3 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 17 | 23 | 2 | 0 | 21 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 3 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 19 | 23 | 3 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 20 | 23 | 3 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 21 | 23 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 22 | 23 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=1.0dB for Bands n77 and n78 when BW ≤ 40MHz; TT=[1.0] for Band n79 when BW ≤ 40MHz; TT=[1.0] for 40MHz < BW ≤ 100MHz. |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements(for Bands n41, n77, n78, n79) for Power Class 2

TBD

<End of modified section>