3GPP TSG-RAN5 Meeting #81 	R5-186443r1

# Spokane, USA, 12 -16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4514 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.6.2.1A.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.1A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Original text updated to version 15.3.1 (previously 15.2.0). |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.6.2.1A.2	Spectrum emission mask for CA (inter-band DL CA and UL CA)

6.6.2.1A.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

6.6.2.1A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.2.4	Test description

6.6.2.1A.2.4.1	Initial condition

Same as in clause 6.6.2.1A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.6.2.1A.1.4.1-1  use Table 6.6.2.1A.2.4.1-1.

-	Instead of clause 6.6.2.1A.1.4.3 use clause 6.6.2.1A.2.4.3.

Table 6.6.2.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@1 | S_8@17 | - | - |
| 6 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 |  |  |
| 6 | 25 |  | 16QAM | 2 | P_1@5 | S_1@24 |  |  |
| 6 | 25 |  | 16QAM | 13 | P_5@0 | S_8@0 |  |  |
| 6 | 25 |  | 16QAM | 13 | P_5@1 | S_8@17 |  |  |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@1 | S_12@38 | - | - |
| 6 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | 16QAM | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | 16QAM | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | 16QAM | 17 | P_5@1 | S_12@38 | - | - |
| 15 | 25 |  | QPSK | 2 | S_1@0 | S_1@0 |  |  |
| 15 | 25 |  | QPSK | 2 | S_1@14 | S_1@24 |  |  |
| 15 | 25 |  | QPSK | 12 | S_4@0 | S_8@0 |  |  |
| 15 | 25 |  | QPSK | 12 | S_4@11 | S_8@17 |  |  |
| 15 | 25 |  | 16QAM | 2 | S_1@0 | S_1@0 |  |  |
| 15 | 25 |  | 16QAM | 2 | S_1@14 | S_1@24 |  |  |
| 15 | 25 |  | 16QAM | 12 | S_4@0 | S_8@0 |  |  |
| 15 | 25 |  | 16QAM | 12 | S_4@11 | S_8@17 |  |  |
| 25 | 6 |  | QPSK | 2 | S_1@0 | P_1@0 |  |  |
| 25 | 6 |  | QPSK | 2 | S_1@24 | P_1@5 |  |  |
| 25 | 6 |  | QPSK | 13 | S_8@0 | P_5@0 |  |  |
| 25 | 6 |  | QPSK | 13 | S_8@17 | P_5@1 |  |  |
| 25 | 6 |  | 16QAM | 2 | S_1@0 | P_1@0 |  |  |
| 25 | 6 |  | 16QAM | 2 | S_1@24 | P_1@5 |  |  |
| 25 | 6 |  | 16QAM | 13 | S_8@0 | P_5@0 |  |  |
| 25 | 6 |  | 16QAM | 13 | S_8@17 | P_5@1 |  |  |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 15 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | 16QAM | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | 16QAM | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | 16QAM | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | 16QAM | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | 16QAM | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | 16QAM | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@17 | S_12@38 | - | - |
| 25 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | 16QAM | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | 16QAM | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | 16QAM | 20 | P_8@17 | S_12@38 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@38 | S_12@38 | - | - |
| 50 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | 16QAM | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | 16QAM | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | 16QAM | 24 | P_12@38 | S_12@38 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@38 | S_18@82 | - | - |
| 50 | 100 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | 16QAM | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | 16QAM | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | 16QAM | 30 | P_12@38 | S_18@82 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | 16QAM | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | 16QAM | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | 16QAM | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@59 | S_16@59 | - | - |
| 75 | 75 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | 16QAM | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | 16QAM | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | 16QAM | 32 | P_16@59 | S_16@59 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 |  |  |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 |  |  |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 |  |  |
| 100 | 50 |  | QPSK | 30 | P_18@82 | S_12@38 |  |  |
| 100 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 |  |  |
| 100 | 50 |  | 16QAM | 2 | P_1@99 | S_1@49 |  |  |
| 100 | 50 |  | 16QAM | 30 | P_18@0 | S_12@0 |  |  |
| 100 | 50 |  | 16QAM | 30 | P_18@82 | S_12@38 |  |  |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 75 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | 16QAM | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | 16QAM | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | 16QAM | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@82 | S_18@82 | - | - |
| 100 | 100 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | 16QAM | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | 16QAM | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | 16QAM | 36 | P_18@82 | S_18@82 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

6.6.2.1A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power for each CC, which shall meet the requirements described in Table 6.2.3A.2.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC with a measurement filter of bandwidths according to table 6.6.2.1A.2.5-1 or 6.6.2.1A.2.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC with a measurement filter of bandwidths according to table 6.6.2.1A.2.5-1 or 6.6.2.1A.2.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.2.5	Test Requirements

Editor's note:	Need further investigation for test requirements in case some frequency spectrum emission masks of component carriers overlap for future coming inter-band UL CA combinations

The measured UE mean power in the channel bandwidth for each CC, derived in step 6, shall fulfil requirements in Table 6.2.3A.2.5-1 as appropriate, and the power of any UE emission per component carrier derived in step 7 and 8 shall fulfil requirements in Table.6.6.2.1A.2.5-1 or Table.6.6.2.1A.2.5-2, as applicable.

Table 6.6.2.1A.2.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1A.2.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

< End of changes >