3GPP TSG-RAN5 Meeting #81	R5-186444r1

# Spokane, USA, 12-16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4515 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.6.2.2A.2_1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Original text updated to version 15.3.1 (previously 15.2.0). |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.6.2.2A.2	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA)

6.6.2.2A.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

6.6.2.2A.2.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2.3 per component carrier.

6.6.2.2A.2.4	Test description

6.6.2.2A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2.4.1-1: Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-3A CA_1A-5A CA_1A-7A CA_1A-8A CA_1A-18A CA_1A-26A CA_1A-42A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_1A-28A | NS_05 | NS_17 | N/A | N/A |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_3A-5A CA_3A-7A CA_3A-8A CA_5A-7A CA_39A-41A CA_41A-42A | NS_01 | NS_01 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_3A-20A CA_7A-20A | NS_01 | NS_10 | N/A | N/A |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote 5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_5A-12A CA_5A-17A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_7A-28A CA_18A-28A | NS_01 | NS_17 | N/A | N/A |
| CA_7A-28A | NS_01 | NS_18 | N/A | N/A |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_12A-66A | NS_06 | NS_03 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.1-1 |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2.4.1-2 to 6.6.2.2A.1.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.2.4.1-2: Test Configuration Table (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 16QAM | 16QAM | 50 | 100 |
| 9 | 50 | 100 |  | 16QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 12 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 15 | 100 | 100 |  | 16QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.6.2.2A.2.4.1-1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-3: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 9 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-4: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-5: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 12 | 100 | 100 |  | 16QAM | QPSK | 100 | 10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-6: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 16QAM | 16QAM | 25 | 15 |
| 6 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 12 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 15 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 18 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| X+1 | 75 | 25 |  | QPSK | QPSK | 8 | 8 |
| X+2 | 75 | 25 |  | 16QAM | 16QAM | 75 | 25 |
| X+3 | 75 | 25 |  | 16QAM | QPSK | 75 | 8 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-7: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 7@13 |
| 2 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 7@13 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 7@13 |
| 8 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 16QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 16QAM | 16QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 16QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-9: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.2.4.3.

6.6.2.2A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Tables 6.2.4A.2.5-2 to 6.2.4A.2.5-12. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC if NS-value other than NS_01 with a measurement filter of bandwidths according to tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4 as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC if NS-value other than NS_01 with a measurement filter of bandwidths according to tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4 as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value. These exceptions indicate that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

6.6.2.2A.2.4.3.1	Message contents exceptions (network signalled value " NS_03" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.6.2.2A.2.4.3.2	Message contents exceptions (network signalled value " NS_03" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_03 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.2-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (NS_03) |  |  |

6.6.2.2A.2.4.3.3	Message contents exceptions (network signalled value " NS_06" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_06 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.3-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (NS_06) |  |  |

6.6.2.2A.2.4.3.4	Message contents exceptions (network signalled value " NS_07" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_07 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.1.4.3.4-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (NS_07) |  |  |

6.6.2.2A.2.4.3.5	Message contents exceptions (network signalled value " NS_21" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_21 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.5-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 21 (NS_21) |  |  |

6.6.2.2A.2.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Tables 6.2.4A.2.5-1 to 6.2.4A.2.5-12 as appropriate,

and

The power of any UE emission per component carrier derived in step 7 and 8 shall not exceed the levels specified in Tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4.

Table 6.6.2.2A.2.5-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.5-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.5-3: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2A.2.5-4: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

< End of changes >