3GPP TSG-RAN5 Meeting #81 	R5-186446r1

# Spokane, USA, 12-16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4517 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.6.2.3A.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Original text updated to version 15.3.1 (previously 15.2.0). |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.6.2.3A.2	Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA)

6.6.2.3A.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA. The requirements also apply to all types of E-UTRA UE release 14 and forward that support uplink LAA.

6.6.2.3A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.2.4	Test description

6.6.2.3A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 6 | 25 | N/A for this test | QPSK | QPSK | 13 | 5 | 8 |
| 2 | 6 | 25 |  | QPSK | QPSK | 31 | 6 | 25 |
| 3 | 6 | 25 |  | 16QAM | 16QAM | 13 | 5 | 8 |
| 4 | 6 | 25 |  | 16QAM | 16QAM | 31 | 6 | 25 |
| 5 | 6 | 25 |  | QPSK | 16QAM | 30 | 5 | 25 |
| 6 | 6 | 50 |  | QPSK | QPSK | 17 | 5 | 12 |
| 7 | 6 | 50 |  | QPSK | QPSK | 56 | 6 | 50 |
| 8 | 6 | 50 |  | 16QAM | 16QAM | 17 | 5 | 12 |
| 9 | 6 | 50 |  | 16QAM | 16QAM | 56 | 6 | 50 |
| 10 | 6 | 50 |  | QPSK | 16QAM | 55 | 5 | 50 |
| 11 | 25 | 15 |  | QPSK | QPSK | 12 | 8 | 4 |
| 12 | 25 | 15 |  | QPSK | QPSK | 40 | 25 | 15 |
| 13 | 25 | 15 |  | 16QAM | 16QAM | 12 | 8 | 4 |
| 14 | 25 | 15 |  | 16QAM | 16QAM | 40 | 25 | 15 |
| 15 | 25 | 15 |  | QPSK | 16QAM | 23 | 8 | 15 |
| 16 | 25 | 25 |  | QPSK | QPSK | 16 | 8 | 8 |
| 17 | 25 | 25 |  | QPSK | QPSK | 50 | 25 | 25 |
| 18 | 25 | 25 |  | 16QAM | 16QAM | 16 | 8 | 8 |
| 19 | 25 | 25 |  | 16QAM | 16QAM | 50 | 25 | 25 |
| 20 | 25 | 25 |  | QPSK | 16QAM | 33 | 8 | 25 |
| 21 | 25 | 50 |  | QPSK | QPSK | 16 | 4 | 12 |
| 22 | 25 | 50 |  | QPSK | QPSK | 65 | 15 | 50 |
| 23 | 25 | 50 |  | 16QAM | 16QAM | 16 | 4 | 12 |
| 24 | 25 | 50 |  | 16QAM | 16QAM | 65 | 15 | 50 |
| 25 | 25 | 50 |  | QPSK | 16QAM | 54 | 4 | 50 |
| 26 | 50 | 25 |  | QPSK | QPSK | 20 | 12 | 8 |
| 27 | 50 | 25 |  | QPSK | QPSK | 75 | 50 | 25 |
| 28 | 50 | 25 |  | 16QAM | 16QAM | 20 | 12 | 8 |
| 29 | 50 | 25 |  | 16QAM | 16QAM | 75 | 50 | 25 |
| 30 | 50 | 25 |  | QPSK | 16QAM | 37 | 12 | 25 |
| 31 | 50 | 50 |  | QPSK | QPSK | 24 | 12 | 12 |
| 32 | 50 | 50 |  | QPSK | QPSK | 100 | 50 | 50 |
| 33 | 50 | 50 |  | 16QAM | 16QAM | 24 | 12 | 12 |
| 34 | 50 | 50 |  | 16QAM | 16QAM | 100 | 50 | 50 |
| 35 | 50 | 50 |  | QPSK | 16QAM | 62 | 12 | 50 |
| 36 | 50 | 100 |  | QPSK | QPSK | 30 | 12 | 18 |
| 37 | 50 | 100 |  | QPSK | QPSK | 150 | 50 | 100 |
| 38 | 50 | 100 |  | 16QAM | 16QAM | 30 | 12 | 18 |
| 39 | 50 | 100 |  | 16QAM | 16QAM | 150 | 50 | 100 |
| 40 | 50 | 100 |  | QPSK | 16QAM | 112 | 12 | 100 |
| 41 | 75 | 75 |  | QPSK | QPSK | 32 | 16 | 16 |
| 42 | 75 | 75 |  | QPSK | QPSK | 150 | 75 | 75 |
| 43 | 75 | 75 |  | 16QAM | 16QAM | 32 | 16 | 16 |
| 44 | 75 | 75 |  | 16QAM | 16QAM | 150 | 75 | 75 |
| 45 | 75 | 75 |  | QPSK | 16QAM | 91 | 16 | 75 |
| 46 | 100 | 50 |  | QPSK | QPSK | 30 | 18 | 12 |
| 47 | 100 | 50 |  | QPSK | QPSK | 150 | 100 | 50 |
| 48 | 100 | 50 |  | 16QAM | 16QAM | 30 | 18 | 12 |
| 49 | 100 | 50 |  | 16QAM | 16QAM | 150 | 100 | 50 |
| 50 | 100 | 50 |  | QPSK | 16QAM | 68 | 18 | 50 |
| 51 | 100 | 75 |  | QPSK | QPSK | 34 | 18 | 16 |
| 52 | 100 | 75 |  | QPSK | QPSK | 175 | 100 | 75 |
| 53 | 100 | 75 |  | 16QAM | 16QAM | 34 | 18 | 16 |
| 54 | 100 | 75 |  | 16QAM | 16QAM | 175 | 100 | 75 |
| 55 | 100 | 75 |  | QPSK | 16QAM | 93 | 18 | 75 |
| 56 | 100 | 100 |  | QPSK | QPSK | 36 | 18 | 18 |
| 57 | 100 | 100 |  | QPSK | QPSK | 200 | 100 | 100 |
| 58 | 100 | 100 |  | 16QAM | 16QAM | 36 | 18 | 18 |
| 59 | 100 | 100 |  | 16QAM | 16QAM | 200 | 100 | 100 |
| 60 | 100 | 100 |  | QPSK | 16QAM | 118 | 18 | 100 |
| 61 | 15 | 25 |  | QPSK | QPSK | 12 | 4 | 8 |
| 62 | 15 | 25 |  | QPSK | QPSK | 40 | 15 | 25 |
| 63 | 15 | 25 |  | 16QAM | 16QAM | 12 | 4 | 8 |
| 64 | 15 | 25 |  | 16QAM | 16QAM | 40 | 15 | 25 |
| 65 | 15 | 25 |  | QPSK | 16QAM | 40 | 15 | 25 |
| 66 | 25 | 6 |  | QPSK | QPSK | 13 | 8 | 5 |
| 67 | 25 | 6 |  | QPSK | QPSK | 31 | 25 | 6 |
| 68 | 25 | 6 |  | 16QAM | 16QAM | 13 | 8 | 5 |
| 69 | 25 | 6 |  | 16QAM | 16QAM | 31 | 25 | 6 |
| 70 | 25 | 6 |  | QPSK | 16QAM | 40 | 25 | 5 |
| 71 | 25 | 75 |  | QPSK | QPSK | 24 | 8 | 16 |
| 72 | 25 | 75 |  | QPSK | QPSK | 100 | 25 | 75 |
| 73 | 25 | 75 |  | 16QAM | 16QAM | 24 | 8 | 16 |
| 74 | 25 | 75 |  | 16QAM | 16QAM | 100 | 25 | 75 |
| 75 | 25 | 75 |  | QPSK | 16QAM | 100 | 25 | 75 |
| X+1 | 75 | 25 |  | QPSK | QPSK | 24 | 16 | 8 |
| X+2 | 75 | 25 |  | QPSK | QPSK | 100 | 75 | 25 |
| X+3 | 75 | 25 |  | 16QAM | 16QAM | 24 | 16 | 8 |
| X+4 | 75 | 25 |  | 16QAM | 16QAM | 100 | 75 | 25 |
| X+5 | 75 | 25 |  | QPSK | 16QAM | 41 | 16 | 25 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |  |

Table 6.6.2.3A.2.4.1-2: Test Configuration Table for UL LAA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC (Note 2) and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 100 | 100 | N/A for this test | QPSK | QPSK | 11 | 1 | L=1 |
| 2 | 100 | 100 |  | QPSK | QPSK | 108 | 18 | L=9 |
| 3 | 100 | 100 |  | 16QAM | 16QAM | 11 | 1 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 108 | 18 | L=9 |
| 5 | 100 | 100 |  | QPSK | QPSK | 200 | 100 | L=10 |
| 6 | 100 | 100 |  | 16QAM | 16QAM | 200 | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=N-L, with N defined in Clause 8.1.4 of [10]. Note 4: Applicable only for an LAA Scell configured in Band 46. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.2.4.1-1 or Table 6.6.2.3A.2.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.2.4.3.

6.6.2.3A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.2.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 or DCI format 0A for LAA SCC for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.2.4.1-1 or Table 6.6.2.3A.2.4.1-2 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power of the component carrier of the UE in the first band of the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.2.5-1 or Table 6.2.3A.2.5-2. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA in the first band of the CA configuration.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

12. Repeat steps 6 to 11 for the second band of the CA configuration.

6.6.2.3A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508[7] subclause4.6 with the following exceptions specified in Tables 6.6.2.3A.1.2.4.3-1, 6.6.2.3A.1.2.4.3-2 and 6.6.2.3A.1.2.4.3-3.

Table 6.6.2.3A.1.2.4.3-1 LogicalChannelConfig-DRB (Initial Condition, step 14, Table 4.5.2.3-1)

| Derivation Path: 36.508 Table 4.8.2.1.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LogicalChannelConfig-DRB ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| laa-UL-Allowed-r14 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.2.3A.1.2.4.3-2 PhysicalConfigDedicatedSCell (Step 2, test procedure)

| Derivation Path: 36.508 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| laa-SCellConfiguration-v1430 ::= SEQUENCE { |  |  |  |
|  | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.3A.1.2.4.3-3: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_29" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 29 (NS_29) |  |  |

6.6.2.3A.2.5	Test Requirements

6.6.2.3A.2.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.2.5.1-1.

Table 6.6.2.3A.2.5.1-1: UTRA UE ACLR for CA

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

6.6.2.3A.2.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.2.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10, shall be higher than the limits in table 6.6.2.3A.2.5.2-1.

Table 6.6.2.3A.2.5.2-1: CA E-UTRA ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3A.2.5.3	Test requirements of E-UTRA (network signalled value “NS_29”)

When "NS_29" is indicated in the cell, the UE emission shall meet the additional requirements specified in Table 6.6.2.3A.2.5.3-1 for E-UTRA channels assigned within the frequency ranges 5150-5350 MHz and 5470-5725 MHz.

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.2.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR2, derived in step 10, shall be higher than the limits in table 6.6.2.3A.2.5.3-1.

Table 6.6.2.3A.2.5.3-1: Additional E-UTRAACLR requirement

|  | Channel bandwidth / E-UTRAACLR2 / Measurement bandwidth |
| --- | --- |
|  | 20 MHz |
| E-UTRAACLR2 | 38.7 dBc |
| E-UTRA channel Measurement bandwidth | NOTE 1 |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 |
| NOTE 1: 18 MHz for E-UTRA channels assigned within 5150-5350 MHz; 19 MHz for E-UTRA channels assigned within 5470-5725 MHz. |  |

< End of changes >