3GPP TSG RAN WG5#81 	R5-186762r1

# Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4655 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RRC TC Qoemtsi Measurement Collection in 8.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_QMC_Streaming-UEConTest |  |  |  |  |  |  | Date: |  | 2018-10-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of RRC TC Qoemtsi Measurement Collection for LTE QMC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of RRC TC Qoemtsi Measurement Collection for LTE QMC in 8.3.5.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.5.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | R1: Optimize the test procedure, so that the message contents are updated accordingly. |  |  |  |  |  |  |  |  |

8.3.5.2	RRC connection reconfiguration/ Qoemtsi Measurement Collection /QoE measurement setup and report and release

8.3.5.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state without SRB4 setup or it is not configured to provide measurement report for Qoemtsi }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does not transmit a MeasReportAppLayer message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state without SRB4 setup and it is configured to provide measurement report for Qoemtsi }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does not transmit a MeasReportAppLayer message }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state with SRB4 setup and it is configured to provide measurement report for Qoemtsi }

ensure that {

when { UE is caused to send MeasReportAppLayer message }

then { the UE does transmit a MeasReportAppLayer message with measReportAppLayerContainer set to the value of the measurement report information for the service type Qoemtsi from the upper layer}

}

(4)

with { UE in E-UTRA RRC_CONNECTED state and it is configured to provide measurement report for Qoemtsi}

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing a otherConfig that includes the measConfigAppLayer set to Release }

then { the UE does not transmit any more MeasReportAppLayer message }

8.3.5.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.10.9, 5.6.19, and 6.3.6. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.10.9]

The UE shall:

...

1>	if the received otherConfig includes the measConfigAppLayer:

2>	if measConfigAppLayer is set to setup:

3>	forward measConfigAppLayerContainer to upper layers considering the serviceType;

3>	consider itself to be configured to send application layer measurement report in accordance with 5.6.19;

2>	else:

3>	inform upper layers to clear the stored application layer measurement configuration;

3>	discard received application layer measurement report information from upper layers;

3>	consider itself not to be configured to send application layer measurement report.

TS 36.331, clause 5.6.19

5.6.19.1	General

Figure 5.6.19.1-1: Application layer measurement reporting

The purpose of this procedure is to inform E-UTRAN about application layer measurement report.

5.6.19.2	Initiation

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement, i.e. when measConfigAppLayer has been configured by E-UTRAN.

Upon initiating the procedure, the UE shall:

1>	if configured with application layer measurement, and SRB4 is configured, and the UE has received application layer measurement report information from upper layers:

2>	set the measReportAppLayerContainer in the MeasReportAppLayer message to the value of the application layer measurement report information;

2>	submit the MeasReportAppLayer message to lower layers for transmission via SRB4.

[TS 36.331, clause 6.3.2]

MeasReportAppLayer field descriptions

measReportAppLayerContainer The field contains container of application layer measurements, see Annex L (normative) in TS 26.247 [90].

serviceType Indicates the type of application layer measurement. Value qoe indicates Quality of Experience Measurement Collection for streaming services, value qoemtsi indicates Quality of Experience Measurement Collection for MTSI.

8.3.5.2.3	Test Description

8.3.5.2.3.1	Pre-test conditions

Same Pre-test conditions as specified in clause 8.3.5.1.3.1

8.3.5.2.3.2	Test procedure sequence

The same test procedure as specified in clause 8.3.5.1.3.2 with the exception in steps 1, 3, 5, 9, 11, 13, and 17:

The service type is qoemtsi instead of qoe.

8.3.5.2.3.3	Specific message contents

The same message contents as specified in clause 8.3.5.1.3.2, with the following exception:

Table 8.3.5.1.3.3-4 is replaced by Table 8.3.5.2.3.3-1

Table 8.3.5.2.3.3-1 MeasReportAppLayer

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasReportAppLayer-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions ::= CHOICE { |  |  |  |
| MeasReportAppLayer-r15-IEs SEQUENCE { |  |  |  |
| measReportAppLayerContainer-r15 | Octets string (1…8000) | the exact length in each step equals to the length of the container in the previous step |  |
| serviceType | qoemsti |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |