3GPP TSG-RAN5 Meeting#81	Draftv2_R5-187274r1

# Spoken, USA, 12th – 16th November 2018

Title:							SNR estimation for priority 1 and 2 FR2 Tx test cases

Source:			Anritsu

Agenda Item:			5.3.9.17

Document for:			Endorsement

# 1.	Introduction

# In RAN5 NR#3 AdHoc meeting, during the discussion of low SNR test cases on [1], some companies raised that more TCs like SEM, ACLR can have small SNRs. In this document, we focus on the Priority 1 and 2, and re-calculate the SNR for all of them.

# 2.	Discussion

# SNR for Priority 1 and 2 Tx TCs are given in Table 1.

Table 1: SNR for Priority 1 and 2 Tx test cases

| Test case | Applicable Frequency Range [GHz] | Estimated SNR [dB] | Core requirement [dBm] | Required relaxation to ensure SNR=10dB |
| --- | --- | --- | --- | --- |
| MOP (Min Peak EIRP, PC3) | 23.45GHz ≤ f ≤ 30.3 GHz | > 10 (BW ≤ 400MHz) | 22.4dBm/ChBW | - |
|  | 30.3 GHz ≤ f ≤ 40.8 GHz | > 10 (BW ≤ 400MHz) | 20.6dBm/ChBW | - |
| Off power | 23.45 GHz ≤ f ≤ 30.3 GHz | -13.9 + ChBW | -35dBm/ChBW | 23.9 - ChBW |
|  | 30.3 GHz < f ≤ 40.8 GHz | -23.1 + ChBW | -35dBm/ChBW | 33.1 - ChBW |
| Rx spurious | 6 GHz ≤ f ≤ 20 GHz | 6.3 | -47dBm/1MHz | 3.7 |
|  | 20 GHz < f ≤ 40 GHz | -0.7 | -47dBm/1MHz | 10.7 |
|  | 40 GHz < f ≤ 80 GHz | -16.6 | -47dBm/1MHz | 26.6 |
| Tx spurious | 6 GHz ≤ f ≤ 20 GHz | > 20 | -13dBm/1MHz | - |
|  | 20GHz ≤ f ≤ 80 GHz | > 10 | -13dBm/1MHz | - |
| UE co-existence (1) | 26.5 GHz ≤ f ≤ 29.5 GHz | 5.7 | -5dBm/100MHz | 4.3 |
| UE co-existence (2) | 37 GHz ≤ f ≤ 40 GHz | 2.3 | -2dBm/100MHz | 7.7 |
| UE co-existence (3) | 26.5 GHz ≤ f ≤ 29.5 GHz | -19.3 | -50dBm/1MHz | 29.3 |
| SEM | 23.45 GHz ≤ f ≤ 30.3 GHz | > 10 | -13dBm/1MHz | - |
|  | 30.3 GHz < f ≤ 40.8 GHz | 4.2 | -13dBm/1MHz | 5.8 |
| ACLR | 23.45 GHz ≤ f ≤ 30.3 GHz | 4.0 + ChBW | 23dBm(TRP)-17dBc=6dBm/ChBW | 6.0 - ChBW (NOTE2, NOTE3) |
|  | 30.3 GHz < f ≤ 40.8 GHz | -1.8 + ChBW | 23dBm(TRP)-17dBc=6dBm/ChBW | 11.8 - ChBW (NOTE2) |
| OBW, Frequency Error | No impact from SNR |  |  |  |
| NOTE ChBW = 10log10(400MHz/ChBW), 9.03dB for ChBW=50MHz. NOTE 2 : Relaxation for the dBc value with relaxed adjacent channel power and no change for carrier power. NOTE 3 : No relaxation required for smaller ChBw(50MHz and100MHz) if BW dependent relaxation is considered. |  |  |  |  |

# Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43dBm – Path loss tolerable (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

# Other assumptions:

- Path loss (= Free space path loss + Antenna gain) = 52.5 dB @ 29.5GHz 
 (Delta for other frequency follows the delta from Friis' s equation)

- Channel Bandwidth: up to 400MHz

- Spurious measurement path is separated from in-band measurement path

In case LNA can be used, loss between antenna and 1st LNA much depends on the frequency and also test cases, approx. 6dB - 20dB are assumed. For LNA, commercial availability (https://www.sagemillimeter.com/amplifiers/low-noise-amplifiers/) is considered in this estimation. Each LNA has different characteristic of Gain, support frequency, etc then various combination can be possible with the actual implementation considering the cost, space, power consumption, etc, then not always the ones with minimum NF can be used. In this estimation, NF of 3.2 - 5.5dB are used depending on the frequency in the estimation.

For co-existence, SEM and ACLR, with which the low power requirement needs to be measured with the presence of carrier (could be maximum power) in vicinity, LNA cannot be used due to the maximum limit for input power for the LNA. Also, the presence of the carrier limits the maximum gain of power amplifier used or pre-amp of SA etc. Hence, SNR tends to become worse compared to the TC with same level of requirement but for which LNA can be used . ( This is one example why SNR is not always one-to-one relationship with requirement level )

# 3.	Way forward

As the SNR from SS vendors can have variation and requires further assessment of SNR, it is proposed to classify the cases as follows depending on the level of SNR.

Class A: Tests that requires relaxation

- Off Power

- Rx spurious

- UE co-existence (2)

- UE co-existence (3)

Class B1 : Need of relaxation depends on further assessment of SNR and treatment of SNR in MU budget table

- UE co-existence (1)

- SEM(30.3 GHz < f ≤ 40.8 GHz)

Class B2 : Need of relaxation depends on further assessment of SNR, treatment of SNR in MU budget table and selection of channel BW

- ACLR(23.45 GHz ≤ f ≤ 30.3 GHz)

- ACLR(30.3 GHz < f ≤ 40.8 GHz)

Class C : Tests that don’t require relaxation

- MOP (PC3 Min Peak EIRP)

- Tx Spurious

- SEM(23.45 GHz ≤ f ≤ 30.3 GHz)

Followings are proposed as way forwards.

Proposal 1 : For class A, agree on the required relaxation value in Table 1 with [ ] to wait for further assessment of SNR. Consider the MU factor of SNR based on the relaxed requirement. Whether to test with relaxation or not to test is TBD.

Proposal 2 : For class B1 and B2, further assesses the SNR and treatment of SNR in MU budget table to decide the need of relaxation.

# 4.	Conclusion

In this paper, SNR estimation for Priority 1 and 2 Tx test cases are provided. RAN5 is asked to endorse following proposal.

Proposal 1 : For class A, agree on the required relaxation value in Table 1 with [ ] to wait for further assessment of SNR. Consider the MU factor of SNR based on the relaxed requirement. Whether to test with relaxation or not to test is TBD.

Proposal 2 : For class B1 and B2, further assesses the SNR and treatment of SNR in MU budget table to decide the need of relaxation.

Note that some technical background for the need to eliminate the impact from SNR as much as possible(i.e. relax the requirement to ensure enough SNR) is provided in [2]. Threshold SNR=10dB shown in the right most column of Table 1 is aligned with the proposal in [2]. Depending on the threshold SNR discussion in [2], threshold SNR could be changed.

# 5.	References

[1] R5-185806, “Testability issue of low PSD test cases in FR2 (2)”, Anritsu, RAN5 NR#3 AdHoc

[2] R5-187273, “Consideration on MU and TT for FR2 low PSD tests”, Anritsu, RAN5#81,