3GPP TSG-RAN5 Meeting#83	R5-193936r2

# Reno, USA, 13th – 17th May, 2019

Title:							Introduction of NR FR2 WP update

Source:			Huawei, HiSilicon

Agenda Item:

Document for:			Information

# 1.	Background

This paper introduces the main update of the NR FR2 WP between RAN5 #5-NR-Adhoc and RAN5 #83 meeting.

# 2.	Discussion

# 2.1	Newly completed test cases

After RAN5 #5-NR-Adhoc meeting, the 7.3.2 REFSENS is 100% completed. The only pending issue, MU value is determined during the meeting and captured in R5-193465.

# 2.2	Removed test cases

1. RAN4 has removed below test cases from TS 38.101-2 f50, and those are also removed from RAN5 WP.

| 6.5.2.2 | Additional SEM |
| --- | --- |
| 7.7 | Spurious response |
| 7.10 | receive image |

2. The below test cases are verifying time mask for ON/ON TTI boundaries. In LTE testing specification, the ON/ON time mask is not tested, so it is proposed to also follow the same principle in NR, and remove the TCs from NR WP.

In addition as per the discussion in [1], even for ON/OFF time mask testing, considering the long measurement period compared with the transient period, along with the +/-9dB open loop power control requirement, UE could have a far longer transition time than the allowed 20 us and still safely pass the test. For ON/ON time mask testing, with the power difference less than ON/OFF mask, the test would be even more meaningless.

| 6.3.3.3 | Transmit power time mask for slot and short or long subslot boundaries |
| --- | --- |
| 6.3.3.7 | PUSCH-PUCCH and PUSCH-SRS time masks |
| 6.3.3.8 | Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries |
| 6.3.3.9 | Transmit power time mask for consecutive short subslot transmissions boundaries |

According to above discussion, no details would be defined for the test cases. Only text providing the background would be captured in each TC. After that the TCs would be considered as 100% completed.

3. The MB testing is agreed to be merged to single band testing. There for below test cases are removed from WPl

| 38.521-2 | 6.2_1 | Transmit power with multi-band relaxation |
| --- | --- | --- |
| 38.521-2 | 6.2_1.1 | UE maximum output power with multi-band relaxation |
| 38.521-2 | 6.2_1.1.1 | UE maximum output power - EIRP and TRP with multi-band relaxation |
| 38.521-2 | 6.2_1.1.2 | Spherical coverage with multi-band relaxation |
| 38.521-2 | 6.2_1.2 | UE maximum output power reduction with multi-band relaxation |
| 38.521-2 | 6.2_1.3 | UE maximum output power with additional requirements with multi-band relaxation |
| 38.521-2 | 6.2_1.4 | Configured transmitted power with multi-band relaxation |

# 2.3	Changed test cases titles

Title of test case 6.4.2.5 is changed by removing ‘with spectrum shaping’ to keep alignment with RAN4 #90bis agreed R4-1904986.

| 6.4.2.5 | EVM spectral flatness for pi/2 BPSK modulation with spectrum shaping |
| --- | --- |

# 2.4	Handling of test cases with testability issue

RAN5 has decided to not test Maximum input level due to unachievable high power level at RAN5#81. How to handle this TC in FR2 WP is discussed.

There were concerns that validation forums usually refer to RAN5’s WP to generate their validation plan, and the Maximum input level TC might be selected. Therefore there was proposal to remove this TC from RAN5 WP.

On the other hand, there is also need to track this TC in WP so that RAN5 can be aware of its status. Also considering the decision of not testing this TC is already flagged to GCF by LS R5-187685, the risk that GCF selects this TC by mistake is very little. Therefore it’s reasonable to keep the TC in the WP, with a Note indicating the testability issue.

# 2.5 Overall completeness

The overall completeness is changed from 9% to 12%. In addition to progress updated according to RAN5 #5-NR-adhoc output, the place holder in Row 21 and 22 are set to 0 since all CA test cases are listed and no DC test case is expected in Rel-15.

# 3 Reference

[1] R5-187480. Discussion on improved measurement uncertainty for ON/OFF time mask. Ericsson. RAN5#81.